3GPP TSG-RAN WG4 Meeting #112                                                           R4-2413507

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.19.4

Source:	Samsung, Huawei

Title:	Ad-Hoc meeting minutes for NR_duplex_evo [307][308]

Document for:	Approval

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

### Sub-topic 1-1: Draft CR to TR 38.858

##### Issue 1-1-1: Adding a summary sentence in sub-clause 12.2.1

- [Moderator] Quoted from Charter’s Tdoc (R4-2411019) :

  - In TR 38.858 paragraph “12.2.1 North America”, Charter proposed to add a summary sentence regarding SBFD: “Therefore, it is expected that new SBFD operators in AMBIT band or in C-Band will seek a fair coexistence with legacy TDD operating in CBRS band.”  Since RAN1 owns TR 38.858 and RAN4 owns sub-clause 12.2.1, RAN4 would need to endorse this CR before an LS is going out to RAN1 for final approval.

- Moderator Recommendation:

  - Since the sentence is already technically endorsed in the WF in RAN#111 [R4-2409958], the draft CR [R4-2411018] can straightforwardly be endorsed.

- Ad-Hoc discussion:

  - Charter: Another way is sending the LS to RAN1.

- Ad-Hoc Agreement:

  - Sending an LS to RAN1 by citing the sentence endorsed in RAN4#111.

### Sub-topic 1-2: SBFD frequency domain issues (Band/Channel BW/SCS/Subband configuration support and guardband restriction)

##### Issue 1-2-1: Band/channel bandwidth support for SBFD

- [Moderator] Here is the agreement from RAN4#111.

<From RAN4#111> Issue 1-1-1: SBFD as band specific or general feature to all TDD bands - Agreement: - SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations: - Option 1: Declaration based method - The band supported for SBFD shall be manufacturer declaration based - The channel bandwidth supported for SBFD shall be manufacturer declaration based - The supported channel bandwidth can be impacted by the subband/guard band size discussion - Option 2: General principle but with consideration(s) for a specific band - General principle: The channel bandwidth shall be larger than X MHz - FFS the value of X - X can be different for high and middle TDD bands - For a specific band, the following further restriction(s) can be considered: - Certain restriction can be provided by operator(s) for certain band - Other restrictions are not precluded.

- [Moderator] Additional consideration, under which to define the SBFD feature with requirements of which can potentially be applied to all TDD band:

  - Option 1 (CATT, Samsung, ZTE, Qualcomm): Declaration-based method, and SBFD is not restricted on any band or channel bandwidth:

    - The band supported for SBFD shall be manufacturer declaration based

      - Proposal 1-1 (Samsung): All channel bandwidths shall be defined for RF requirements.

    - The channel bandwidth supported for SBFD shall be manufacturer declaration based

  - Option 2 (Qualcomm, CMCC, Huawei, Ericsson, Nokia, Charter): General principle on channel bandwidth, e.g., the channel bandwidth shall be larger than X MHz

    - RAN4 to define RF requirements for all channel bandwidths that satisfy the minimum channel bandwidth criteria (i.e., larger than X MHz):

      - Proposal 2-1 (Huawei): X=60

      - Proposal 2-2 (Ericsson, Nokia): X=50

      - Proposal 2-3 (CMCC): Proposals for detailed value of X:

        - X=30 for middle TDD bands;

        - X=60 for high TDD bands

      - Proposal 2-4 (Charter): X=10

    - Proposal 2-5 (Huawei): For channel bandwidth, applying 100MHz as maximum

    - Proposal 2-6 (Qualcomm): To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with bandwidth > X MHz, given in Table 5.2-1.

    - Proposal 2-7 (Qualcomm): RAN4 to agree on the value of X after agreeing on the minimum guard band requirements and subband sizes for both DUD and DU subband configurations

    - Proposal 2-8 (Nokia): For FR1, RAN4 to prioritize SBFD requirements for BS carrier bandwidths between 50 MHz and 100 MHz. For FR2, both 100MHz and 200MHz.

    - Proposal 2-9 (Charter): Define the SBFD feature as a band specific feature and have Operators drive the requirement of SBFD in a per band basis.

  - Option 3 (CMCC, Ericsson, vivo): Typical channel bandwidth introduced:

    - Proposal 3-1 (CMCC): 100MHz, 60MHz and 30MHz are suggested as typical channel bandwidth to support SBFD.

    - Proposal 3-2 (Ericsson): 50 MHz and 100 MHz for FR1, 200 MHz and 400 MHz for FR2-1

    - Proposal 3-3 (vivo): 25, 50, 100 and 200MHz

- Moderator Recommendation:

  - Further discussion on the above options.

- Ad-Hoc discussion:

  - Charter: difference between Option 2 and 3? Open to X value discussion and wonder about “per-band” discussion for SBFD feature.

  - ZTE: For option 3, it could be too restrictive. For option 2, if going with 60MHz, it could rule out 15SCS cases. Better to check with the needs from operators. For option 1, it could be comprehensive proposal, which is declaration based.

  - Apple: For option 3, want to check the implication of option 3 for “typical channel bandwidth”. For Option 1, if depends on vendors’ declaration, it could be a reasonable approach.

  - CMCC: Want to clarify our proposed Option 3: here are just some typical options, but don’t want to restrict other possible ones. Based on our band status, 30 and 60MHz are proposed.

  - CATT: Option 1 is better choice, by covering option 2 and 3. If only list limited number, does that mean others are restricted to be used? For conformance test, could be also declaration-based.

  - Ericsson: O2 and O3 are preferred. Not intend to give restriction, and if other channel BWs are needed, we can add them in the later, like basket WI.

  - Qualcomm: O1 is preferred. For O2, we need to discuss X after studying subband BW.

- Ad-Hoc Agreement:

  - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

    - The supported channel bandwidth(s) is declared by gNB vendors

    - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

    - The supported channel bandwidth shall be no less than X MHz

      - X = [20]

      - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators.

      - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

# Topic #2: Modification of existing requirements - TX

### Sub-topic 2-1: BS output power

#### Issue 2-1-1: Output power limits for BS classes

- [Moderator]: On BS output power the following agreements are achieved in study item,

Since configuration (e.g. antenna, power configuration etc) between SBFD and non-SBFD symbols/slots might be different, RAN4 reached the following consensus for the BS RF requirement of BS output power for both conducted and OTA output power: - It is allowed to have the different conducted declaration for normal DL symbols/slots and SBFD DL symbols/slots. - It is allowed to have different EIRP/TRP declaration (for level and direction) for normal DL symbols/slots and SBFD DL symbols/slots. - Accuracy requirement for TRP/EIRP and conducted power shall be the same for normal DL symbols/slots and SBFD DL symbols/slots.

And the following agreement is achieved in RAN4#111:

Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols - Agreement: - PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols - Vendors can declare different TX power values for normal and SBFD slots/symbols

- Proposals:

  - Option 1 (Samsung): the existing output power limits for BS classes are only specified for non-SBFD symbols.

    - Proposal 1 (Samsung): For SBFD-capable BS, RAN4 will not specify the output power limits for different BS classes for non-SBFD symbols.

    - Proposal 2 (Samsung): To determine one SBFD-capable BS’s class, the existing requirements for output power limits for BS classes shall be used for non-SBFD symbols, no matter what is the power level declared for SBFD symbols.

  - Option 2: the existing output power limits for BS classes are specified for both non-SBFD symbols and SBFD symbols

- Moderator Recommendation:

  - Discussion on the options.

- Ad-Hoc discussion:

  - Samsung: In SBFD symbol, only half of the antenna will be used for SBFD. The power will be defined on the normal symbol instead of all.

  - ZTE: For SBFD, we want to make sure only single declaration can be conducted.

  - Samsung: Confirm ZTE question. But we wonder on which symbol the BS type/power can be the basic for declaration.

  - Huawei: We also think one declaration is enough. The current output power limit definition is only for upper limit, which has no ambiguity regarding Samsung concern. The current declaration applies to all symbols.

  - Nokia: One declaration for non-SBFD symbol is enough, but power degradation on SBFD symbol is also needed.

Common understanding (with this no agreement was needed during Adhoc): One BS class declaration is given for one BS implementation for SBFD capable gNB.

### Sub-topic 2-2: Output power dynamics

#### Issue 2-2-1: Total dynamic range

- [Moderator]: On output power dynamics the following agreements are achieved in study item,

Regarding the output power dynamic requirement, which mainly consists of RE power control dynamic range requirement and total dynamic range requirement, RAN4 reached the following consensus: - To reuse the existing RE power control dynamic range requirement for SBFD BS; - The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots, it is agreed to define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

- Proposals:

  - Proposal (Ericsson): Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement.

- Moderator Recommendation:

  - The proposal is line with SI agreement. Discussion whether or not the proposal can be agreed.

- Ad-hoc agreement:

  - Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement

### Sub-topic 2-3: Transmit ON/OFF power

#### Issue 2-3-1: Transmit ON/OFF power

- [Moderator]: On Transmit ON/OFF power the following agreements are achieved in study item,

Regarding the transmitter ON/OFF power requirement, RAN4 mainly focus on the ON-OFF time mask and concluded that transmit ON/OFF power requirement is not applicable within SBFD time slot.

- Proposals:

  - Proposal (Ericsson): Transmitter ON/OFF power should apply to normal slot

- Moderator Recommendation:

  - The proposal is line with SI agreement, however, it does not include the agreements for SBFD time slot. Discussion whether or not the following proposal can be agreed.

    - Transmitter ON/OFF power should apply to normal slot and it is not applicable within SBFD time slot.

- Ad-Hoc discussion:

  - Samsung: In the RAN1 discussion, they use symbol rather than slot.

  - Ericsson: If we change to symbol, how about conformance test?

- Ad-Hoc agreement:

  - Transmitter ON/OFF power should apply to normal symbol and it is not applicable within SBFD symbol.

### Sub-topic 2-4: Unwanted emissions

#### Issue 2-4-1: ACLR and OBUE

- [Moderator] The following agreement is achieved in RAN4#111:

#### Issue 2-3-1: The necessity of Co-location ACLR requirement - Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement. #### Issue 2-3-2: OBUE - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

- Proposals:

  - Proposal 1 (CMCC): If co-location ACLR is defined, then it is necessary to ensure that the receiver of the BS is not blocked, otherwise the note in the spec that the ACLR requirement is not applied to the co-location scenario is needed.

  - Proposal 2 (Samsung/Qualcomm): separate the discussion by considering coexisting with legacy TDD or new SBFD system in the adjacent channel:

    - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the existing ACLR requirement for SBFD-capable BS.

    - When coexisting with new SBFD system in adjacent channel, RAN4 to consider additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as that represent the challenging scenario.

  - Proposal 3 (ZTE/Ericsson): follow the legacy ACLR requirement for SBFD capable BS without defining any tougher requirement.

  - Proposal 4 (CATT): No new co-location ACLR requirement is defined in SBFD WI.

  - Proposal 5 (CATT): The assumption for the victim BS DESENS and CL of co-location BS should be decided to further discuss the new OBUE (or the 2nd ACLR) requirement.

  - Proposal 6 (Huawei): to specify additional OBUE requirement for SBFD capable BS in order to ensure co-existence performance for SBFD BSs operating in adjacent frequency.

  - Proposal 7 (Huawei): the additional OBUE requirement should be defined at different frequency offsets from the sub frequency band edge.

- Moderator Recommendation:

  - RAN4 may firstly try to confirm the following bullets is agreeable:

    - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS.

    - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

- Ad-Hoc discussion:

  - Huawei: Qualcomm and Samsung proposed co-ex between SBFD capable BSs, We think this is a good starting point.

  - ZTE: The first bullet would be fine. The second one we observe a lot degradation in our simulation. This is also related to in-band blocking issue.

  - Nokia: We don’t see the necessity of defining new requirements. Customer related requirements can be considered when specific deployment happened.

  - CATT: If we discuss this for all of the band this must be difficult. Can we narrow down the discussion scope e.g. which band(s)?

  - CMCC: We can agree with the first bullet. If we want to define ACLR/OBUE, we should make sure that requirement should be reasonable. Legacy requirement can be reused for the co-located case.

  - Samsung: Not sure if it should be limited to only co-located case. Maybe we can pick band n104 firstly for the second sub-bullet. Per-operator filter can be reflected in this way.

  - Ericsson: Such co-ex case has not been studied during SI phase. We would suggest to think more on the working procedure. Pending on BS evo SI can be considered.

  - Huawei: This is also related to co-ex study, which has been covered in the SI. From our perspective, we think for Macro case, such co-ex we proposed is more common in the field. For CATT comment, we think start the discussion with certain band is a good suggestion.

  - QC: For the second bullet, it may need to be considered in future. We need more time to check the feasibility conclusion from proponent.

- Ad-Hoc agreement:

  - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS.

### Sub-topic 2-5: Transmitter signal quality

#### Issue 2-5-1: Joint measurement for normal DL symbols/slots and SBFD symbols/slots

- [Moderator] The following is agreed in SI which ask for more study on joint measurement for normal DL symbols/slots and SBFD symbols/slots:

#### 10.1.2.4 Transmitted signal quality Regarding the transmitter signal quality, RAN4 agreed that all the existing requirement for frequency error, modulation quality (EVM) and time alignment error (TAE) shall also be applied to BS in SBFD symbols/slots. - Further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots during WI phase.

- Observation/Proposal on joint measurement for normal DL symbols/slots and SBFD DL symbols/slots:

  - Observation 1 (Nokia): New complex test model would be needed to support joint measurement of transmitter signal quality for normal DL symbols/slots and SBFD DL symbols/slots.

  - Proposal 1 (Ericsson): Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

  - Proposal 2 (Ericsson): Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

- Moderator Recommendation:

  - Discussion whether or not P1 and P2 can be agreed.

# Topic #3: Modification of existing requirements - RX

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: OTA sensitivity

#### Issue 3-1-1: OTA sensitivity degradation

- [Moderator]: On Reference sensitivity requirements the following agreements are achieved in study item,

Regarding Reference sensitivity requirement for SBFD-capable BS, due to the self interference caused internally to receiver side, RAN4 reached the following consensus: - For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed. - Otherwise, OTA sensitivity requirement could be derived based on the following equation: PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G - The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase. - The following aspects need more discussion during a WI phase - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

- Assumption for the interference considered in the OTA sensitivity degradation:

  - Alt. 1(Qualcomm): RAN4 to discuss allowable degradation due to self-interference, inter-site interference, and inter-sector interference.

  - Alt. 2(Samsung/ZTE/Huawei): Only self-interference considered

  - Alt. 3: Others

- Options:

  - Option 1: the degradation value is BS declaration based.

  - Option 2: a fixed value for degradation provided in the specification.

    - Option 2a (CATT/ZTE/Huawei): 1.0dB degradation

    - Option 2b (Ericsson): [0.5~1.0] dB degradation

    - Option 2c (Nokia): Use maximum of 0.5dB for desensitization target value for the OTA sensitivity requirement due to self-interference.

- Moderator Recommendation:

  - Discussion on the Options/Alts firstly.

### Sub-topic 3-2: Dynamic range

#### Issue 3-2-1: Dynamic range requirement

- [Moderator]: On Dynamic range requirement the following agreements are achieved in study item,

Regarding the dynamic range requirement, this requirement is still applicable for SBFD-capable BS. The IoT level and wanted signal power level could be further discussed in the WI phase.

- Proposals:

  - Proposal 1 (Qualcomm): RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work.

  - Proposal 2 (Samsung/Huawei/CATT): The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

  - Proposal 3 (ZTE): for receiver dynamic requirement, both uplink signals and BS2BS CLI signal should be considered for IoT levels.

- Moderator Recommendation:

  - Discussion on the proposals.

### Sub-topic 3-3: ACS

#### Issue 3-3-1: ACS requirement

- [Moderator] The following is agreed in SI:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

- Proposals:

  - Proposal 1 (CMCC): If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

  - Proposal 2 (CATT): The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

- Moderator Recommendation:

  - Discussion on the proposals

### Sub-topic 3-4: In-band blocking

#### Issue 3-4-1: in-band blocking

- [Moderator] The following is agreed in SI:

- In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

And per way forward in RAN4#111, companies are encouraged to provide the evaluation results for in-band blocking in this meeting.

- Observations/Proposals:

  - CATT (R4-2411081)

    - The simulation shows that the interference from TDD DL to SBFD UL subband for FR1 and FR2-1 are less than the interference signal level in existing in-band blocking requirements.

    - For FR1 and FR2-1, existing in-band blocking can be reused.

  - Qualcomm (R4-2411515)

    - To derive the in-band requirement for that particular case, it is recommended to consider the 99% percentile point of the received power for SBFD UL as a victim case for each scenario and then get the average point.

    - RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements.

  - Samsung (R4-2411639)

    - Based on the distributions of received power levels plotted for Scenario 1, 3, 5, 6, and 9, the adjacent channel gNB DL power level could be the major factor contributing to the total co-channel received power.

    - The percentage value of grid shifting plays an important role to determine the distributions of adjacent-channel and co-channel gNB DL received power, by translating the distance between interfering gNB and victim gNB into pathloss.

    - RAN4 shall align the co-existence simulation results by comparing the distributions of adjacent-channel and co-channel gNB DL received powers, by comparing the simulation assumptions and other details firstly.

  - CableLabs, Charter Communications (R4-2411723)

    - the new BS receiver in-band blocking study should be captured in a technical report. We prefer Option 3.

      - Option 1: merge into clause 11 in TR 38.858.

      - Option 2: capture it in a new Rel-19 TR.

      - Option 3: capture it in a new Rel-19 TR, briefly summarize the conclusions in TR 38.858 and cite the Rel-19 TR.

    - The in-band blocking study should extend to case 2 because both case 2 and case 3 have strong gNB-to-gNB ACI. The results should be captured in clause 11 in TR 38.858.

    - It is unreasonable to only consider ACI for in-band blocking. Some level of co-channel interference should also be considered. How to consider co-channel interference is FFS.

    - the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement should be X=99.

    - The CDF of the receiver blocking power for scenario 1 (FR1 UMa-to-UMa) case 3 (TDD DL to SBFD UL subband) is provided in Figure 1. 33% of the blocking power is larger than the P2=-25 dBm in the baseline NF model, and 100% of the blocking power is larger than the P2=-15 dBm in the enhanced NF model.

    - The CDF of the receiver blocking power for scenario 5 (FR1 UMi-to-UMi) case 3 (TDD DL to SBFD UL subband) is provided in Figure 2. 35% of the blocking power is larger than the P2=-20 dBm in the NF model.

    - The CDF of the receiver blocking power for scenario 6 (FR2-1 UMa-to-UMa) case 3 (TDD DL to SBFD UL subband) is provided in Figure 3. 99% of the blocking power is larger than the P2=-40 dBm in the NF model.

  - CMCC (R4-2411735)

    - Only collect power level of interference from adjacent channel to derive in-band blocking levels.

    - the reference point for the power level should be before RX beamforming

    - 100% grid-shift should be considered

  - ZTE （R4-2412724）

    - for Scenario 1 FR1 Urban macro scenario, the received power level are much higher than the existing IBB requirement -43dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

    - for Scenario 3 FR1 indoor scenario, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

    - for Scenario 6 FR2 urban macro scenario and Scenario 9 FR2 indoor scenario, the received power is close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

  - Ericsson（R4-2412916）

    - Define in-band blocking interferer signal power to 5 dBm for Wide Area BSs in SBFD networks.

    - Define in-band blocking interferer signal power to -1.5 dBm for Medium Range BSs in SBFD networks.

    - Re-use the existing in-band blocking requirement for Local Area BSs in SBFD networks.

    - Investigate whether an additional requirement based on a single input signal placed to cause IM with the RX sub-band provides any additional robustness, and whether such a requirement is anyhow implicitly captured by the SBFD RX blocking requirement.

  - Nokia（R4-2413235）

    - RAN4 should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

    - Consider only the adjacent channel interference power to determine the in-band blocking power.

    - To reduce the scope of the simulations, 10% grid-shift is the only considered grid-shift value.

    - To reduce the scope of the simulations, consider only SBFD antenna configurations and TDD BS transmit power defined as baseline or 1st priority according to TR 38.858 Annex E.

    - Use the 99th percentile of the in-band blocking power CDF (X = 99) to derive the BS RF requirements.

    - Re-use the excel spreadsheet from simulator calibration during the study item to collect the in-band blocking power results from different companies.

    - RAN4 should not consider any effect of the CLI handling mechanisms for the calculation of the in-band blocking power levels.

  - Huawei（R4-2413284）

    - 60~ 65 dB MCL can cover most of deployments scenarios since the measured cases can represent a certain worse case.

    - -10~ -15 dBm is considered as the in-band blocking level for FR1 Macro BS.

- Moderator Recommendation:

  - RAN4 need to discuss on contributing source for in-band blocking requirement:

    - Option 1: DL co-channel interference and DL adjacent channel interference

    - Option 2: only the DL adjacent channel interference

  - RAN4  need to discuss on how in-band blocking requirement are derived:

    - Option 1: MCL assumption for BS2BS CLI interference

    - Option 2: Co-existence study

    - Option 3: Both

  - For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption

    - the power level which should be collected from the simulation results to derive in-band blocking levels: power level of interference from adjacent channel only

    - The reference point for the power level which should be before array gain

    - FFS grid-shift values should be considered.

      - Option 1: 10%

      - Option 2: 20%

      - Option 3: 50%

      - Option 4: 100%

    - 99% of the UL SBFD wideband received power CDF to define the blocking requirement

  - RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements

- Ad-Hoc discussion (only for the first two bullets from Moderator Recommendation):

  - For contributing source for in-band blocking requirement:

    - ZTE: Option 2 is agreeable.

    - Huawei: Current in-band blocking is quite different from the legacy one.

    - QC: We share the same view as ZTE.

    - Nokia: In reality there is also other interference. Do we need to introduce co-channel as well?

    - Samsung: Option 2.

    - Huawei: Option 2. Co-channel can be considered for the new requirements.

    - Ericsson: Same view as Huawei.

    - Ericsson: Fine with core requirement. If we change to symbol, how about conformance test?

  - How in-band blocking requirement are derived:

    - Samsung: There is a link between Op1 and Op2. The change on grid shift, the MCL will be changed as well. We are open to both options. For Option 1, the exact value e.g. 30dB should be revisited.

    - ZTE: Option 2 is more reasonable.

    - Ericsson: Option 2 is more straightforward.

    - Huawei: We also need to give option 1 a try, since it will share more lights on the whole picture.

    - Nokia: For the time being, Option 2 is feasible from work load perspective.

    - Samsung: BS is not randomly dropped. Option 2 is adopted, MCL can still be calculated.

    - CATT: MCL calculation can be considered.

    - ZTE: The close BS may not be the only source, so Option 2 is preferable.

    - Cable lab: Agree with ZTE. Interference contributor is random.

    - Ericsson: Option 3 would lead to a difficult way.

- Ad-Hoc agreement:

  - On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference.

### Sub-topic 3-5: Receiver intermodulation

#### Issue 3-5-1: Necessity of New RX intermodulation requirement with 1 interfering signal

- Proposals:

  - Proposal 1 (Ericsson/ZTE): Whether IMD requirement for single interfering signal scenario is needed. Investigate whether such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Proposal 2 (Samsung): If RAN4 want to introduce the additional RX intermodulation requirement (a single input signal placed to cause IM with the RX sub-band), the new intermodulation scenario shall be confirmed firstly with evidence showing the proposed scenario exists in practice.

- Moderator Recommendation:

  - Discussion on proposals firstly.

#### Issue 3-5-2: Impact on existing RX intermodulation requirement with 2 interfering signal

- Proposals:

  - Proposal 1 (ZTE): for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

- Moderator Recommendation:

  - Discussion on proposal firstly.