3GPP TSG-RAN WG4 Meeting #112                                                           R4-2413514

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.19.4

Source:	Samsung

Title:	Way Forward for [112][307] NR_duplex_evo_General

Document for:	Approval

# Introduction

This document is provided for capturing agreements and way forward on the thread [307] NR_duplex_evo_general based on the discussion in RAN4#112.

# For Agreement

## Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

#### Issue 1-2-1: Band/channel bandwidth support for SBFD

- Agreement:

  - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

    - The supported channel bandwidth(s) is declared by gNB vendors

    - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

    - The supported channel bandwidth shall be no less than X MHz

      - X = [20]

        - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value.

      - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators.

      - FFS how to implement in the spec:

        - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

        - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

- Agreement:

  - BS channel bandwidth for SBFD-capable BS:

    - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation.

    - The existing definition of BS channel bandwidth (for information only):

      - BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink.

#### Issue 1-2-3: Transmission bandwidth configuration for SBFD

- Agreement:

  - BS transmission bandwidth configuration for SBFD:

    - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS.

    - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1.

    - FFS a corresponding figure for SBFD is needed

    - The existing definition of BS transmission bandwidth configuration (for information only):

      - BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS.

#### Issue 1-2-4: DL/UL subband configurations support for SBFD

- Agreement:

  - Transmission bandwidth configuration for SBFD UL/DL subband:

    - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks.

    - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

  - Guardband between SBFD UL and DL subband:

    - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

- Agreement:

  - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified:

    - Option-1 (with the least flexibility):

      - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement.

      - Example (given in Nokia Tdoc R4-2413238):

        - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5>

        - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5>

    - Option-2 (with the middle flexibility):

      - Option-2a:

        - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

        - UL subband size and guardband size(s) depends on BS implementation

        - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB.

      - Option-2b:

        - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

        - DL subband size(s) and guardband size(s) depends on BS implementation

        - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

    - Option-3 (with the highest flexibility):

      - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations).

      - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB

  - FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

  - BS transmission configuration in BS RF requirements (for information only):

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

#### Issue 1-2-5: Restriction on guardband size for SBFD

- Agreement:

  - Restriction on guardband size for SBFD:

    - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband)

    - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications.

    - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband:

    - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

#### Issue 1-3-1: SBFD time-domain terminology

- Agreement:

  - When specifying RAN4 requirements for SBFD-capable BS, SBFD symbols (instead of SBFD slots/symbols) shall be used to designate the time domain duration in which SBFD related requirements is applied.

#### Issue 1-4-1: CLI handling impact on RF requirement

- Agreement:

  - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline

    - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

## Topic #2: Potentially new requirements for SBFD

#### Issue 2-1-1: Requirement for transient period between SBFD and non-SBFD

- Agreement:

  - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD:

|  | Frequency regions for Transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD to DL) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD to UL) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL to SBFD) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL to SBFD) | SBFD DL subband | BS transmitter OFF-to-ON |

#### Issue 2-1-2: Transient period length between SBFD and non-SBFD

- Agreement:

  - Transient period length between SBFD and non-SBFD:

    - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1.

#### Issue 2-1-3: Location of transient period between SBFD and non-SBFD

- Agreement:

  - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement:

    - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

#### Issue 2-2-1: Requirement for transient period between different SBFD configurations

- Agreement:

  - No need to specify transmitter transient period requirement between different SBFD configurations.

# For Way Forward

## Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

#### Issue 1-4-2: BS RF Specification structure for SBFD requirements

- Way forward:

  - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS:

    - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104)

    - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

      - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable.

      - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements

    - Option 3: A new specification for SBFD BS RF requirements

## Topic #2: Potentially new requirements for SBFD

#### Issue 2-3-1: Necessity of in-channel adjacent subband leakage ratio requirements

- Way forward:

  - FFS the necessity of in-channel adjacent subband leakage ratio requirements

    - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio.

    - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined.

      - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

#### Issue 2-4-1: Necessity of in-channel adjacent subband selectivity requirements

- Way forward:

  - FFS the necessity of in-channel adjacent subband selectivity requirements

    - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity.

    - Option 2: New requirement for in-channel adjacent subband selectivity should be defined.

      - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

#### Issue 2-4-2: Necessity of in-channel adjacent subband blocking requirement

- Way forward:

  - FFS the necessity of in-channel adjacent subband blocking requirements

    - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking.

    - Option 2: New requirement for in-channel adjacent subband blocking should be defined.