3GPP TSG-RAN WG4 Meeting #112	R4-2413524

Maastricht, Netherland, August 19 – 23, 2024

Title: 	Way Forward for Doppler pre-compensation issue

Agenda Item:	5.23.5

Source: 	Huawei, HiSilicon

Document for:	Approval

Background

In the RAN4#110 meeting, although the draft big CR [1] was endorsed in RAN4#110 meeting, doppler shift issues were not extensively discussed. Especially, RAN4 didn’t fully discuss the relationship among the guard band and transmission bandwidth configuration under large doppler shift condition. In RAN4#110bis meeting, this issue was raised by one paper [2]. Finally, a WF [3] was approved to further discuss this issue.

Way forward Further discuss if extra guard-band is required/should be specified when VSAT UE is pre-compensating Doppler.

In this meeting, company provided a discussion paper [4] to address this issue. It’s identified that there is a potential risk for NTN UE to comply with the out-of-band emission requirements when the edge RB falls into the guard band due to Doppler pre-compensation or Doppler shift in NGSO deployment.

After the discussion in the RAN4#112 meeting, the following potential solutions were considered to address this issue.

Identified issue

Issue: There is a potential risk for NTN UE to comply with the out-of-band emission requirements when the edge RB falls into the guard band due to Doppler pre-compensation or Doppler shift in NGSO deployment.

Way forward

Proposal: The following solutions are considered to address the issue if identified that the edge RB fall into the guard band due to the doppler pre-compensation in NGSO deployment.

Solution 1: the first RB and the last RB in the Maximum transmission bandwidth configuration can’t be deployed by NGSO SAN when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Solution 2: UE can report a static capability whether UE is capable of supporting the deployment in NGSO scenario that the edge RB fall into the guard band.

Solution 3: UE can indicate a dynamic information to network whether the edge RB can be deployed or not based on whether the doppler shift is negligible or not.

Solution 4: A guard band at least equivalent to maximum doppler shift expected the NGSO constellation between the channel edge of UE channel bandwidth and spectrum block edge is accounted for as a part of system deployment configuration by the operator.

NOTE 1: other solutions are not precluded.

Reference

[1] R4-2401116, Big CR on TS38.101-5 for UE RF Requirements, Samsung Electronics France SA

[2] R4-2405340, Doppler shift issues for guard band and transmission bandwidth configuration, Huawei, HiSilicon

[3] R4-2405995, Way Forward for [110bis][305] NR_NTN_enh_Part1 Doppler pre-compensation into the guard band, Huawei, Thales

[4] R4-2412949, (NR_NTN_enh-Core) Discussion on potential solution on Doppler shift issues for guard band and transmission bandwidth configuration, Huawei, HiSilicon