3GPP TSG-RAN WG4 Meeting #112	                                               R4-24xxxxx

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:			5.19.2

Source:	Apple

Title:	Way forward on extension of RAN4 default assumption for MU-MIMO advanced receiver

Document for:	Approval

# Introduction

For precoding granularity and resource allocation RAN1 has introduced a limitation since Rel-15 on co-scheduled UE(s) on DMRS ports within the same CDM group when narrowband precoding is configured.

In RAN4 discussion, RAN1 limitation and RAN2 NWA signaling the assumption has only focused on PRG granularity=2,4, but there was no discussion on the assumption when precoding granularity is set to wideband.

There is no default assumption when target UE is configured with wideband precoding.

# Way Forward

Candidate options -

When UE is configured with wideband precoding, for advanced receiver for MU-MIMO

- Option 1: Default assumption is that precoding granularity of co-scheduled UE is also wideband

  - Send LS to RAN2 to add additional NWA to update AdvancedReceiver-MU-MIMO-r18 to cover case for wideband precoding

- Option 2: Up to UE implementation - No requirements for advanced receiver with MU-MIMO

  - FFS whether to send LS to RAN2 to update AdvancedReceiver-MU-MIMO-r18

- Other options not precluded