3GPP TSG-RAN WG4 Meeting #112														R4-2413871

Maastricht, Netherlands, 19 – 23, 2024

Agenda item:			5.24.3

Source:	Apple

Title:	Ad-hoc minutes on RRM requirements for NR_Mob_enh2

Document for:	Approval

# LTM

Issue 1-5-1: Capability for supporting RTD>CP

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 39-1 | SSB based L1-RSRP measurements for multiple cells with RTD > CP | Capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. | 45-1 from RAN1 Rel-18 feature list or 39-2 or 39-2a |
| 45-3a | MAC-CE activated joint LTM TCI states | 1. Supported QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of MAC-CE activated joint LTM TCI states per candidate cell 3. Maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cells | 45-3 |
| 45-4a | MAC-CE activated DL/UL LTM TCI states | 1. Supported QCL source RS for MAC-CE activated DL/UL LTM TCI states 2. Maximum number K1 of MAC-CE activated DL TCI states per candidate cell 3. Maximum number K2 of MAC-CE activated UL TCI states per candidate cell 4. Maximum number of MAC-CE activated DL TCI states across all candidate cells and serving cells 5. Maximum number of MAC-CE activated UL TCI states across all candidate cells and serving cells | 45-4 |
| 45-5 | RACH-based early TA acquisition | 1. Maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE 2. Power ramping for PRACH retransmission based on PDCCH order indication 3. Support of dropping the serving cell UL to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) | 45-7, RAN2 FG for LTM |

- Proposals

  - Option 1 (CATT, MTK):

    - The current capability of SSB based L1-RSRP measurements for multiple cells with RTD > CP (39-2) should be revised.

      - It only needs to describe the capability of handling multiple cells with RTD > CP.

      - This capability will be supported together with the capabilities of SSB based L1-RSRP measurements and/or early T/F tracking and/or PDCCH order RACH.

- Recommended WF

  - Need more discussion.

Discussion:

MTK: intention is to allow UE without supporting L1-RSRP can indicate support of early sync and/or PDCCH order RACH when RTD>CP.

CATT: we share same view as MTK, since RAN4 already agreed to decouple LTM and L1-RSRP.

HW: sometimes TSSB in PDCCH order RACH is not zero. For this case, regardless of whether UE support RTD>CP or not, there is additional time already, for UE to perform T/F tracking.

MTK: that’s exactly why we proposed this. UE may not support this tracking if RTD>CP, e.g. UE doesn’t have extra FFT.

HW: how about we define some interruption for this case, rather than updating capability?

QC: we don’t understand why this cannot be supported by UE. UE shall have at least rough time info of the target cell, based on L3 MO.

E///: is this only for UE which support L1-RSRP?

Agreement:

Issue 1-5-2: Capability for RACH-less LTM cell switch

- Proposals

  - Proposal 1 (Ericsson, QC): RAN4 to define the following UE capability:

    - RACH-less LTM cell switch can be conducted to one of ‘N’ cells to which the UE most recently transmitted the ‘PDCCH-order PRACH’ except for the cell configured as SCell.

      - N = {[1], 2, …, 7}, if not reported, N=8.

      - Granularity: Per UE

- Recommended WF

  - Need more clarification and discussion.

Discussion:

QC: if it is too late for new UE capability, alternative way is to update the RAN1 capability 45-5.

Agreement:

Issue 1-4-1-1: T/F tracking when TRS as QCL source in cell switch delay

- Proposals

  - Option 1 (CMCC):

    - It is proposed to consider TRS as QCL source RS for cell switch and early TCI state.

  - Option 2 (Huawei):

    - If TRS is configured as a resource RS in TCI state,

      - the current RACH based LTM cell switch delay can keep unchanged with using SSB for T/F tracking.

      - for RACH-less based LTM cell switch delay, either using SSB or using TRS for T/F tracking can work.

  - Option 3 (Nokia)

    - Add TRS as a possible QCL source for T/F tracking in RAN4 cell switch delay requirements.

  - Option 4 (Ericsson, QC):

    - modify the TCI known condition to following

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

[-	The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or]

[-	The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

    - RAN4 to clarify, if TCI state in LTM Cell Switch command is “unknown”/not activated, UE should perform the cell switch with additional time for T/F tracking in the cell switch delay.

- Recommended WF

The proposals are discussing T/F tracking time needed in the following case:

|  | Early TCI state activation | TCI state indication in cell switch command |
| --- | --- | --- |
| Source RS | SSB | TRS |

In moderator understanding, whether to skip T/F tracking during cell switch delay is agnostic to the Type A source is SSB or TRS for early T/F tracking as long as they are on the same QCL chain.

  - Recommend agree on

    - Confirm current cell switch delay requirements are applicable to the case that TRS is configured as a QCL source in the indicated TCI state in cell switch command.

Discussion:

CMCC/Nokia: following the recommended WF, we also need to update Tfirst-ssb.

MTK: we prefer to keep exisitng Tfirst-ssb. MCS is not expected to be very high immediately after cell switch.

HW: based on RAN1 design, SSB can be the source. Current RAN4 requirement can still work.

Agreement:

- Confirm current cell switch delay requirements are applicable to the case that TRS is configured as a QCL source in the indicated TCI state in cell switch command.

- FFS whether Tfirst-ssb in cell switch delay requirements need to be udpated, e.g. based on TRS.

Issue 1-1-1: SFN acquisition

38.133:

The requirements for inter-frequency cell switch to an FR2 target cell in this clause are only applicable, when - network has configured UE to perform L3 measurement with SSB index or L1 measurement for the target cell before the cell switch command, or - the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

38.211:

For handover purposes to a target cell in paired or unpaired spectrum where the target cell uses Lmax=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in clause 8.1 of [5, TS 38.213] is not equal to 10 ms. For inter frequency handover purposes where the source cell is either in paired or unpaired spectrum and the target cell is in unpaired spectrum and uses Lmax=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts.

As a very soft tone is used in RAN1 spec “UE may assume”, proponent pointed out that  NW may not strictly follow this especially for RACH-less scenario.

- Proposals

  - Option 1 (Apple):

    - in RRM requirements applicability of LTM cell switch delay and PDCCH order RACH delay, clarify that SFN between serving and target cell should be the same even for inter-frequency target cell in FR1. Otherwise, additional time for UE to perform SFN acquisition is expected as for minimum requirements.

- Recommended WF

  - Need more discussion.

Discussion:

QC: we probably can use the wording in LTE HO requirement.

Agreement:

FFS the following options:

- Option 1:

  - In cell switch delay requirements, add the applicability that UE is not required to obtain SFN information.

- Option 2:

  - in RRM requirements applicability of LTM cell switch delay and PDCCH order RACH delay, clarify that SFN between serving and target cell should be the same even for inter-frequency target cell in FR1.

- Discuss the wording in CR.

Issue 1-4-3-1: Which cell(s) TLTM-RRC-processing = 0 apply to when candidate cells configured are more than UE capability?

- Proposals

  - Proposal 1 (Huawei, CTC):

    - TLTM_RRC-processing =0 applies to the cells with early TCI activation or early PDCCH order RACH, provided that the number of these cells doesn’t exceed UE capability  [Fast processing of LTM candidate cell RRC configuration].

    - Otherwise TLTM_RRC-processing =0 applies to the latest cell(s) with early TCI activation or early PDCCH order RACH before LTM cell switch command.

  - Proposal 2 (MTK):

    - When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, UE will perform early RRC decoding on the last X cells which TCI state activation MAC-CE or PDCCH order command is sent for and

      - NW will not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

      - If NW deactivates all the TCI states of a candidate, this cell will be removed from the early RRC decoding list until any of its TCI states is added back again.

  - Proposal 3 (ZTE, Nokia): When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed.

  - Proposal 4 (Ericsson, QC)

    - Fast RRC processing is applicable to the following candidate cells (ltm-CandidateConfig):

      - The ltm-CandidateConfig IEs associated with at least one active TCI state

      - The ltm-CandidateConfig IEs associated with previously performed PDCCH-order PRACH.

      - The current serving cells and the cells inside the ltm-CandidateConfig, chosen by the above condition, across cell groups (i.e. MCG and SCG) is not larger than maxServingAndCandidteCells

      - If the number of the ltm-CandidateConfig IEs associated with active TCI state and PDCCH-order PRACH transmission is larger than maxLTMCandidateConfig, the ltm-CandidateConfig IEs for fast RRC processing are chosen in reverse chronological order of Candidate Cell TCI States Activation MAC CE and PDCCH-order PRACH, i.e. maxLTMCandidateConfig ltm-CandidateConfig IEs with the most recently activated TCI states and PDCCH-order PRACH transmission. And in case a tie-break rule is needed, the ltm-CandidateConfig associated with the most recent PDCCH-order PRACH transmission will be chosen.

- Recommended WF

The common part of the proposals are:

    - If the total number of stored cells does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH,

    - Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states and PDCCH-order PRACH transmission within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

The divergence is how to avoid misunderstanding of “most recent” candidates in the 2nd bullet.

    - Option 1: add some limitation to avoid mis-understanding

      - NW will not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

    - Option 2: in case a tie-break rule is needed, the ltm-CandidateConfig associated with the most recent PDCCH-order PRACH transmission will be chosen.

This is also related to Issue 1-2-1: Whether to consider early TCI state activation for multiple cells at the same time.

  - Recommend agree on:

    - If the total number of stored cells does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH,

    - Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states or PDCCH-order PRACH transmission within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

      - This is at least applicable to the case that NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

      - FFS: Whether to support and further optimize the case that NW triggers TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

Discussion:

Agreement:

Issue 3-1-1: Definition of L1-RSRP measurement relative accuracy

- Proposals

  - Proposal 1 (MTK):

    - The intra-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on the same frequency, or between any two SSBs measured on the same cell.

    - The inter-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on a different frequency.

- Recommended WF

  - Need more discussion.

Discussion:

CMCC: we understand the motiation. However, it may cause misalignment to existing repot mapping.

Agreement:

Issue 1-2-2: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case

RAN4#111 < Agreement>: - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - Same agreement applies to the definition of Tfirst-SSB in PDCCH order RACH delay requirement.

- Proposals

  - Option 1 (Apple, MTK):

    - remove the FFS regarding support of inter-frequency L1 measurement with MG in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements.

  - Option 2 (ZTE, vivo):

    - Confirm the FFS.

- Recommended WF

  - Need more discussion.

Discussion:

Apple: this is to allow UE without L1 RSRP supporting to support early TCI activation for inter-frequency case.

Vivo: T/F tracking is assumed to be based on L1 RSRP measurement.

ZTE: we can compromise to option 1.

Agreement:

Issue 1-4-1-2: Conditions of Tfirst-RS =0 in cell switch delay

- Proposals

  - Proposal 1 (Nokia): For cell switch with L1 measurement: Extend the condition for Tfirst-RS = 0 “the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms]” as much as feasible from the agreed 160 ms, to TCI state activation delay stated in section 8.25.3 + 160 ms.

  - Proposal 2 (Nokia): For cell switch with L1 measurement: Tfirst-RS and TRS-proc = 0 in cell switch delay if UE successfully completed PDCCH ordered RACH preamble transmission within 160 ms before the cell switch command, and if the SSB in the PDCCH order is associated to the target TCI state.

  - Proposal 3 (Nokia): For cell switch without L1 measurement in FR1: Tfirst-RS and TRS-proc = 0 in cell switch delay if the measurement period of the SSB associated to target TCI state is not larger than 160 ms.

  - Proposal 4 (Ericsson, QC): In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than 480ms and the L3 measurement interval is larger than 160 ms, or

    - the time gap between early RACH transmission and cell switch command is larger than 480ms and the L3 measurement interval is larger than 160 ms.

- Recommended WF

  - Need more discussion.

Discussion:

Agreement:

- For cell switch with L1 measurement: Extend the condition for Tfirst-RS = 0 “the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms]” from the agreed 160 ms, to TCI state activation delay stated in section 8.25.3 + 160 ms.

  - Discuss the wording in CR.

- FFS the following assumption

  - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than [480ms] and the L3 measurement interval is larger than 160 ms, or

    - the time gap between early RACH transmission and cell switch command is larger than [480ms] and the L3 measurement interval is larger than 160 ms.

Issue 1-2-5: Conditions to support unknown TCI state in FR1 for early TCI state activation

RAN4#111

< Agreement>: Unknown TCI state in FR1 for early TCI state activation is supported with the following conditions: - UE has reported beam-level L3 measurement result of the associated SSB of the TCI state within [1280ms or 5 seconds] before the LTM TCI state activation command. - SNR of the associated SSB is above -3dB.

- Proposals

  - Option 1 (Huawei):

    - Unknown TCI state in FR1, provided the following conditions are met:

      - UE has reported L3 measurement result with the associated SSB index of the TCI state within 1280ms before the LTM TCI state activation command.

      - SNR of the associated SSB is above -3dB.

  - Option 2 (Ericsson, QC)

    - Unknown TCI state activation requirements are applicable if the TCI state activated is based on the measurement report (e.g., L3-RSRP or L1-RSRP) within last 5 seconds and the SNR of the associated SSB is above -3dB.

- Recommended WF

  - Need more discussion.

Discussion:

Agreement:

Issue 1-4-5-1: Whether are cell switch delay requirements applicable to FR2 without L1 measurement report?

- Proposals

  - Option 1 (OPPO):

    - Yes when SNR of the TCI state≥ -3dB and TCI state has been activated, it is also required that the RS associated to the target TCI state is available at least every 1280ms after TCI state activation command.

  - Option 2 (Nokia): Yes with the following conditions:

    - The target TCI state in the cell switch command is activated not more than 1280 ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or

    - The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every 1280 ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB

  - Option 3 (Ericsson, QC): Yes with the following conditions:

    - [-	The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or]

    - [-	The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

  - Option 4 (MTK):

    - Not to extend cell switch delay requirements to FR2 without L1 measurement report

- Recommended WF

  - Need more discussion.

Discussion:

Agreement:

Issue 3-1-2: TCI state configurations

- Proposals

  - Proposal 1 (vivo): In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

- Recommended WF

  - Need more discussion.

Discussion:

Agreement:

# Improvement on SCell/SCG setup delay

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay

- Proposals

  - Proposal 1 (Nokia):

    - Introduce test cases with valid reporting for FR1 and FR2 also for the case when the UE is configured with EMR measurements.

- Recommended WF

  - Need more discussion.

Discussion:

Agreement:

Issue 2-1-1: Mis-alignment between RAN2 and RAN4 spec when measReselectionCarrierListNR is not configured

TS 38.331 clause 5.7.10.3 1> if the reselectionMeasurementReq is included in the UEInformationRequest: 2> if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig; 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig; 2> else: 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available; 38.133 clause 4.3 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

- Proposals

  - Option 1 (ZTE):

    - RAN4 should confirm what the UE behavior is if measReselectionCarrierListNR is not present. UE will report any measurement results if available or UE will not report any measurement results.

- Recommended WF

  - Need more discussion.

Discussion:

Agreement: