3GPP TSG-RAN WG4 Meeting # 112											R4-2413872

Maastricht, Netherlands, 19th ‒ 23rd August, 2024

Agenda item:			5.27.4

Source:	Moderator (Samsung)

Title:	Ad-hoc minutes for [112][208] NR_MIMO_evo_DL_UL

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the below issues:

Topic #1: RRM core requirements maintenance

Reference: Agreement in GTW on Monday:

Issue 1-1-1-b: Reference DL timing point for PUCCH/PUCCH/SRS

Agreement: - For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAG as the uplink signal.

Issue 1-1-1-c: Reference DL timing point for PDCCH order RACH for CFRA in intra-cell case.

Last meeting, the association in RAN1 LS is confirmed to be used.

- Proposals

  - Proposal 1: Confirm the wording in CR R4-2413010 (Apple, Ericsson)

    - For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the [active] DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

  - Proposal 2: (Huawei)

    - For PRACH transmission for intra-cell multi-DCI based multi-TRP operation with Two TAs,

      - for PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 0,

        - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0.

        - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1.

      - For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 1,

        - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 1.

        - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with a coresetPoolIndex with value 0.

- Recommended WF

  - Discuss based on the CR R4-2413010. The differences in proposals are whether the DL TCI states(s) is active or not to remove the square bracket. NTA = 0 for PRACH.

  - Consider the agreement in GTW, discuss on:

    - For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the [active] DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

Discuss:

E///: In the offline discussion, no concern from companies. The only difference is what we agreed on Monday with removal “active”

HW: Technically, it is correct. But whether to capture in RAN4 spec like this.

Agreement:

- For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

Issue 1-1-1-a: For PDCCH order RACH, define uplink timing and DL timings association in inter-cell case in RAN4 spec:

- Proposals

  - Proposal 1: Yes (Apple, Nokia, Samsung, Huawei, Ericsson)

  - Proposal 1a:

    - For PRACH transmission for inter-cell multi-DCI based multi-TRP operation with Two TAs,

      - If “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of [active] DL TCI state(s) of the reference cell associated with the first TAG, where NTA offset is as defined in n-TimingAdvanceOffset.

      - if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of [active] DL TCI state(s) of the reference cell associated with the second TAG, where NTA offset is as defined in n-TimingAdvanceOffset2.

- Recommended WF

  - Discuss based on the CR R4-2413010. The differences in proposals are whether the DL TCI states(s) is active or not.

  - Consider the agreement in GTW, discuss on:

    - For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order,

      - If “PRACH association indicator” is 0, the reference point is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with PCI of the serving cell [with the first TAG].

      - If “PRACH association indicator” is 1, the reference point is the first detected path (in time) of one of the corresponding downlink reference signal(s) of  DL TCI state(s) of the reference cell associated with the additional PCI  [the second TAG].

  - Proposal 1a (HW)

    - if “PRACH association indicator” is 0, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG, where NTA offset is as defined in n-TimingAdvanceOffset.

if “PRACH association indicator” is 1, the uplink transmission timing for PRACH takes place NTA offset×Tc before the reception of the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG, where NTA offset is as defined in n-TimingAdvanceOffset2.

Discuss:

E///: The CR is the same as RAN1 spec. It should be agreeable.

QC: postpone the CR. Prefer Huawei’s version.

Nokia:

MTK: RAN1 has already defined the association. We just want to define the timing of reference. Just want to use the general way.

Apple: No reference timing in RAN1.

Agreement:

- For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order,

  - If “PRACH association indicator” is 0, the reference point is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with PCI of the serving cell [with the first TAG].

  - If “PRACH association indicator” is 1, the reference point is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the additional PCI  [the second TAG].

E/// needs to further confirm the wording in [] by this week.

Issue 1-2-1: For mDCI mTRP, whether to need additional DL RS tracking time for UL TCI state switching if UE supporting two TAs (RTD<CP and RTD>CP)?

- Proposals

  - Proposal 1: Yes (Apple, Samsung)

  - Proposal 1a: (Apple)

    - For multi-DCI multi-TRP with 2TA for separate UL TCI state switch, if no DL TCI is in the active TCI state list associated with the TAG of the target UL TCI state choose one of the 2 options -

      - Additional time for DL timing reference tracking is needed for the UL TCI state switch

      - No requirements are applicable for this case

  - Proposal 1b: (Samsung)

    - For joint TCI state, the UE is not expected to transmit on UL based on the target TCI state before UE completes the DL and UL TCI state switch. The DL timing can always be achieved by DL TCI. No additional DL RS tracking time for UL TCI state switching.

    - For separate UL TCI state switch:

      - If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as:

        - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

        - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

        - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex

      - For other cases, no additional DL tracking is needed.

  - Proposal 2: (MediaTek, Nokia, Ericsson)

    - No. Reuse the same requirements as Rel-17

    - TOk-ref = 0 in any case

- Recommended WF

  - TBA

Discuss:

Apple: if companies don’t want to define such case, if it is a corner case, can comprise to no requirements in such case.

Nokia: we still think PDCCH can be achieved before that UL TCI state switching. The previous DL timing can still be used.

QC: UE has to keep the DL timing for uplink for previous DL timing.

HW: UE keep the “cell” level timing.

Nokia: not prefer no requirement in spec. If so, prefer samsung’s proposal.

Agreement:

Option 1:

- No requirement for separate UL state TCI switching if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex. Otherwise, the same as Rel-17 requirements.

Option 2:

- For joint TCI state, the UE is not expected to transmit on UL based on the target TCI state before UE completes the DL and UL TCI state switch. The DL timing can always be achieved by DL TCI. No additional DL RS tracking time for UL TCI state switching.

- For separate UL TCI state switch:

  - If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as:

    - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

    - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

    - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex

  - For other cases, no additional DL tracking is needed.

Issue 1-2-2: Update RLM/BFD/CBD requirements for restriction when RTD is larger than CP

- Proposals

  - Proposal 1: (MediaTek)

    - Add the measurement restriction and applicability for RLM/BFD/CBD when RTD is larger than CP

- Recommended WF

  - Discuss Proposal 1 and CR R4-2411708

Discuss:

Vivo:  no need to add further restriction. MRTD/MTDD has already defined. Further check on CR.

Agreement:

Issue 1-1-2: Whether to add the condition to check MTTD before applying gradual timing adjustment?

- Proposals

  - Option 1: (Apple)

    - For multi-DCI multi-TRP with 2TA the gradual timing adjustment is applied until the timing error for each TAG is within ±Te or the TTD between pair of TAGs doesn’t exceed the MTTD requirement for 2TAs.

- Recommended WF

  - Discuss option 1.

Discuss:

Agreement:

Issue 1-2-3: Further clarification for OL definition for mDCI mTRP scenario

[Background]

In RAN4#111 agreement: OL = 1 if the SSB overlaps or is adjacent to the SSB from the other TRP in FR2 and the SSB is associated to the TRP with the lowest corestPoolIndex, 0, otherwise. In RAN4#109 agreement: SSB overlaps or adjacent to SSB from other TRP in FR2 and SSB periodicity is less than that of other TRP

- Proposals

  - Proposal 1: (Nokia)

    - Further clarify based on previous agreement:

          - OL = 1, if the SSB overlaps or is adjacent to the SSB from the other TRP, and

            - the SSB periodicity is less than that of other TRP, or

            - the SSB periodicity is the same of that of the other TRP and the SSB is associated to the TRP with the lowest coresetPoolIndex.

          - Otherwise, OL = 0.

- Recommended WF

[Moderator view]: According to our understanding, it is aligned with previous agreements and the current spec cover both cases. No CR. Check with companies.

Topic #2: RRM performance requirements maintenance

[Background]: In last meeting, the WF is: for this case, Reported TDCP index is [equal to or] larger than 6, with at least 80% probability

Issue 2-1-1: For high doppler condition (300Hz) + 30kHz SCS TDD, SNR = 10dB, TDCP test requirements in the test?

- Proposals

  - Option 1: (Ericsson)

    - Reported TDCP index is larger than 6, with at least 80% probability. Confirm to delete “equal to 6”

- Recommended WF

  - To check whether option 1 is agreeable

Discuss:

Confirm with Qualcomm with option 1.

Agreement:

Delete [equal to 6]