3GPP TSG-RAN WG4 Meeting #112															R4-2413885

Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

Agenda item:			8.21.3

Source:	Ericsson.

Title:	WF on Netw_Energy_NR_enh

Document for:	Approval

# Introduction

Note: The detail open issue list is captured in the moderator summary R4-2411817.

# Topic #1: On-demand SSB(OD-SSB) requirements

### Sub-topic 1-1: General OD-SSB requirements

Issue 1-1-1: OD-SSB work plan

- Agreement:

  - RAN4 to first focus on the MAC CE based single SCell activation case as starting point.

  - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed.

Issue 1-1-2: OD-SSB scenarios

- Agreement:

  - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios.

    - Case #1 and Scenario #2

    - Case #1 and Scenario #2A

    - Case #2 and Scenario #2

    - Case #2 and Scenario #2A

    - Note 1: Follow RAN1 definition on the scenarios and cases.

    - Note 2: This list can be updated pending on RAN1 further agreement.

Issue 1-1-3: Deactivated SCell measurement requirement

- Agreement:

  - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement.

    - Includes both FR1 and FR2-1

### Sub-topic 1-2: OD-SSB requirements(Case 1)

Deactivated SCell L3 measurement part

Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement

- Proposals

  - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle).

  - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell.

  - Other option is not precluded

Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication

- Agreement:

  - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered.

    - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion.

Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation

- Wayforward:

  - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A).

  - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A).

  - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement.

  - Other option is not precluded

# Topic #2: On-demand SIB1(OD-SIB1) requirements

Issue 2-1-1: RRM impact of OD-SIB1

- Agreement

  - Postpone RAN4 work until any RRM impact is identified.

# Topic #3: Adaptation of Common Signals and Channels

### Sub-topic 3-1: SSB Adaptation requirements

Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode

- Agreement

  - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode.

  - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress

Issue 3-1-2: SSB adaptation impact in RRM requirement

- Agreement: SSB adaptation impact in RRM requirement

  - RAN4 to discuss the L1/L3 measurement requirement impact as starting point.

- Wayforward:

  - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point.

    - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

### Sub-topic 3-2: Impact on Paging Adaptation requirements

Issue 3-2-1: Paging adaptation

Agreement:

  - Deprioritize Paging adaptation until the RAN1 concrete conclusion is available.

### Sub-topic 3-3: Impact on PRACH Adaptation requirements

Issue 3-3-1: PRACH adaptation

Agreement:

  - Deprioritize PRACH adaptation until the RAN1 concrete conclusion is available.