3GPP TSG-RAN WG4 Meeting # 112					                            R4-2413899

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.22.5

Source:	vivo

Title:	WF for RRM core requirements for LP-WUSWUR

Document for:	Approval

- Topic #1: RRM core requirements for LP-WUS/WUR

- Sub-topic 1-1 General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Agreement:

- Support case #3

  - For serving cell measurement, further discuss:

    - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR

    - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR

    - Option 3: It is up to UE implementation whether to combine the measurement across the two radios.

    - Other options are not precluded.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

  - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

Recommendations:

Suggest to discuss issues related to higher priority frequency layers under 1-1-11.

Continue discuss case 2 and other issues.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Recommendations:

Suggest to discuss this issue on the separately issue listed as the following:

- On requirements for evaluating entry/exit criteria for LP-WUR monitoring

- On requirements for evaluating entry/exit criteria for RRM relaxation and serving cell measurement offloading

Issue 1-1-4-1: Entry/exit criteria evaluation when results from MR and/or LP-WUR are avaiable for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Recommendations:

Companies are encouraged to provide views on whether combine results from LP-WUR and MR is needed or if so, solutions on how to combine

Issue 1-1-4-2: Reference signals and measurement types to be used for Entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement

Recommendations:

To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself.

Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation

Recommendations:

To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself.

Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring

Recommendations:

To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself.

Issue 1-1-8: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7

Issue 1-1-9: LP-WUR status

Issue 1-1-10: UE measurement behivour after receiving LP-WUS signal

Recommendations:

Maybe it is not a RAN4 issue, suggest to check RAN1/2 for clarification.

Issue 1-1-11: Considerations on higher priority frequency layer

Recommendations:

Companies are encouraged to bring solutions/views on this issue.

Issue 1-1-12: Impact on specification

Recommendations:

Postpone the discussion until more RAN4’s conclusion are available.

- Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Accuracy requirements

Recommendations:

How and where to capture the accuracy requirement can be discussed later.

Issue 1-2-2: Possible interruption related requirements for LP-WUR

Recommendations:

Companies are encouraged to bring solutions/views on this issue.

Issue 1-2-3: Possible cell selection evaluation requirements for fully offloading case based on LP-WUR

Recommendations:

Companies are encouraged to bring solutions/views on this issue.

Issue 1-2-4: LP-WUR operating carrier frequency

Issue 1-2-5: Timeline on RRM requirement evaluation work

Recommendations:

Postpone the discussion on this issue.

Issue 1-2-6: Nubmer of Rx for LP-WUR requirements

Recommendations:

Encourage companies to check whether P1 of this issue is agreeable or not.

Issue 1-2-7: Other considerations

- Sub-topic 1-3 MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

Issue 1-3-2: On neighbour cell and serving cell relaxation factor

Issue 1-3-3: Accuracy for relaxed MR measurement

- Sub-topic 1-4 LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Recommendations:

Suspend discussion on this topic until more progress from other group.

- Sub-topic 1-5 Others

Issue 1-5-1: eDRX related

Recommendations:

Suggest postpone discussion on this issue.

- Topic #2: Simulation assumptions and results

- Sub-topic 2-1 On simulation assumptions and parameters

Issue 2-1-1: SINR setting

Agreement:

  - As starting point for RAN4 RRM simulation purpose only, -3dB, 0.5dB, 2dB dB Ês/Iot value is used for serving cell for both OOK based and OFDM based LP-WUR.

    - The SINR value can be updated if any update on the noise figure value in RF session.

  - FFS the side condition for RAN4 requirement.

Issue 2-1-1-1: SNR setting for serving and interference cell derivation from SINR setting

Agreement:

  - Consider two cells for RAN4 simulation: one serving cell and one interference cell.

Agreement:

To derive SNR for serving cell and interference cell from serving cell Ês/Iot, a relationship for the SNR or transmission power between serving cell and interference cell need be pre-defined.

Consider the SNR/transmission power of the interference cell is 9 dB or 6 dB lower compared with that of the serving cell. Other value can be considered.

Issue 2-1-1-2: Same or different SINR setting for OOK based and OFDM based LP-WUR

Recommendations:

Covered by issue 2-1-1. Close this issue

Issue 2-1-2: Measurement metrics

Recommendations:

Follow RAN1’s conclusion

Issue 2-1-3: Time/frequency error

Agreement:

Residual frequency error:

OFDM based receiver [5] ppm

OOK based receiver [ 10 20] ppm

Issue 2-1-4: Accuracy baseline for simulation

Issue 2-1-5: Measurement interval

Agreement:

Use 320ms for SSB based LP-WUR firstly.

- Sub-topic 2-2 Simulation assumptions and results summary

Simulation assumptions

Note:

Parameters highlighted by green color are agreed during the meeting.

Other parameters are only for recommendation and information purpose.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | [Data, SSB and LP-SS have the same SCS] |
| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| Measurement period (in number of measurement samples) | [4, 5, other number could be studied upon a need] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [ 10 20] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: company report |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |