3GPP TSG-RAN WG4 Meeting #112											                  R4-2413900

Maastricht, The Netherlands, 19 – 23 August, 2024

Agenda item:			8.19.4

Source:	Huawei, HiSilicon

Title:	WF on SBFD RRM

Document for:	Approval

- Sub-Topic #1: Agreements

### Sub-topic 1-1: Work plan

Agreements (from online session)

- The work plan in R4-2412534 is approved.

#### Issue 2-1-1: Scope of requirements for UE-to-UE CLI handling

Agreements (from online session)

- RAN4 to define RRM requirements for L1 based UE-to-UE CLI measurement and reporting.

Issue 2-1-7: Side condition

Agreements (from online session)

- RAN4 to define side conditions for L1 based UE-to-UE CLI measurement requirements.

- RAN4 to discuss at least time offset between DL timing and SRS arrival timing, SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP.

  - The RAN1 progress will be taken into account when discussing the values for the side conditions.

#### Issue 2-1-8: Measurement period

Agreements (from online session)

- RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements,

- RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design.

  - Number of shots for simulation: 1, 2, 3.

  - Companies to bring proposals on other simulation parameters in the next meeting.

- Issue 2-2-1: Scope of requirements for gNB-to-gNB CLI handling

Agreements (from online session)

- RAN4 not to define RRM requirements for gNB to gNB CLI handing.

- Issue 2-3-1: Requirements for legacy UE

Agreements (from online session)

- The legacy UEs shall perform and operate according to the legacy RRM requirements and no new RAN4 RRM requirement for legacy UEs, despite being served by a gNB operating with SBFD.

- Clarification on the applicability of the existing RRM requirements in RAN4 spec is not precluded.

- The legacy UEs refer to non-SBFD aware UE.

- Issue 2-3-2: Requirements for SSB based measurement

Agreements (from online session)

- For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4.

- Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell.

# Sub-Topic #2: For further discussion potential issues

### Sub-topic 2-1: RRM impacts of UE-to-UE CLI handling

#### Issue 2-1-2: Measurement quantities

Recommended WF

- RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. This can be revisited based on further RAN1 agreement.

- FFS if other measurement quantities to be considered in the core requirements based on further RAN1 agreement.

#### Issue 2-1-3: Baseline for defining requirements

Recommended WF

- Both R16 CLI measurement requirements and L1-RSRP measurement requirements are to be considered for defining requirements for L1 based UE-to-UE CLI measurement.

- RAN4 can directly discuss the requirements for L1 based UE-to-UE CLI measurement in next meeting.

#### Issue 2-1-4: Measurement methods

Recommended WF

- RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement.

- Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements.

#### Issue 2-1-5: Rx beam

Recommended WF

- RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement.

- Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements.

#### Issue 2-1-6: Measurement resources

Recommended WF

- RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

#### Issue 2-1-7: Side condition

Options for further discussion and down-selection (for information purpose):

- Time offset between DL timing and SRS arrival timing

  - Option 1: Rel-16 CLI SRS RSRP assumption

  - Option 2: Remove cell phase error from option 1

  - Other options are not precluded, pending on RAN1 progress.

- SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP

  - Option 1: Rel-16 CLI SRS RSRP assumption

  - Other options are not precluded.

#### Issue 2-1-9: Measurement reporting

Recommended WF:

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement

- FFS whether L1-RSRP measurement reporting requirement can be re-used.

#### Issue 2-1-10: Measurement accuracy

Recommended WF:

- RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

#### Issue 2-1-11: Scheduling and measurement restriction

Recommended WF

- RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed.

- FFS the impact of measurement methods and Rx beam assumption

#### Issue 2-1-12: Measurement capability

Recommended WF

- FFS whether RAN4 needs to discuss measurement capability for L1 based UE-to-UE CLI measurement in terms of number of resources UE shall be able to monitor.

#### Issue 2-1-13: Report mapping

Recommended WF

- FFS whether RAN4 needs to discuss report mapping for L1 based UE-to-UE CLI measurement.

- FFS whether R16 report mapping can be re-used.

### Sub-topic 2-2: RRM impacts of gNB-to-gNB CLI handling

### Sub-topic 2-3: RRM impacts of SBFD operation

- Issue 2-3-2: Requirements for SSB based measurement

Recommended WF

RAN4 further discuss impact on SSB based neighbour cell measurement in RAN4.

- Issue 2-3-3: Requirements for CSI-RS based measurement

Recommended WF

- RAN4 to discuss the impact of SBFD operation on the CSI-RS measurement requirements.

- Issue 2-3-4: Requirements for scheduling restriction

Recommended WF

- RAN4 to discuss whether scheduling restriction requirements are impacted due to SBFD operation based on RAN1 agreements.

- Issue 2-3-5: Requirements for RACH requirements

Recommended WF

- RAN4 to discuss whether RACH requirements are impacted due to SBFD operation based on RAN1 agreements.

- Issue 2-3-6: Requirements for UL PC and or spatial relation update

Recommended WF

- RAN4 to discuss whether requirements for UL PC and or spatial relation switch are impacted due to SBFD operation based on RAN1 agreements.

- Issue 2-3-7: Requirements for MG and BWP switch

Recommended WF

- RAN4 to discuss whether to clarify the term “UL slot” in MG and BWP switch requirements when SBFD operation is enabled.

- Issue 2-3-8: Requirements for UL resource muting

Recommended WF

- RAN4 further to discuss whether additional RRM requirements would be defined for UL resource muting for PUSCH feature.

- Issue 2-3-9: Requirements for generic SBFD operation

Recommended WF

- RAN4 to discuss whether there is any RRM impact e.g. to scheduling restrictions/interruptions requirements due to SBFD operation.

- Issue 2-3-10: Limits on the maximum number of DL/UL switching

Recommended WF

- RAN4 to discuss whether to put limits on the maximum number of DL/UL switching during SBFD slots within the SBFD periodicity