3GPP TSG-RAN WG4 Meeting # 112					       			               R4-2413907

Maastricht, Netherlands, August 19 ‒ August 23, 2024

Agenda item:			5.29.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR network energy saving

Document for:	Approval

- Topic #1: Core requirements maintenance

- Sub-topic 1-1 SSB-less related

Issue 1-1-1: Power difference conditions

Agreement:

  - For EPRE difference within X dB, keep the existing requirement.

    - X equals to or larger than 12 dB

  - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence.

    - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB.

  - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec.

  - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon.

Additional Agreement: X = 12; Y = [30]

Issue 1-1-2: Multiple SSB-less SCells activation

Agreement:

- For TRS-based multiple SSB-less SCell activation, for each to-be-activated SCell, single SCell activation requirements apply respectively.

- For A-TRS-based and mixed scenario (TRS-based and A-TRS based) for multiple SSB-less SCell activation, for each to-be-activated SCell, single SCell activation requirements apply respectively.

Issue 1-1-3: Relation to R15 intra-band SSB-less

Agreement:

  - No requirements when the SSB-less SCell is configured with QCL source to both intra-band contiguous and inter-band Cells. And no spec impacts expected.

Issue 1-1-4: Neighbour cells on carrier of SSB-less SCell

Agreement:

  - Intra-frequency/Inter-frequency definition:

    - Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition.

  - FFS: With/without gap:

    - Whether UE can support inter-f w/o gap following existing rules (conditions in 9.3.1, e.g supporting of R16 inter-frequency without gap, R17 R18 NeedForGap)

- Sub-topic 1-2 NES-based CHO related

Issue 1-2-1: When CHO condition is not met anymore

Discuss the CR directly.

- Topic #2: Performance part for NES

Issue 2-1-1: Test case for SSB-less

Discuss in CR directly.

Issue 2-1-3: Test case for Cell DTX

Discuss in CR directly.