3GPP TSG-RAN4 Meeting #112          		R4-2413961

Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4736 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on fast CA/DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update capability names ased on RAN2 capability design. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update capability names based on RAN2 capability design. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of 1st change >>

4.7	Measurement report for fast CA/DC setup

4.7.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for:

-	UE supporting measValidationReportEMR-r18 and configured with  measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers.

-	UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionCarrierListNR-r18 by higher layers.

A 	UE supporting measValidationReportEMR-r18  and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report the results of the idle mode measurement which is specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3.

A UE supporting measValidationReportReselectionMeasurements-r18 shall report the results of the idle mode measurement specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3.

UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

4.7.2	Void

4.7.3	Measurement Report Requirements

A UE shall perform validity check and report valid measurement results:

-	if the UE supports  measValidationReportEMR-r18 and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16,

-	if the UEsupports measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18

The measurement results are considered valid if the following conditions are met for the validity check:

-	the measurements are performed before msg1 transmission for RRC resume/setup request within the last:

-	 measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or

-	 measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18,

-	the measurement results satisfy measurement accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured.

If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

## << End of 1st change >>

<< Start of 2nd change >>

5.8	Measurement report for fast CA/DC setup

5.8.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for:

-	UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers.

-	UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionCarrierListNR-r18 higher layers.

A UE supporting  measValidationReportEMR-r18 and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report the results of the inactive mode measurement specified in the clause 5.4 , and according to the measurement reporting requirements specified in the clause 5.8.3.

A UE supporting measValidationReportReselectionMeasurements-r18 shall report the results of the inactive-mode cell re-selection requirements specified in clause 5.1.2, and according to the measurement reporting requirements specified in the clause 5.8.3.

UE is not required to report for fast CA/DC setup from any measurements during INACTIVE mode for carriers which are not configured by higher layers.

## << End of 2nd change >>