3GPP TSG-RAN4 Meeting #112	R4-2413977

# Maastricht, The Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM requirements for RedCap positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The UE buffering capability N3 is not accounted in the number of hops UE can take within an MG occasion. - Current definition for Lavailable_PRS,i only accounts for one MG occasion, and does not support measurement of different PRS resources acorss differnet MG occasions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add an upper bound to the number of hops (Nhop) within an MG occasion for N3 capability. - Update the requirements on Lavailable_PRS,i by considering measurement of different PRS resources in different MG occasions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for PRS measurement with FH are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9A.2.6, 9.9A.3.6, 9.9A.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.9A.2.6	Measurements Period Requirements with FH

##### 9.9A.2.6.1	Measurements Period Requirements with FH with MG

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in Clause 9.9A.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in Clause 9.9A.2.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

-	Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

-	Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i*Lper-hop,i

where,

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 9.9A.2.6.1-1, and RRTFH is the retuning time between Rx hops as reported by UE in dl-PRS-MeasurementWithRxFH-RRC-Connected via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.2.6.1-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled in the UE capability (FG 41-5-1) for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times, MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 9.9A.2.6.1-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

- BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18, and

-	total BW of all hops requested by LMF via maximumPRS-BandwidthAcrossAllHopsFR1-r18

- Nhop is number of hops within a single MG occasion as define above

- BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16

- BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18.

<End of Change 1>

<Start of Change 2>

#### 9.9A.3.6	Measurements Period Requirements with FH

##### 9.9A.3.6.1	Measurements Period Requirements with FH with MG

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure PRS-RSRP measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCapUE capability specified in Clause 9.9A.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms as defined Clause 9.9A.3.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

-	Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

-	Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i*Lper-hop,i

where,

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 9.9A.3.6.1-1, and RRTFH is the retuning time between Rx hops as reported by UE in dl-PRS-MeasurementWithRxFH-RRC-Connected via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.3.6.1-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled in the UE capability (FG 41-5-1) for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times, MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 9.9A.3.6.1-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

- BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18, and

-	total BW of all hops requested by LMF via maximumPRS-BandwidthAcrossAllHopsFR1-r18

- Nhop is number of hops within a single MG occasion as define above

- BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16

- BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18.

<End of Change 2>

<Start of Change 3>

#### 9.9A.4.8	Measurements Period Requirements with FH

The requirements in this clause apply when a RedCap UE is requested by the LMF to perform measurements with FH, and UE reports measurement based on multiple hops.

The requirements in clause 9.9A.4.5 shall apply with the following modifications.

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap.

-	Lavailable_PRS,i=j=1JNhop,i,j*Lper-hop,i,jNhop,i*Lper-hop,i, where

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 9.9A.4.8-1, and RRTFH is the retuning time between Rx hops as reported by UE in dl-PRS-MeasurementWithRxFH-RRC-Connected via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.4.8-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled in the UE capability (FG 41-5-1) for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times, MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap and Nhopsslot is the applicable number of hops per slot as defined in Table 9.9A.4.8-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

- BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18, and

-	total BW of all hops requested by LMF via maximumPRS-BandwidthAcrossAllHopsFR1-r18

- Nhop is number of hops within a single MG occasion as define above

- BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16

- BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18.

<End of Change 3>