3GPP TSG-RAN4 Meeting #112	R4-2414015

# Maastricht, The Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on performance requirements for PRS BWA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The RSTD accuracy requirements for PRS BWA are incomplete. - Phase II TCs need to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the RSTD accuracy requirements for PRS BWA - Combine the accuracy for 2-PFL and 3-PFL into one table (to align with the approach how Rx-Tx accuracy for PRS BWA is defined). - Add the missing condition on PFL spacing. - Based on work split in R4-2410191, define TCs for Phase II, including sets 6-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirements for PRS BWA are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23A, A.7.9.1.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The draftCR is based on Big draftCR R4-2410160 from RAN4#111. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.1.23A		RSTD Measurements Based on PRS Aggregation

#### 10.1.23A.1	Introduction

The requirements in Clause 10.1.23A shall apply, provided the UE has received nr-DL-TDOA-RequestLocationInformation message with nr-DL-PRS-JointMeasurementRequestedPFL-List from LMF via LPP [34] requesting the UE to report one or more DL RSTD measurements defined in TS 38.215 [4] with aggregated measurement. The requirements in Clause 10.1.23A shall apply:

-	when UE is in RRC_CONNECTED state and the measurement is performed with MG or without MG,

-	when UE is in RRC_INACTIVE state,

-	when UE is in RRC_IDLE state.

#### 10.1.23A.2	Measurement Accuracy Requirements

When UE measures RSTD on PRS resources belonging to different PFLs or different PFL combinations, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs or different PFL combinations.

The requirements in this clause for 3-PFL and 2-PFL apply provided that

-	PRS resources linked for aggregation saftisfy all the conditions specified in 38.214 clause 5.1.6.5.3.

-	The spacing between the center frequencies of adjacent PFLs containing PRS resources linked for aggregation does not exceed the nominal channel spacing for intra-band contiguous CA defined in 38.101-1, clause 5.4A.1 for FR1 and in 38.101-2, clause 5.4A.1 for FR2-1..

The accuracy requirements for RSTD measurement based on PRS Aggregation shall be within ±(X+Y+Z+Δ) Tc.

X is defined in Table 10.1.23A.2-1 for AWGN channel and Table 10.1.23A.2-3 for fading channel for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE does not perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23A.2-2 for AWGN channel and Table 10.1.23A.2-4 for fading channel for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

- 	UE does not perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23A.2-5 for AWGN channel in FR1 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

X is defined in Table 10.1.23A.2-6 for AWGN channel in FR2 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

Note: The requriements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

Editor’s Note: values of Y, Z and Δ are FFS.

Table 10.1.23A.2-1: RSTD absolute accuracy in FR1 for AWGN channel for 2-PFL

| Accuracy for 3 PFLs | Accuracy for 2 PFLs | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS SCS | PRS bandwidth per PFL Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 |  | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| TBD | TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 104 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| TBD | TBD |  | 30 | ≥ 132 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| TBD | TBD |  |  | 272 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  | 60 | ≥ 64 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| TBD | TBD |  |  | 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth per PFL, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i among all the aggregated PFLs. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |  |  |

|  |  |  |  | TrepPRS*LPRS/KcombPRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TrepPRS, LPRS ,KcombPRS |  |  |  |  |  |  |  |

Table 10.1.23A.2-2: RSTD absolute accuracy in FR2 for AWGN channel for 2-PFL

| Accuracy for 3 PFLs | Accuracy for 2 PFLs | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS SCS | PRS bandwidth per PFL Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 |  | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| TBD | TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 64 | ≥ 1 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| TBD | TBD |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| TBD | TBD |  | 120 | ≥ 64 | ≥ 1 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| TBD | TBD |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth per PFL, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i among all the aggregated PFLs. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |  |

|  |  |  |  | TrepPRS*LPRS/KcombPRS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TrepPRS, LPRS ,KcombPRS |  |  |  |  |  |  |

Table 10.1.23A.2-3: RSTD absolute accuracy in FR1 for fading channel for 2-PFL

| Accuracy for 3 PFLs | Accuracy for 2 PFLs | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS SCS | PRS bandwidth per PFL Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 |  | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| TBD | TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  | 30 | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  |  | 272 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  | 60 | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  |  | 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth per PFL, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i among all the aggregated PFLs. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS as defined in Table Table 10.1.23A.2-1. |  |  |  |  |  |  |  |  |

|  |  |  |  | TrepPRS*LPRS/KcombPRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TrepPRS, LPRS ,KcombPRS |  |  |  |  |  |  |  |

Table 10.1.23A.2-4: RSTD absolute accuracy in FR2 for fading channel for 2-PFL

| Accuracy for 3 PFLs | Accuracy for 2 PFLs | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS SCS | PRS bandwidth per PFL Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 |  | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| TBD | TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| TBD | TBD |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| TBD | TBD |  | 120 | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| TBD | TBD |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth per PFL, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i among all the aggregated PFLs. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS as defined in Table 10.1.23A.2-2. |  |  |  |  |  |  |  |

|  |  |  |  | TrepPRS*LPRS/KcombPRS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TrepPRS, LPRS ,KcombPRS |  |  |  |  |  |  |

Table 10.1.23A.2-5: RSTD absolute accuracy in FR1 for AWGN channel with reduced number of samples for 2-PFL

| Accuracy for 3 PFLs | Accuracy for 2 PFLs | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS SCS | PRS bandwidth per PFL Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 |  | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| TBD | TBD | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 15 | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  | 30 | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  |  | 272 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  | 60 | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| TBD | TBD |  |  | 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth per PFL, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i among all the aggregated PFLs. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS as defined in Table Table 10.1.23A.2-1. |  |  |  |  |  |  |  |  |

|  |  |  |  | TrepPRS*LPRS/KcombPRS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TrepPRS, LPRS ,KcombPRS |  |  |  |  |  |  |  |

Table 10.1.23A.2-6: RSTD absolute accuracy in FR2 for AWGN channel with reduced number of samples for 2-PFL

| Accuracy for 3 PFLs | Accuracy for 2 PFLs | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS SCS | PRS bandwidth per PFL Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 |  | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| TBD | TBD | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 60 | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| TBD | TBD |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| TBD | TBD |  | 120 | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| TBD | TBD |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth per PFL, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i among all the aggregated PFLs. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS as defined in Table 10.1.23A.2-2. |  |  |  |  |  |  |  |

|  |  |  |  | TrepPRS*LPRS/KcombPRS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TrepPRS, LPRS ,KcombPRS |  |  |  |  |  |  |

#### 10.1.23A.3	Report Mapping

##### 10.1.23A.3.1		Absolute DL RSTD Measurement Reporting

The report mapping as defined in clause 10.1.23.3.1 shall apply.

##### 10.1.23A.3.2		Differential Reporting for DL RSTD Measurement

The report mapping as defined in clause 10.1.23.3.2 shall apply.

##### 10.1.23A.3.3		Additional Path Report Mapping for DL RSTD

The report mapping as defined in clause 10.1.23.3.3 shall apply.

<End of Change 1>

<Start of Change 2: Set 6-4>

#### A.7.9.1.3	RSTD measurement accuracy TC for PRS aggregation in FR2 in RRC_INACTIVE state

##### A.7.9.1.3.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement with PRS aggregation on two PFLs meets the accuracy requirements specified in clause 10.1.23A.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.7.9.1.3.1-1.

Table A.7.9.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 200 MHz bandwidth, TDD duplex mode |

In the test there are four synchronous cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR2. Cell 2 is a neighbour cell on the same NR RF channel as Cell 1. Cell 3 and Cell 4 are neighbor cells in a different NR RF channel #2 in FR2. Cell 1 and Cell 3 are intra-band contiguous, and PRS resources from Cell 1 and Cell 3 are transmitted by the same Tx chain. Cell 2 and Cell 4 are intra-band contiguous, and PRS resources from Cell 2 and Cell 4 are transmitted by the same Tx chain.

GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. The UE is configured with DRX cycle of 0.64s.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 5.6.2.6.

In NR-TDOA-ProvideAssistanceData, there are two NR-linkedDL-PRS-ResourceSetID-PRS-AggregationList. The first list indicates aggregation of resource sets from Cell 1 and Cell 3, and the second list indicates aggregation of resource sets from Cell 2 and Cell 4. In NR-TDOA-RequestLocationInformation, nr-DL-PRS-JointMeasurementRequestedPFL-List is included, and indicates aggregation of PFLs on NR RF channel #1 and NR RF channel #2.

Table A.7.9.1.3.1-2: RSTD accuracy test parameters

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| PRS ARFCN |  | freq1 | freq1 | freq2 | freq2 |
| Duplex mode |  | TDD |  |  |  |
| TDD configuration |  | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 200: NRB,c = 132 |  |  |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | - | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | - | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | - | - |
| TRS configuration |  | TRS.2.1 TDD | - | - | - |
| TCI state |  | TCI.State.0 | - | - | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | - | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | - | - |
| Control channel RMC |  | CCR.3.1 TDD | - | - | - |
| OCNG Patterns |  | OP.3 |  |  |  |
| SSB configuration |  | SSB.3 FR2 |  |  |  |
| SMTC configuration |  | SMTC.1 |  |  |  |
| PRS configuration |  | PRS.1.2 FR2 |  |  |  |
| PRS Resource slot offset | slot | 0 | 4 | 0 | 4 |
| Expected RSTD | s | N/A | 3 | 0 | 3 |
| Expected RSTD uncertainty | s | N/A | 5 | 0 | 5 |
| Time offset with Cell 1 | s | - | 3 | 0 | 3 |
| EPRE ratio of PSS to SSS | dB | 0 |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN |  |  |  |
| Antenna configuration |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. |  |  |  |  |  |

Table A.7.9.1.3.1-3: RSTD accuracy OTA related test parameters

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 5 |  | Rough |  | Rough |  |
| Note1 | dBm/SCSNote3 | -89 |  | -89 |  |
|  | dB | -5.7 | -11.9 | -5.7 | -11.9 |
| PRS-RSRPNote2 | dBm/SCS | -94.7 | -100.9 | -94.7 | -100.9 |
| BB Note4 | dB | -6 | -13 | -6 | -13 |
| IoNote2 | dBm/190.08 MHz Note3 | -55.75 | -55.75 | -55.75 | -55.75 |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.9.1.3.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23A.2.

<End of Change 2: Set 6-4>