# 3GPP TSG-RAN WG4 Meeting #112	R4-2414020

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			5.14.3

Source:	Apple

Title:	WF for NR_RRM_enh3

Document for:	Approval

# Topic #1: Core part maintenance (5.14.1)

Issue 1-1-1: Requirement for the case when NOT all the unknown to-be-activated SCells have L3 report on FR1 band (some have L3 report but others not)

Agreement:

  - Not define RAN4 requirement for the case where not all the unknown to-be-activated SCells are reported in the L3 reporting and cell detection is still needed on some of the unknown SCells on the same FR1 band.

Issue 1-1-3: Uncertainty part for SCell activation requirement with L3 reporting

FFS:

- If UE sends L1 reports earlier than L3 reports and L3 report is later than TCI activation command, the SCell activation delay requirements in TS38.133 clause 8.3.2 apply for the to-be-activated SCell.

Issue 1-1-4: FR1 SCell activation enhancement with one SSB transmitted in ssb-PositionInBurst.

Agreement:

  - SCell activation with L3 reporting does not apply when the single SSB is present.

Issue 1-1-5: FR1 SCell activation enhancement considering the measurement period

Agreement:

  - Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed.

Issue 1-2-1: whether to enhance Tsearch for RACH based PSCell activation

- Option 1 (OPPO):

  - For RACH based PSCell activation, it is not necessary to improve the delay requirements for RACH-based PSCell activation. Keep the same Tsearch for all cases, including that UE is configured with bfd-and-RLM with value true and without detecting RLF or BFD.

- Option 2 (Nokia):

  - In general, a UE which has not detected neither BFD nor RLF on the deactivated PSCell while deactivated, need no additional search time at PSCell activation (Tsearch = 0ms).

  - A UE which has detected either BFD or RLF on the deactivated PSCell is allowed search time at PSCell activation.

- Recommended WF:

  - TBD in next meeting.

Issue 1-2-3: DRX application and PDCCH monitoring of PSCell immediately after SCG activation

- Option 1 (Nokia):

  - UE shall start monitoring PDCCH on the activated PSCell immediately after the SCG activation delay. It means “The UE shall apply no DRX immediately after Tactivation_time.” (excerpt from R4-2411964)

- Option 2 (vivo):

  - UE starts to monitor PDCCH with/without DRX applying once upper layers indicate that SCG is activated

- Option 3 (vivo):

  - UE won’t monitor PDCCH during SCG activation procedure and starts to monitor PDCCH with/without DRX applying after SCG activation delay (i.e., Tactivation_time specified in TS38.133)

- Proposal (Nokia, vivo): For activation of SCG, RAN4 to send LS to RAN2 clarifying the UE behavior on PDCCH monitoring

- Recommended WF:

  - TBD in next meeting.

# Topic #2: RRM performance requirements maintenance (5.14.2)

Issue 2-1: missing TCs for FG31-1

- Option 1 (Nokia):

  - To specify the test cases for TC#1 in the other agreed CA/EN-DC modes:

    - (1) FR1 PCell+FR2 target SCell and

    - (2) FR2 PCell+FR2 inter-band target SCell..

- Recommended WF

  - Discuss in CR directly.

Issue 2-2: SR transmission in the FG31-1 test case for L3 report

Agreement:

  - For single SCell Activation of unknown SCell with valid L3 measurement results in FR1, RAN4 to introduce another sub-test with SR transmission in the existing test case for L3 report based fast SCell activation test case.

  - Corresponding CR to be submitted in the next meeting.