3GPP TSG-RAN WG4 Meeting #112	R4-2414033

Maastricht, Netherlands, August 19 ‒ 23, 2024

Title: 	WF on SL positioning and carrier phase positioning

Agenda Item:	6.1.3

Source: 	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [112][211] NR_pos_enh2_part2.

Topic #1: Sidelink Positioning Requirements

Sub-topic 1-1 SL Positioning Core Requirements Maintenance (agenda 6.1.1.2)

Issue 1-1-1: End point of SL-PRS based RSTD measurement period requirements

Conclusion:

    - No common understanding on RAN2 procedure for reporting behaviour. Companies check offline.

    - No consensus at the moment to revisit previous agreement

Issue 1-1-2: Impact of Uu link connect

The issue is resolved already by adding Note 3 in the introduction part of SL positioning section in the latest version of specification.

Issue 1-1-3: Measurement period requirements for SL PRS-RSRP(P)

Agreements:

    - Update the measurement period definition of SL PRS-RSRP and SL PRS-RSRPP to refer to SL RSTD, SL Rx-Tx, SL AoA, and SL RTOA, depending on which of them the SL PRS-RSRP and SL PRS-RSRPP measurements are configured.

Sub-topic 1-2 SL Positioning Performance Requirements (agenda 6.1.2.2)

Issue 1-2-1: RF calibration margin for SL RSTD/Rx-Tx

Agreements:

  - Reuse the RF calibration margins defined for Uu positioning for SL RSTD and SL Rx-Tx:

      - The RF calibration margin for SL RSTD measurements in FR1:

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 48 | ≥ 24 | N/A | Z1=72 |
| ≥ 96 | ≥ 48 | ≥ 24 | Z2=36 |

      - The RF calibration margin for SL Rx-Tx measurements in FR1:

| [Min(SL PRS Rx BW, SL PRS Tx BW) (RB)] |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 48 | ≥ 24 | N/A | 1=80 |
| ≥ 96 | ≥ 48 | ≥ 24 | 2=56 |

Issue 1-2-2: Frequency drift margin for SL Rx-Tx measurement

Agreements:

  - The frequency drift margin for SL Rx-Tx is defined as Y=32 Tc, provided that the time offset between the SL PRS transmission and reception, which are used for a single SL Rx-Tx estimate, is no greater than 160 ms.

Issue 1-2-3: General aspects for measurement accuracy requirements for SL positioning

Agreements:

- On SL-PRS bandwidth 24 PRBs with 15 kHz SCS in the accuracy requirements for SL RSTD and SL Rx-Tx measurements:

  - This case was not simulated, the performance can be worse than legacy.

  - Do not include 24 PRBs in Rel-18 accuracy requirements for SL positioning.

- The following band groups are included in the accuracy requirements for SL positioning and in the conditions for NR SL-PRS based measurements:

  - NR_TDD_FR1_B,

  - NR_TDD_FR1_C,

  - NR_FDD_FR1_G,

  - NR_TDD_FR1_J.

Issue 1-2-4: Measurement accuracy for SL PRS-RSRP(P)/Rx-Tx

Agreement:

  - Accuracy requirements:

    - The numbers refer to the simulation summary (R4-2410196) where the accuracies have been derived.

Issue 1-2-5: Delay and accuracy TCs for SL PRS-RSRP(P)

Agreements:

- Whether to verify the accuracy of legacy measurements in SL RSRP(P) TCs

    - verify only the accuracy of SL PRS-RSRP/RSRPP

  - For SL RSRP(P) delay TCs:

    - a new TC with both measurements, but add a clarification that the UE is tested only once for RSTD/Rx-Tx

      - For UE supporting SL RSTD + SL PRS-RSRP, it is sufficient the UE passes the TC with both measurements

      - For UE supporting SL Rx-Tx + SL PRS-RSRPP, it is sufficient the UE passes the TC with both measurements

  - For SL RSRP(P) accuracy TCs:

    - a new TC with both measurements, but add a clarification that TE verifies only the accuracy of SL PRS-RSRP/RSRPP

      - For UE supporting SL RSTD + SL PRS-RSRP, it is sufficient the UE passes the TC with both measurements

      - For UE supporting SL Rx-Tx + SL PRS-RSRPP, it is sufficient the UE passes the TC with both measurements

Issue 1-2-6: SL-PRS signal level configurations

Agreements:

- Check the RAN1 specification and the Uu TCs in relation to overlapping SL-PRS from different UEs. Further discuss the options:

  - Option 1: overlapping SL-PRS can be used in TCs.

  - Option 2: overlapping SL-PRS should not be used in SL TCs.

Issue 1-2-7: SL-PRS bandwidth configurations in TCs

Agreements:

- Update the bandwidth of SL-PRS configurations as follows:

| RB numbers containing SL PRS within channel Bandwidth Note 1 | [2448] | [4896] |
| --- | --- | --- |

- BW for accuracy TCs: 24 and 48 RBs.

- BW for delay TCs: 48 RBs.

Issue 1-2-8: Other test case configurations

Agreements:

- FFS: Shared and dedicated resource pools are randomly used among different TCs.

- FFS: (symbol num, comb size): (4, 4) and (2, 4).

Issue 1-2-9: Spec structure for SL-PRS test configuration

Agreements:

- The following specification clause numbers are updated:

    - A.3.21A	NR Sidelink Measurements for Positioning

    - A.3.21A.1	Introduction

    - A.3.21A.2	NR SL-PRS configurations

    - A.3.21A.2.1	NR SL-PRS configurations for FR1

Topic #2: Carrier Phase Positioning Requirements

Sub-topic 2-1 Carrier Phase Positioning Core Requirements Maintenance (agenda 6.1.1.2)

Issue 2-1-1: Measurement period requirements for DL RSCP/DL RSCPD with aperiodic time window

Agreements:

- Define the requirement for one-shot window.

- The requirement is defined by taking the time window length + processing time.

Issue 2-1-2: Measurement period requirements for DL RSCP/DL RSCPD with periodic time window with multiple PFLs configured

Agreements:

- RAN4 to clarify that the application of the periodic time window is limited to the PFL indicated for CPP measurement.

Issue 2-1-3: The impact of carrier frequency offset

See agreements in Issue 2-2-2.

Sub-topic 2-2 Carrier Phase Positioning Performance Requirements (agenda 6.1.2.6)

Issue 2-2-1: Whether to verify the accuracy of legacy measurements in RSCPD/RSCP TCs

Agreements:

- For the CPP test case, both RSTD/Rx-Tx measurement and CPP reporting are configured, and TE check the success rate for CPP reporting.

- If UE supports CPP measurement, UE shall also pass the test case for RSTD/Rx-Tx measurement.

Issue 2-2-2: Additional margins due to frequency drift and RF calibration

Agreements:

- RAN4 to clarify that the RSCPD accuracy requirement and the relative RSCP accuracy requirement apply given that the carrier phase measurements are performed on PRS resources within a slot/a set of symbols, which can be controlled at least by the window configuration.

- Companies can bring proposals on other scenarios in WI performance maintenance phase.

Reference

- R4-2411806, Topic summary for [112][211] NR_pos_enh2_part2, CATT, RAN4#112.

- R4-2413873, Ad-hoc minutes #1 for NR_pos_enh2 WI, Ericsson, RAN4#112.

- R4-2413874, Ad-hoc minutes #2 for NR_pos_enh2 WI, Ericsson, RAN4#112.

- R4-2413873, Ad-hoc minutes #3 for NR_pos_enh2 WI, Intel, RAN4#112.