3GPP TSG-RAN WG4 Meeting #112	R4-2414035

Maastricht, Netherlands, 19th ‒ 23rd August, 2024

Title: 	WF on RRM requirements for NR_MIMO_Ph5

Agenda Item:	8.18.4

Source: 	Samsung

Document for:	Approval

<Topic 1: RRM Core Requirement>

Issue 1: RRM core impact analysis

Agreement: RRM core impact analysis:

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

<Sub-topic 2-1: Enhancement for UE-initiated/event-driven beam management>

Issue 2-1-2: Scope of the scenario of event-triggered L1 beam reporting

Agreement:

- Intra-cell and inter-cell for FR2.

- Note: If specific requirements cannot be applicable, discuss it case by case.

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period?

Agreement:

- RAN4 will define Event triggered measurement reporting delay requirement

- Further discuss:

  - Whether to support both mode A and mode B

  - Whether to differentiate mode A and mode B

  - Other related aspects

Issue 2-1-5: measurement metrics for beam report?

Agreement:

- Based on L1-RSRP

- Note: others can be revisited if RAN1 have further agreement

Issue 2-1-6: L1-RSRP for CSI-RS measurement?

Enhancement for UE-initiated/event-driven beam management

Agreement: Define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS.

Issue 2-1-7: Whether to specify new unified TCI state switching delay?

<Way forward>

- FFS on whether to define new TCI state switching delay or existing requirements are sufficient

Issue 2-1-8: Whether to filter L1-RSRP?

<Way forward>

- FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

Issue 2-1-9: Impact of beam on known/unknown conditions for TCI states?

<Way forward>

- FFS. Depends on RAN1 further progress.

Issue 2-1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward>

- FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement.

Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward>

- Option 1: No new accuracy requirements for existing L1 measurements are required.

- Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

<Sub-topic 2-2: CSI enhancement to support up to 128 ports >

Issue 2-2-1: Whether RRM impacts exist by CSI enhancement to support up to 128 ports?

Agreement:

- No RRM impact by CSI enhancement to support up to 128 ports

<Sub-topic 2-3: UE reporting enhancement for CJT calibration >

Issue 2-3-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration?

<Way forward>

- Option 1: No RRM core requirements impact.

- Option 2: RAN4 to define the measurement delay or measurement behaviour for the aperiodic standalone CJT reporting.

Issue 2-3-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration?

<Way forward>

- Option 1: No RRM performance requirements impact.

- Option 2: No accuracy requirements. Potential mapping tables are needed if any.

- Option 3: Define accuracy requirements.

- Option 4: Define accuracy requirements and report mapping tables.

<Sub-topic 2-5: Enhancement for asymmetric DL sTRP/UL mTRP scenarios >

Issue 2-5-3: Whether RRM impacts exist by 2TA enhancement?

<Way forward>

- FFS on whether there are RRM impacts by 2TA enhancement.

Issue 2-5-4: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

<Way forward>

- Option 1: The legacy requirements can be reused.

- Option 2: The legacy requirements cannot be reused.

- Option 3: RAN4 to discussion how to update R17/R18 requirement for asymmetric DL sTRP/UL mTRP scenarios

Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update?

Agreement:

Whether to define RRM core requirements of pathloss offset update?

- Not define RRM requirement for MAC CE decoding delay.

- The agreement can be revisited depending on further RAN1 progress.