3GPP TSG-RAN WG4 Meeting #112	R4-2414036

Maastricht, Netherlands, 19th ‒ 23rd August, 2024

Title: 	WF on RRM requirements for NR_MIMO_evo_DL_UL

Agenda Item:	5.27.4

Source: 	Samsung

Document for:	Approval

<Topic 1: RRM core requirements maintenance >

<Sub-topic 1-1: Timing Requirements >

Issue 1-1-1-a: For PDCCH order RACH, define uplink timing and DL timings association in inter-cell case:

Agreement:

- For inter-cell multi-DCI based multi-TRP operation with two TAs, for PRACH transmission triggered by PDCCH order,

  - If “PRACH association indicator” is 0, the reference point is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG

  - If “PRACH association indicator” is 1, the reference point is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

Issue 1-1-1-b: Reference DL timing point for PUCCH/PUCCH/SRS:

Agreement:

- For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAG as the uplink signal.

Issue 1-1-1-c: Reference DL timing point for PDCCH order RACH for CFRA in intra-cell case:

Agreement:

- For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

<Sub-topic 1-2: Other Requirements >

Issue 1-2-1: For mDCI mTRP, whether to need additional DL RS tracking time for UL TCI state switching if UE supporting two TAs (RTD<CP and RTD>CP)?

Agreement:

- In Rel-18, no consensus on the requirement for separate UL state TCI switching if there is no active DL TCI-State associated with the same coresetPoolIndex. For the other cases, reuse Rel-17 requirements.

<Topic 2: RRM performance requirements maintenance >

Issue 2-1-1: For high doppler condition (300Hz) + 30kHz SCS TDD, SNR = 10dB, TDCP test requirements in the test?

Agreement:

- TC4 for high doppler condition+30kHz SCS TDD, 300Hz, SNR=10dB

  - Reported TDCP index is larger than 6, with at least 80% probability