3GPP TSG-RAN WG4 Meeting #112				                                              R4-2414041

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	WF on NR_RRM_Ph5_Part2 - fast SCell activation

Agenda item:			8.15.4

Source:	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [112][219] NR_RRM_Ph5_Part2.

- Topic #1: Fast SCell activation for UE supporting Rel-18 EMR

  - Sub-topic 1-1 General

Issue 1-1-1: Clarification on Rel-18 eEMR

Agreements:

- Do not change the Rel-18 eEMR definition in this fast SCell activation discussion.

Issue 1-1-2: Applicability of fast SCell activation delay requirements

Agreement:

- The fast SCell activation delay requirements are defined for the case when

  - the UE supports Rel-18 eEMR:

    - Including both EMR and cell reselection measurement,

    - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured

    - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured

  - and the UE has reported valid results on the SCell to be activated before SCell activation command.

Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR

Agreement:

- Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR:

  - Direct SCell Activation at SCell addition

  - FFS: Single MAC CE based DL SCell Activation

  - FFS: Single MAC CE based PUCCH SCell Activation

- RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

Issue 1-1-4: Target scenarios of fast SCell activation

Proposals

- Proposal 1:

  - RAN4 shall specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay).

  - Fast SCell activation for UE supporting Rel-18 EMR should aim to achieve less than 100ms activation delay. RAN4 to discuss the options how this is achieved.

- Proposal 2:

  - Suggest to focus on the following scenario for fast SCell activation for UE supporting Rel-18 EMR: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1.

- Proposal 3:

  - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1.

  - Sub-topic 1-2 SCell activation delay requirements

Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting

Agreement:

- RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command.

  - FFS how to update the known condition (issue 1-2-1a).

  - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting

FFS on proposals listed in [1] for issue 1-2-1a.

Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation

Proposals

- Option 1:

  - RAN4 to discuss whether it’s necessary to define a consistent SINR condition during the whole procedure for known SCell activation requirement (including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode), e.g., SINR>=-2dB

Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation

Proposals

- Option 1:

  - RAN4 to discuss whether and how to change the measurement period condition of FR1 known SCell activation (i.e., 2400ms in current requirement) for fast SCell activation with EMR.

Issue 1-2-4: Whether the indication to network is needed

Proposals

- Option 1:

  - The Rel-19 RRM enhancement in fast scell activation for UE support Rel-18 EMR shall not only be RAN4 side condition update, certain activation fast or slow indication to the network is needed.

Reference

- R4-2411814 Topic summary for [112][219] NR_RRM_Ph5_Part2, CATT, RAN4#112