3GPP TSG-RAN WG4 Meeting #112	R4-2414043

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Title: 	WF on RRM requirements for XR_Ph3

Agenda Item:	8.24.3

Source: 	Nokia

Document for:	Approval

# Topic #1: Scenarios for XR enhancements

## Issue 1-1: Workscope

< Agreement>:

- As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement.

  - The progress in RAN1/2 should be taken into account.

- FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation.

- RAN4 to discuss

  - The potential impact on measurement requirements due to measurement cancellation

  - The corresponding solution to address the impact if needed.

## Issue 1-2: Deployment scenarios

<Way forward>:

- Discuss if there are aspects related to deployment scenarios to be considered.

## Issue 1-3: Types of measurement gaps to consider

In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

<Way forward>: Please discuss the priorities for RAN4 to work based on the following table:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | Nokia, MTK, QC, Vivo, ZTE, Apple, CMCC, HW, Ericsson, OPPO, CATT, IDC |  |  |
| Type 2 | QC, vivo, Apple, CMCC, HW, OPPO, CATT | Nokia | ZTE |
| NCSG | Apple, CMCC, HW, OPPO, MTK | QC, Nokia, CATT, IDC | vivo |
| MUSIM |  | HW | Vivo, Apple, Nokia, Ericsson, CATT |
| Positioning |  | QC, vivo, HW | ZTE, Apple, Nokia, Ericsson, CATT, IDC |
| Rel 18 concurrent gap | QC, Apple, CMCC, HW, CATT, MTK |  | Vivo, ZTE, Nokia, Ericsson |
| PreConfigured | Apple, QC, CATT, CMCC, HW, OPPO, MTK |  |  |

## Issue 1-4: Types of measurements

<Way forward>:

- Discuss types of measurements that may potentially be impacted by the XR operation.

  - FFS: measurements without gaps

# Topic #2: Need/feasibility of UE assistance information

## Issue 2-1: General on UAI

<Way forward>:

  - Discuss if UAI is needed and which aspects to consider.