3GPP TSG-RAN WG4 Meeting #112		R4-2414100

Maastricht, Netherlands, 19th – 23rd Augus

Agenda item:			5.23.9

Source:	Moderator (Qualcomm Incorporated)

Title:	WF on RRM requirements for NR_NTN_enh

Document for:	Approval

# Introduction

The summary covers the contributions submitted under the following AI:

- 5.23.6	RRM core requirements	[NR_NTN_enh-Core]

- 5.23.7	RRM performance requirements	[NR_NTN_enh-Perf]

# Topic #3: Network verified UE location

### Issue 3-5: Other impact on RRM

Agreement (ad-hoc):

- RAN4 to clarify that UE is only required to measure PRS from the serving cell for PRS measurement for NW verified location.

# Topic #4: Idle/Inactive mode mobility enhancements

### Issue 4-1: TN to NTN cell reselection

Agreement (online):

- For NTN incapable UE, if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode, the existing TN-to-TN cell reselection requirements apply.

FFS:

- For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied

### Issue 4-2: NTN to TN cell reselection

FFS:

- Clarify that the requirements for NR NTN to LTE TN inter-RAT cell re-selection is defined as NEUTRA_carrier * Tdetect/measure/evaluate,EUTRAN, where

  - The parameter NEUTRA_carrier is the number of configured E-UTRA carriers indicated in the neighbour frequency list by serving cell, except for the frequency carrier where there is no coverage of that frequency based on the provide TN cell coverage information and UE GNSS position information.

  - Tdetect/measure/evaluate,EUTRAN is the inter-RAT E-UTRAN TN cell re-selection requirement defined in Table 4.2.2.5-1 in TS 38.133

# Topic #5: Connected mode mobility enhancements

### Issue 5-2: NTN to NTN Satellite switching without PCI change

Agreement (ad-hoc):

- RAN4 to clarify that the ending point of satellite switch with re-sync is the time point when UE is ready to receive DL channels/signals and transmit UL channels/signals from/to the target satellite, and to remove TIU in the delay/interruption time.

### Issue 5-2-S: Soft’ Satellite switch

FFS:

- RAN4 to not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change.

FFS:

- To enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

# Topic #6: Performance requirements

### Issue 6-2-2: (FR2-NTN) Rx beam gain

Agreement (online):

- For the minimum SSB_RP condition for electronic steering antenna,

  - RAN4 to confirm Y (gain difference between fine and rough beams) = 0

- Remove the bracket for the following 1dB relaxation:

  - The existing absolute measurement accuracy requirement and relative measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with 1dB relaxation

Agreement (ad-hoc):

- For RLM for FR2-NTN, RAN4 to discuss the following options

  - Update core requirements (PDCCH parameters, evaluation period) as for R17 RedCap

Agreement (ad-hoc):

- Value of Gmin:

  - Further discuss the detailed parameter assumptions to be aligned with core specification TS 38.101-5 and TR 38.863.

  - Separate values will be introduced for NTN VSAT Type 3 and other VSAT types.

Agreement (ad-hoc):

- Gmax leave to be declaration basis.

  - Note: FFS whether RAN4 needs to specify the possible range for Gmax

### Issue 6-2-3: (FR2-NTN) UL timing accuracy

Agreement (online):

- For FR2-NTN UL timing test, RAN4 to define TGNSS_margin as 65.536 Tc and 196.608 Tc for Fixed VSAT and Mobile VSAT, respectively.

- For FR2-NTN UL timing test, the test requirement for Case 3 is same as that for Case 1.

  - With the same test configuration (including the AWGN channel condition) for case 1 and 3.

# References

[1] R4-2411801, “Topic summary for [112][206] NR_NTN_enh,” 3GPP TSG-RAN WG4 Meeting #112

[2] R4-2414028, “Ad-hoc mintues for [112][206] NR_NTN_enh,” 3GPP TSG-RAN WG4 Meeting #112