3GPP TSG-WG4 Meeting #112	 R4-2412606

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	WF on simultaneous Rx-Tx for CA_n40-n41

Agenda Item:	7.1

Source: 	Huawei

Document for:	Approval

Background:

In Rel-19 new WI: Simultaneous RxTx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19

Objective #3:

Identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values. If needed, specify the MSD requirements for the identified band combinations.

- Note 1: Band combinations considered in this WI have to be introduced first via basket WIs (see 2.3) or completed in previous releases if necessary.

- Note 2: Whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case.

- Note 3: Applicability of simultaneous Rx/Tx capability to FDD-TDD band pairs should be reviewed

Table 4.1.3-1: Identified NR band combinations for evaluation to support simultaneous Rx/Tx

| Band combination | Power Class | Note | status |
| --- | --- | --- | --- |
| CA_n40A-n41A | PC3, PC2 | This is a leftover band combination in Rel-18, the following aspects should be further studied: - Study, and specify if needed, diversity receiver relaxation for band combination with narrow frequency separation. - Evaluate decoupling of simultaneous Rx-Tx and UL MIMO/TxD or SRS antenna switching for the bands in the band combination with narrow frequency separation |  |

In RAN4#112, the following proposal are provided by companies.

    - Proposal 1 (Apple):

Alt-1a) Consider relaxing four Rx requirement for n41 when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured.

Alt-1b) A new signalling introduced to indicate that a UE supporting MIMO and simultaneous Rx/Tx cannot support both features at the same time for a specific combination.

    - Proposal 2 (Murata):

Relax 4 Rx requirement to 2 Rx when UE is near cell edge and keep 4 Rx requirement when UE requires high data T-put using 4x4 DL MIMO. FFS how to switch between 4Rx and 2Rx.

Figure 1. RF front end architecture for CA_n40-n41 using 4 antennas in n41 with simultaneous Rx-Tx operation

Figure 2. RF front end architecture for CA_n40-n41 using 2 antennas in n41 with simultaneous Rx-Tx operation when UE is near cell edge.

<Way forward>

The way to handle the simultaneous Rx-Tx for CA_n40-n41 in Rel-19.

- Option 1: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41

- Option 2: Analyse the UE architecture with n41 using 2Rx in simultaneous Rx-Tx for CA_n40-n41, and try to define the separate requirement including MSD optionally from that in Rel-18, taking the implementation difficulties into consideration.

  - However, the UE should support 4Rx for n41 when the UE does not indicate simultaneous Rx-Tx for CA_n40-n41, i.e single n41 or other CA combo.

- Option 3: Study whether new UE signalling(s) could be introduced for a certain band combination to indicate that:

  - concurrent support of 4x4 MIMO and simultaneous Rx/Tx is not supported

  - concurrent support of UL MIMO, and/or TxD and/or SRS antenna switching and simultaneous Rx/Tx is not supported

- Other options are not precluded