# 3GPP TSG-RAN WG4 Meeting # 112      													R4-2414272

# Maastricht, Netherlands, 19th-23th, Aug, 2024

Title: 	WF on introduction of NR bands n87 and n88

Agenda item:	7.15.2

Source:	Nokia

Document for:	Approval

# 1	Background

This is a WF on bands n87 and n88.

# 2 	Discussion

2.0	Regulatory aspectsRF requirements are based on ECC/Dec/(16)02 and ECC/Dec/(19)02 regulatory decisions, together with underlying ECC Report 283 on compatibility studies.

The following tasks are to be completed in this WID:

- Verify regulatory requirements for NB-IoT operation in 410-430 MHz frequency range

- Analyse annex 4 in ECC/Dec/(19)02 for the identification of any applicable requirements for in-band/guardband/standalone NB-IoT operation in bands n87 and n88

## 2.1	System parameters

System parameters in  R4-2411945 are used.

No need for enhanced channel raster points.

## 2.2	UE RF requirements

Agreed changes are listed in Table 2.2-1 for both bands separately. As a reference LTE band 31 and 72 requirements are listed.

Table 2.2-1: Changes for UE specification 38.101-1 due to introduction of bands n87 and n88

| Requirement | B87 in 36.101 | n87 proposal | B88 in 36.101 | n88 proposal |
| --- | --- | --- | --- | --- |
| Operating bands | 410 MHz – 415 MHz 420 MH z– 425 MHz | Re-use LTE | 412 MHz – 417 MHz 422 MH z– 427 MHz | Re-use LTE |
| Channel bandwidths | 1.4, 3 and 5 MHz | 3 and 5 MHz | 1.4, 3 and 5 MHz | 3 and 5 MHz |
| EARFCN / NR-ARFCN | LTE band specific | Needs n87 specific | LTE band specific | Needs n88 specific |
| Sync-raster | Not in LTE | Needs n87 specific | Not in LTE | Needs n88 specific |
| TX–RX frequency separation | 10 MHz | 10 MHz | 10 MHz | 10 MHz |
| UE maximum output power | PC1 and PC3 | PC1 and PC3 | PC1 and PC3 | PC1 and PC3 |
| MPR | Not band but LTE specific | PC3 MPR is not band specific. PC1 reuse MPR for PC1 for bands other than Band n14 | Not band but LTE specific | PC3 MPR is not band specific. PC1 reuse MPR for PC1 for bands other than Band n14 |
| A-MPR | No A-MPR | Not needed | No A-MPR | Not needed |
| Output power dynamics | Not band specific | Not band specific | Not band specific | Not band specific |
| Transmit signal quality | Not band specific | Not band specific | Not band specific | Not band specific |
| Occupied bandwidth | Not band specific | Not band specific | Not band specific | Not band specific |
| Spectrum emission mask | Not band specific | Not band specific | Not band specific | Not band specific |
| ACLR | Not band specific | NR ACLR is not a band specific. UTRAACLR is not applicable. | Not band specific | NR ACLR is not a band specific. UTRAACLR is not applicable. |
| General spurious emissions | Not band specific | Not band specific | Not band specific | Not band specific |
| Spurious emissions for UE co-existence | Is band specific | LTE requirement is a baseline | Is band specific | LTE requirement is a baseline |
| Transmit intermodulation | Not band specific | Not band specific | Not band specific | Not band specific |
| REFSENS power level | -95.7 for 3 MHz -93.5 for 5 MHz | Re-use LTE REFSENS as NRB is same for LTE and NR | -95.7 for 3 MHz -93.5 for 5 MHz | Re-use LTE REFSENS as NRB is same for LTE and NR |
| REFSENS UL allocation | 5 RB’s with Note 4 restriction | Re-use LTE specification with note 4 content | As an error not specified, maintenance CRs are provided into this meeting 5 RB’s with Note 4 restriction | Re-use LTE specification with note 4 content |
| Max input level | Not band specific | Not band specific | Not band specific | Not band specific |
| ACS | Not band specific | Not band specific | Not band specific | Not band specific |
| IBB | Is band specific | Add n87 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table | Is band specific | Add n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table |
| OBB | Is band specific | Add n87 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table | Is band specific | Add n88 into FDL_high < 2700 MHz and FUL_high < 2700 MHz Table |
| NBB | Not band specific | Add n87 into NBB table | Not band specific | Add n88 into NBB table |
| Spurious response | Not band specific | Is frequency specific but no change is needed | Not band specific | Is frequency specific but no change is needed |
| Wideband intermodulation | Not band specific | Is frequency specific but no change is needed | Not band specific | Is frequency specific but no change is needed |
| Spurious emissions | Not band specific | Not band specific | Not band specific | Not band specific |

With the above table being the baseline, reassure that the UE RF requirements are aligned with the regulation captured in:

- Table 6 and table 7 of Annex 3 in ECC/Dec/(16)02, and

- Table 5 and table 6 of Annex 2 in ECC/Dec/(19)02.

## 2.2	BS requirements

Reassure that the BS RF requirements are aligned with the regulation captured in:

- Annex 3 in ECC/Dec/(16)02, and.

- Annex A2.2 and A2.3 in ECC/Dec/(19)02.

## 2.3	AOB

### 2.3.1	TR

Findings in TR 36.762 are considered as a baseline. Refrain from creating a new NR-specific TR.

Any NR-specific aspects can be captured in TR 36.762 (e.g. as informative note).

### 2.3.2	Work split

| Specification | Responsible company for draft and formal CRs |
| --- | --- |
| 38.101-1 | CATT |
| 38.133 | Ericsson |
| 38.106 | ZTE |
| 38.115-1 | ZTE |
| 38.174 | ZTE |
| 38.176-1 | ZTE |
| 38.176-2 | ZTE |
| 36.104 | Ericsson |
| 36.141 | Nokia |
| 37.104 | Nokia |
| 37.141 | Nokia |
| 38.104 | Nokia |
| 38.141-1 | Ericsson |
| 38.141-2 | Ericsson |
| 38.307 | Huawei |
| 37.105 | Huawei |
| 37.145-1 | Huawei |
| 37.145-2 | Huawei |
| 38.101-5 | Huawei |

# 3	Conclusion

This contribution captures the agreements for introduction of bands n87 and n88 made in RAN4#112.

# 4	References

- [1] RP-241663 New WID on introduction of NR bands n87 and n88, RAN4#104, Nokia