3GPP TSG-RAN WG4 Meeting # 112												                          R4-2414274

Maastricht, Netherlands, August 19 – August 23, 2024

Agenda item:			 7

Source:	CMCC, CBN, China Broadnet

Title:	WF on introduction of PC2 and 40MHz CBW in NR band n28

Document for:	Approval

Introduction

This way forward captures the agreements for  [112][115] NR_n28_PC2_40MHz. The summary in RAN4#112 is R4-2412817.

UE RF requirements

Sub-topic 2-1 General issue

Issue 2-1-1: UE architecture assumption

Agreement:

- No need to study the feasibility of UE architecture, companies are encouraged to provide analysis on RF requirements based on their own implementation.

- RAN4 RF requirements should accommodate different UE architecture assumption.

- FFS on whether single set of requirements for different UE architectures should be defined.

Issue 2-1-2: A-MPR simulation assumption

Agreement:

- Reuse Rel-18 A-MPR simulation assumption in this WI.

Sub-topic 2-2 PC2 requirements for BW<=30MHzs

Issue 2-2-1 PC2 RSD for 1Tx and 2Tx for BW<=30MHz

Agreement:

- Define RSD requirements as following:

Table 1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for single Tx

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 0.6 | 0.6 | 0.7 | 0.8 | 1.3 | 2.4 | 2.9 |  |  |  |  |

Table 2 Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for dual Tx

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 1.1 | 1.1 | 1.1 | 1.3 | 3.0 | 6.6 | 7.9 |  |  |  |  |

Issue 2-2-2  NS_17 A-MPR for PC2 for BW<=30MHz

Agreement:

- Use following values in Rel-18 WF as starting point:

- Table 1: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | > 5.4 | A2 |

- Table 2: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ [3] | ≤ [4] |
|  | QPSK | ≤ [3] | ≤ [4] |
|  | 16 QAM | ≤ [3.5] | ≤ [4] |
|  | 64 QAM | ≤ [4] | ≤ [4.5] |
|  | 256 QAM |  | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [5] | ≤ [5.5] |
|  | 16 QAM | ≤ [5] | ≤ [5.5] |
|  | 64 QAM | ≤ [5] | ≤ [5.5] |
|  | 256 QAM |  |  |

Issue 2-2-3 NS_18 A-MPR for PC2 for BW<=30MHz

Agreement:

- Reuse Rel-18 requirement for NS_18. (refer to R4-2310245)

Sub-topic 2-3 40MHz requirements for PC3 and PC2

Issue 2-3-1 Uplink configuration for n28 REFSENS

Agreement:

- Adopt the following UL configuration for 40MHz

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Issue 2-3-2 n28 REFSENS for PC3

Agreement:

- FFS on PC3 REFSENS based on more companies’ input.

- Following values are proposed in this meeting as starting point.

| Source | 40 MHz (dBm) |
| --- | --- |
| Skyworks(R4-2413062) | -66.3 |
| Qualcomm (R4-2413149) | -65.9 |
| Muruta (R4-2411476) | -67.1 for 15KHz -67.2 for 30KHz |
| Average | -66.4 |

Issue 2-3-3 PC2 RSD for 1Tx and 2Tx for 40MHz

Agreement:

- FFS on PC2 RSD for 40MHz

Issue 2-3-4 ∆MPR for 40MHz

Agreement:

- Further check if ΔMPR= 0.5dB is sufficient

Issue 2-3-5 NS_17 for 40MHz

Agreement:

- Do not specify NS_17 for 40 MHz CBW

Issue 2-3-6 NS_18 for 40MHz

Agreement:

- Specify NS_18 for 40MHz. FFS on the requirements.

Issue 2-3-7 General coex requirements for 40MHz

Agreement:

- Further check whether all currently specified UE-to-UE coex requirements are applicable for 40 MHz CBW

Issue 2-3-8 channel location

Agreement:

- FFS on how to capture the UL channel location for UE supporting maximum bandwidth of 40MHz in n28.

Issue 2-3-9 channel raster

Agreement:

- The following options can be considered for n28 40MHz

  - Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

  - UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

Issue 2-3-10 Release independence

Agreement:

- FFS on the release independence of 40MHz for n28

  - Option 1: Rel-15

  - Option 2: Rel-16