3GPP TSG-RAN4 Meeting # 112	R4-2414276

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Simulation assumption for co-existence study

Source:	Samsung

Agenda item:	8.8.5

Document for:	Information

1	Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. In [2], the objective includes the co-existence study for NR and IoT NTN high power UE, and the related content is captured below:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

In this paper, we summarized the baseline assumptions from TR38.863 in Rel-17 and Rel-18 discussions according to WID. This document is subject to update in order to capture the latest agreed assumptions in each meeting as a running document for information purpose.

In this version, the content are developed based on the following:

- TR 38.863 v18.2.0 [1]

- WF R4-2217473 [2]

- WF R4-2414275 [3]

2	NR-NTN and IoT-NTN HPUE co-existence assumptions

2.1	Co-existence simulation scenario

Scenarios for coexistence study are listed in Table 2.1-1.

Table 2.1-1: Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  | NTN1,4,5 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  |
|  |  | GEO3 | LEO 600km | LEO 1200km |
| NR | Rural | X | X | X |
|  | Urban macro | X | X | X |
|  | Dense Urban6 | N/A | N/A | N/A |
| NOTE 1: Earth Fixed beam has been considered for co-existence studies. NOTE 2: Use Set 1 satellite antenna has been used as the starting point for co-existence studies. NOTE 3: GEO and LEO only operate at adjacent channel. NOTE 4: Use GEO and LEO@600km when TN is victim. NOTE 5: The satellite-to-satellite coexistence scenarios are not in the scope of this study considering this is already addressed by ITU (ITU RR Article 9 etc.) and regional regulations (e.g. FCC rules). NOTE 6: Dense Urban is not considered as it is expected NTN UE will connect to terrestrial networks rather than satellite networks in such scenario. |  |  |  |  |

[Editor’s Note: Rel-18 IoT NTN study does not have Dense Urban scenario for NR.]

Editor note: Agreement of Issue 2-2-1 in [3]:

- Prioritize GEO and LEO1200 for co-existence evaluation

  - LEO600 is not precluded for the requirements and the conclusion for LEO1200 can be applied to LEO600

The aggressor and victim combinations are listed in Table 2.1-2.

Table 2.1-2: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. |

Editor note: Agreement of Issue 2-2-3 in [3]:

- Consider both Scenario 4 and 5 for co-existence study

  - Prioritize scenario 4 since it is the worst case

- Do not consider Scenario 1, 2, 3 and 6

For further visualisation, the interference charts with respect to scenarios 1 to 6 are further represented with respect to NTN-TN coexistence scenarios in Figure 2.1-1 (scenarios 1 to 4) and Figure 2.1-2 (scenarios 5 and 6).

Figure 2.1-1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

Figure 2.1-2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

The frequency and bandwidth are listed in Table 2.1-3.

Table 2.1-3: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth |  | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- | --- |
|  |  | NR-NTN | IoT-NTN |  |  |
| TN Rural | 2 GHz | 20MHz |  | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz |  | FDD, TDD | 1 |
| GEO | 2 GHz | 20 MHz for FR1 | 0.18MHz | FDD | 1, 31 |
| LEO | 2 GHz | 20 MHz for FR1 | 0.18MHz | FDD | 1, 31 |
| NOTE: Only FRF=1 has been used in co-existence studies for simplification. |  |  |  |  |  |

Editor’s note: Agreement of Issue 2-3-6 in [3]

- Consider 20MHz system bandwidth for NR-NTN and NR in co-existence study.

2.2	Co-existence simulation assumption

2.2.1	Network layout model

2.2.1.1	Co-existence between NTN satellite and TN

Cellular cell structure is considered for both NTN satellite and TN network layout.

Referring to TR 38.811[5] Section 2.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN satellite beams direction and location on the earth surface. It means the NTN satellite beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Deployment of NTN satellite and TN cells and UEs for co-existence study is listed in Table 2.2.1.1-1.

NOTE:	The NTN UE(s) shall be dropped at the edge of the “central 19 TN cells (cluster)”. For Case 1 (Urban scenario), an Isolation distance of 1500m as 2*ISD is considered to reflect the NTN-TN selection algorithm at the border, assuming TN connection will always be prioritized over NTN.

As defined in Figure 2.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity.

Figure 2.2.1.1-1: Isolation distance

Editor note: Agreement of Issue 2-3-1 in [3]:

- Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex.

  - Use the isolation distance 1.5km as the starting point

  - Other values for isolation distance are not precluded

Table 2.2.1.1-1:	Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | TN with NTN satellite | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the TN sectors which have NTN UE(s) at their edges. Note 1 |
| 5 | TN with NTN satellite | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which has the NTN UE(s) at its edge |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |
| Note 1: Agreement of Issue-2-3-4 in [3]: Option 1 as baseline assumption, other options not precluded |  |  |  |  |  |  |

2.2.2	System parameters

2.2.2.1	Satellite parameters

Two sets of satellite parameters are listed in Table 2.2.2.1-2 and Table 2.2.2.1-3 according to TR 38.821[6].

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.2.2.1-1: NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | For IoT-NTN | For NR-NTN |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NRB (Subcarriers) | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| For IoT-NTN : SCS 3.75kHz | 1 (12) | - | - | - | - |
| For NR-NTN and IoT-NTN : SCS 15 kHz | 1 (48) | 25 | 52 | 79 | 106 |
| For NR-NTN : SCS 30 kHz | - | 11 | 24 | 38 | 51 |

Table 2.2.2.1-2a: Set-1 satellite parameters for co-existence study

(Note: For NR-NTN)

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.81 | 46.53 | 49.71 | 51.53 | 52.81 | 40.53 | 43.71 | 45.53 | 46.81 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.64 | 45.98 | 49.37 | 51.36 | 52.64 | 39.98 | 43.37 | 45.36 | 46.64 |
| Satellite Tx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |
| Channel bandwidth |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg |  |  |  | 4.4127 deg |  |  |  | 4.4127 deg |  |  |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg |  |  |  | 3.8206 deg |  |  |  | 3.8206 deg |  |  |  |
| Satellite beam diameter |  |  | 250 km |  |  |  | 90 km |  |  |  | 50 km |  |  |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  |
| Satellite Rx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |

Table 2.2.2.1-2b Set-1 satellite parameters for co-existence study

(Note: For IoT-NTN)

| Satellite orbit |  |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 1200km | 600 km |
|  |  |  | Payload characteristics for DL transmissions |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite max TX power in dBm | BW (MHz) |  | 0.18 (NB-IoT) | 0.18 (NB-IoT) | 0.18 (NB-IoT) |
|  | SCS 15kHz |  | 30.55 | 32.55 | 26.55 |
|  | SCS 3.75kHz |  | 30.55 | 32.55 | 26.55 |
| Satellite Tx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg | 3.8206 deg | 3.8206 deg |
| Satellite beam diameter |  |  | 250 km | 90 km | 50 km |
|  |  |  | Payload characteristics for UL transmissions |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |

Table 2.2.2.1-3: Other parameters for NTN satellite

| Parameters | NR-NTN | IoT NTN |
| --- | --- | --- |
| Carrier frequency | 2GHz | 2GHz |
| The number of active UE (UL) | 9 UEs and 2RBs per UE for GEO and LEO1 NOTE 2 | 9 for 15kHz SCS 18 and 36 for 3.75kHz SCS (1 subcarrier per UE) |
| The number of active UE (DL) | 1 | 1 |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | See Session 2.2.6.2 NOTE 2 | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |
| UE Tx power in dBm (PC3) | -40 to 23 | -40 to 23 |
| UE Tx power in dBm (PC2) | -40 to 26 | -40 to 26 |
| UE Tx power in dBm (PC1.5) | -40 to 29 | - |
| UE Tx power in dBm (PC1) | -40 to 31 | -40 to 31 |
| NTN satellite Noise Figure in dB | See Table 2.3-3-1 | See Table 2.3-3-1 |
| Handover margin | 3dB | 3dB |
| NOTE 1: UEs are equally splitted inside the channel bandwidth into ACIR 3 regions. Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. NOTE 2: Agreement of Issue 2-4-4 in [3]: For NR-NTN HPUE: - Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only) - Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption NOTE 3: Agreement of Issue 2-3-5 in [3]: - Assuming all NR-NTN/IoT-NTN UEs are HPUE, i.e. 100% HPUE ratio as baseline. - FFS other ratio values |  |  |

Table 2.2.2.1-5: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

2.2.2.2	NTN UE parameters

NTN UE parameters are given in Table 2.2.2.2-1

Table 2.2.2.2-1: NTN UE characteristics for system level simulations

| Characteristics | For NR-NTN and IoT-NTN |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx antenna gain | 0 dBi per element |

Editor note: Agreement of Issue 2-4-3 in [3]

- Baseline: Use NF as 9dB, and Tx power 26/29/31 dBm for PC2/1.5/1 NTN HPUE

- FFS option of 7dB NF for PC2 and LEO600.

2.2.2.3	TN parameters

TN parameters for co-existence study are given in Table 2.2.2.4-1, 2.2.2.4-2 and 2.2.2.4-3.

Table 2.2.2.3-1: Simulation assumptions of TN respectively based on NR

|  | NR |
| --- | --- |
| Carrier frequency in GHz | 2 |
| Size of each nominal channel BW in MHz | 20 |
| Transmission bandwidth in MHz | N/A |
| Environment | Deployment scenario related, see Table 2.2.2.4-2. |
| Network layout | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | Deployment scenario related, see Table 2.2.2.4-2 |
| System loading and activity | See Table 2.2.1.1-1 |
| Network location | See Table 2.2.1.1-1 |
| DL subcarrier spacing | 15kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | TR 36.942[8] |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | 1 |
| Number of scheduled UE per cell (UL) | 3 |
| UE antenna height in meter | 1.5m |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| Building penetration loss | In pathloss model, TR 38.901[10] |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 5 |
| UE noise figure in dB | 9 |
| BS-UE path-loss model | TR 38.901[10] |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901[10] |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 |
| Link-level performance model | See Section 2.2.8 Throughtput-SINR mapping |
| UE distribution | Uniform |
| Evaluation metrics | See Section 2.2.8 Throughtput or SNR loss criteria |

Table 2.2.2.3-2: Deployment-related parameters of TN (2 GHz)

|  | Urban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- |
| ISD in meters | 750 | 7500 | ITU-R Report M.2292[11] |
| BS Antenna height in meters | 25 | 30 |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |
| UE height in meter | 1.5 | 1.5 | RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13] |

Editor’s note: Agreement of Issue 2-2-2 in [3]:

- Prioritize Urban Macro for Scenario 4 and Rural Macro for Scenario 5

Table 2.2.2.3-3: ACLR/ACS for TN (2GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | ACS | 33 |

2.2.3	Antenna and beamforming pattern modelling

2.2.3.1	Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811[5].

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθkasinθ2        for 0<θ≤90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument ‘x’ , where x is kasinθ;

-	a, a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The antenna patterns for LEO 600km, 1200km and GEO are shown in Figure 2.2.3.1-1 and 2.2.3.1-2.

Figure 2.2.3.1-1:  Antenna pattern for LEO 600KM and 1200KM (4.4127 deg for 3dB beamwidth)

Figure 2.2.3.1-2: Antenna pattern for antenna aperture of GEO (0.4011 deg for 3dB beamwidth)

The beam layout definition for a single satellite simulation in S-Band is defined in Table 2.2.3.1-1.

Table 2.2.3.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 45° for GEO and LEO |
|  | Option 1: FRF=1 |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE orientation | Omnidirectional |
| UE attachment | RSRP |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |

2.2.3.2	TN BS and UE antenna and beam forming pattern modelling

For AAS antenna, it refers to the pattern in TR 38.921 [14].

Table 2.2.3.2-1: AAS antenna parameters for 2GHz

|  |  | Rural | Macro urban |
| --- | --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |  |
| 1.1 | Antenna pattern | TR 38.921[14] |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 | 25 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-120 |
| 1.12 | Mechanical downtilt (degrees) | 3 | 10 |

For non-AAS antenna, the parameters in Table 2.2.3.2-1 are used for 2GHz BS antenna pattern in the NTN system simulation.

Table 2.2.3.2-1: FR1 BS Non-AAS antenna pattern for 2GHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |  |
| Combining method for 3D antenna pattern (dB) | A“θ",φ”=-min-AE,Vθ"+AE,Hφ”, Am |  |
| Maximum directional gain of an antenna GE,max | 17 dBi |  |
| Conducted power | 46 dBm |  |
| Mechanical downtilt in degrees | Rural | 3 |
|  | Urban | 10 |

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.

2.2.4	ACIR model

For NR-NTN:

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 2.2.2.1-4 and Table 2.2.2.4-1 respectively.

For IoT-NTN:

For Case 1: Flat model with adjusting the TN ACLR according to TR 36.942, i.e. adjusting the TN ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS. FACLR = ~20.5 dB with 1.5km isolation distance.

Case 4: Symmetric bandwidth NTN ACLR model according to TR 36.942 section 5.1.1.4.2 with Flat NTN ACLR model and FACLR = 0 dB, assuming flat ACS.

2.2.5	Propagation model

2.2.5.1	Propagation model between NTN SAN and UE

Propagation model between NTN SAN and UE should reference to section 6.6 in TR 38.811[5].

2.2.5.2	Propagation model between TN BS and UE

Propagation model between TN BS and UE should reference to section 7.4 in TR 38.901[10].

2.2.5.3	Propagation model between NTN SAN and TN BS

Propagation model between NTN SAN and TN BS should reference to TS 38.811[5] which is used for DL-UL cross link interference for S band.

2.2.6	Transmission power control model

2.2.6.1	TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = -63 dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

2.2.6.2	NTN UL TPC

For NTN HPUE UL TPC:

where:

-	Pmax = 26dBm for PC2, 29 dBm for PC1.5, 31 dBm for PC1.

-	Rmin = -66 dB for PC2, -69 dB for PC1.5, -71 dB, assuming -40dBm minimum output power,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ),

  - Where:

  - SNRtarget and BW

For NR-NTN HPUE:

    - Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only)

    - Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption

  - NF is the SAN noise figure, i.e. 4.3dB.

  - 10log10(kT) = -174dBm/Hz.

  - γ = 1 For uplink scenario.

Editor note: Agreement of Issue 2-4-4 in [3]

2.2.6.3	NTN DL TPC

For downlink scenario, no power control scheme is applied.

2.2.7	Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

2.2.8	Performance metric

For NR-TN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For NR-NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For IoT-NTN, the SINR loss should refer to TR 36.802 (i.e. no greater than 1dB) – same as 36.802.

2.2.9	Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

2.3	Co-existence simulation methodology

2.3.1	Simulation procedure

Adopt following simulation steps.

-	Step 1: Generate aggressor and victim networks.

-	NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default.

-	Deployment of TN network (19 cells with wraparound) refers to Table 2.2.1.1-1

-	Step2: UE associations

-	TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

-	Deployment of NTN UE refers to Table 2.2.1.1-1.

-	Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS/SAN-UE. This is done for both victim and aggressor networks.

-	Step 4: Throughput is computed in the victim systems without considering ACI as below:

ThputNO ACIbpshz=fSINRICI=fSN+IICI,

where:			IICI is the inter-cell interference.

For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821[6] section 6.1.3 equations. Thus those equations should be used for SINR calculation.

-	Step 5: Throughput is computed considering ACI as below:

ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI,

where:			IACI is the adjacent channel interference.

-	Step 6: RF parameters are determined based on the degradation cause by ACI as below:

LossACI=1-ThputACIThputSINGLE

To simplify the simulation of interference from TN to NTN UL in Case 2 and 6, following method can be used. Consider the active TN cells from central NTN beam for the ACI evaluation from TN to NTN UL. The scaling factor is to be discussed and determined if any in next meeting. There is a view that simplifying such coexistence simulation work for Case 2 may even not be required.

-	Step 1: to drop NTN UE per beamprint randomly;

-	Step 2: to drop N clusters consisting of 57 sectors per beamprint randomly:

-	Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor:

where:			active_TN = active_factor*round (the area per beam/the area of 57 sectors)

active_factor = 20% (or lower, particularly for urban scenarios)

-	Step 4: to calculate the total ACI from all beams (e.g. M=7 ) for NTN:

2.3.2	Methods and principle to process co-existence simulation results

In order to process the co-existence simulation results received for all different scenarios and assumptions, the following steps are adopted:

-	Step 1: Discuss and agree on the most stringent scenario(s) for each scenario (Scenario 1, 2, 3…,6);

-	Step 2: Discuss and determine the required ACIR from results of the most stringent case(s) for each scenario;

-	Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each scenario

It is noted that the averaged ACIR for the most stringent case in each scenario would be derived by taking the average among the interpolated ACIR results derived from each company’s results for that case.

Moreover, the following considerations are adopted to deal with major disputes for the selected cases’ results in each scenario:

-	If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.

-	If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.

The processed results by adopting above principles and methods for scenarios 1 to 7 identified in Table 2.1-2 are captured in section 2.4. It is noted that due to the space limitation, only part of the simulation results for each case are presented, and the whole results for all studied options, as listed in Table 2.1-1 and section 2.2, can be found in Annex C.

Table 2.3.2-1: Selected option for each scenario

| Scenario | Aggressor system | Victim system | Environment | Contributing |
| --- | --- | --- | --- | --- |
| 4 | NTN GEO UL | TN UL | Urban | NTN UE ACLR |
| 5 | NTN GEO UL | TN DL | Rural | NTN UE ACLR |

3	Reference

[1] TR 38.863, v18.2.0

[2] R4-2217473, Way Forward on coexistence study for IoT over NTN, MediaTek

[3] R4-2414275, WF on UE RF requirements for NTN HPUE, Samsung