3GPP TSG-RAN WG4 Meeting #112	R4-2414284

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:			8.2.6

Source:	Moderator (Qualcomm)

Title:	WF for 15GHz UE parameters

Document for:	Approval

# Sub-topic 1-1: UE Type

At last meeting the following agreement was captured in the WF (R4-2410741):

Agreement: - For simulations, consider both options. - FR1 like - RX diversity gain: [0 or 5] dB assuming 4RX - TX: 0dBi omnidirectional - Max Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm as max TRP

Issue 3-1: UE Type assumption for comparison of FR1 Vs FR2 like UE performance

- FFS the below options for UE types:

  - Revisit RAN4#111 UE type assumption with the following options

  - Companies are encouraged to clarify their assumption when they provide simulation results.

  - FR1-like UE:

    - Option 1: 1TX assumption with omnidirectional antenna

    - Option 2: Two discrete elements following Table 1-1 below with maximum-gain across elements selection criterion.

    - Option 3: For [N] Tx/Rx, [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA.

      - Companies to clearly state what value of N and G assumed.

  - FR2-like UE:

    - FR2-likUE: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels critertion.

Table 1-1: FR1-like UE discrete antenna model

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |

Table 1-2: FR2-like UE antenna model

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 1, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

## Sub-topic 1-3 UE noise factor in the LS reply

At last meeting the following agreement was captured in the WF (R4-2410741):

Agreement: - 11dB for simulations. Actual noise factor for reply will be decided based on feasibility. Note that RAN4 agreed on 13 dB for UE in 7.125 – 8400 MHz frequency range.

Issue 3-3: UE noise factor for both simulation and LS reply

- FFS the below options:

  - Option 1: 11 dB i.e. follow simulation assumption

  - Option 2: 8 dB

  - Option 3: More than 11 dB

## Sub-topic 1-4: UE RF and Antenna parameter update to TR 38.922

Since the discussion on the UE beamforming feasibility is still ongoing, it is recommended to note R4-2411143 for this meeting and continue the discussion in the next two meetings.