3GPP TSG-RAN WG4 Meeting #112	R4-2414285

Maastricht, Netherlands, August 19 – August 23, 2024

Title: 	WF on 6Rx UE requirements

Agenda Item:	8.1.3

Source: 	AT&T

Document for:	Approval

# Topic 1:	REFSENS (delta RIB,6R)

## Sub-topic 1-1:	General considerations for specifying ΔRIB,6R value

Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case

Agreement: Include n104 in the high band (n77, n78, n79) category.

Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA

Agreement: Different value for handheld UE and FWA.

Issue 1-1-3: Verification of 6Rx receiver requirements

Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined.

## Sub-topic 1-2:	ΔRIB,6R values for handheld UE and FWA

Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA

Agreement: Based on the outcome of Issue 1-1-2, RAN4 to specify ΔRIB,6R values for handheld UE and FWA as follows.

-	Adopt the average value of ΔRIB,6R for handheld UE from company proposals (updated to include company proposals from RAN4#111) and specify it for handheld UE devices.

Table: Averaged values for handheld UE based on proposals

|  | Operating Bands | ΔRIB,6R (dB) |
| --- | --- | --- |
| MediaTek | Band n41 | -3.4 |
|  | Band n77, n78, n79, n104 | -3.1 |
| Xiaomi | Band n41 | -3.2 |
|  | Band n77, n78, n79, n104 | -3.0 |
| LGE | Band n41 | -3.5 |
|  | Band n77, n78, n79, n104 | -3.0 |
| Spreadtrum | Band n41 | -3.3 |
|  | Band n77, n78, n79, n104 | -3.0 |
| Meta | Band n41 | -3.5 |
|  | Band n77, n78, n79, n104 | -3.1 |
| vivo #1 | Band n41 | -3.0 |
|  | Band n77, n78, n79, n104 | -3.0 |
| vivo #2 | Band n41 | -3.2 |
|  | Band n77, n78, n79, n104 | -3.0 |
| ZTE | Band n41 | -3.5 |
|  | Band n77, n78, n79, n104 | -3.0 |
| Ericsson | Band n41 | -3.6 |
|  | Band n77, n78, n79, n104 | -3.2 |
| Google | Band n41 | -3.4 |
|  | Band n77, n78, n79 | -3.0 |
| Huawei | Band n41 | -3.3 |
|  | Band n77, n78, n79, n104 | -3.0 |
| Qualcomm | Band n41 | -3.4 |
|  | Band n77, n78, n79, n104 | -3.0 |
| Apple (R4-2407071) | Band n41 | -3.3 |
|  | Band n77, n78, n79, n104 | -3.0 |
| OPPO (R4-2408759) | Band n41 | -3.6 |
|  | Band n77, n78, n79, n104 | -3.2 |
| Average | Band n41 | -3.4 |
|  | Band n77, n78, n79, n104 | -3.0 |

-	Adopt the average value of ΔRIB,6R for FWA UE from company proposals indicating different FWA UE values from handheld UE values and specify it for FWA UE devices.

Table: Averaged values for FWA UE based on proposals

|  | Operating Bands | ΔRIB,6R (dB) |
| --- | --- | --- |
| LGE | Band n41 | -3.7 |
|  | Band n77, n78, n79, n104 | -3.4 |
| Spreadtrum | Band n41 | -3.6 |
|  | Band n77, n78, n79, n104 | -3.2 |
| vivo | Band n41 | -3.4 |
|  | Band n77, n78, n79, n104 | -3.2 |
| Google | Band n41 | -3.7 |
|  | Band n77, n78, n79 | -3.3 |
| Nokia | Band n41 | -4.0 |
|  | Band n77, n78, n79, n104 | -3.4 |
| Average | Band n41 | -3.7 |
|  | Band n77, n78, n79, n104 | -3.3 |

# Topic 2:	SRS antenna switching and ΔTRxSRS

## Sub-topic 2-1:	General considerations for SRS antenna switching and ΔTRxSRS

Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Way forward: Further discuss the following options.

Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Issue 2-1-2: Whether to consider separate ∆TRxSRS values when the device is power class 2 in the band and ΔPPowerClass = 0dB and not indicating Tx diversity capability

Agreement: Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability.

## Sub-topic 2-2:	ΔTRxSRS values

Issue 2-2-1: Proposed ∆TRxSRS values

Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers.

-	Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n41, n77, n78 | 4.0 | 3.5 | 4.5 |

-	Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

-	Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n104 | 5.7 | 5.0 | 6.3 |

# Topic 3:	MIMO layer evaluation for 6Rx UE

## Sub-topic 3-1:	General considerations for MIMO layer evaluation for 6Rx UE

Issue 3-1-1: Tightening BS EVM requirement

Agreement:

-	Do not tighten the minimum requirements of BS Tx EVM

-	Further discuss the assumption of Tx EVM for 6-layer performance evaluation.

Issue 3-1-2: 6-Layer Performance Evaluation Assumptions

Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios.

Issue 3-1-4: Performance requirements for 6Rx

Agreement: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices.

## Sub-topic 3-2:	6-layer Support

Issue 3-2-1: 6-layer Support

Way Forward: RAN4 to further discuss the following options.

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices.

-	Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point.

-	Feasibility criteria

-	Consider 4-layer vs 6-layer throughput performance.

-	Simulation assumptions

-	Only consider link-level simulation for performance evaluations.

-	Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM.

-	Option 4: 6-layer support is not feasible for handheld UEs.

Issue 3-2-2: 6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

# Topic 4:	SRS IL imbalance issue

## Sub-topic 4-1:	General considerations for SRS IL imbalance issue

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751.

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL.

-	Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL.

-	Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

## Sub-topic 4-2:	SRS IL imbalance issue solutions

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

Annex A:
Feasibility Study Link-Level Simulation Assumptions

Table: Feasibility Study Link-Level Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz or 100MHz |
| SCS |  | 30 |
| Antenna configuration |  | Option 1: 16Tx, 6Rx Option 2: 8Tx, 6Rx |
| Propagation channel |  | Option 1: TDLA30-5 Option 2: TDLA30-10 Option 3: TDLC300-100 |
| Rank |  | Fixed rank [4,5,6] |
| MCS |  | Option 1: Adaptive MCS (target BLER 10%) Option 2: MCS13, MCS17 from Table 1 Other MCS options are not precluded |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Rel-15 Type I |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Configurations | Option 1: [1+1(4L), 2+0(6L)] Option 2: [2+2(4L), 2+2(6L)] Option 3: [1+1 (4L), 2+2 (6L)] |
|  | Precoding | Random (further check) |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: Other options are not precluded. |