3GPP TSG-RAN WG4 Meeting #112	R4-2414304

# Maastricht, Netherlands,19 – 23 Aug, 2024

Agenda item:			8.20.4

Source:	CMCC

Title:	WF on Co-existence study for ambient IoT

Document for:	Approval

Introduction

This way forward captures the agreements for co-existence evaluation for Rel-19 ambient IOT study item.

The summary in RAN4#112 is R4-2412833. The way forward agreed in previous RAN4 meetings are R4-2406714 and R4-2410567.

Deployment scenarios and spectrum usage

- Topic 2-1: Deployment scenario

Issue 2-1-1: deployment scenarios for D1T1

Agreement in RAN4#110bis: Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting) Agreement: - RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2. Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation Agreement: - Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation - A-IoT is located within a NR transmission bandwidth configuration - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS Agreement in RAN4#111: - Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT - Encourage companies to provide the simulation results for option 1-1 and 1-2 - FFS on co-site scenario (option 2-1 and 2-2)

Agreement in RAN4#112:

- Feasibility of option 2-1/2-2 for device 1 and 2a can be evaluated with LLS. Parameters for LLS are based on company report.

  - Encourage companies to provide the co-existence simulation results.

- Topic 2-2: Spectrum usage

Issue 2-2-1: Spectrum usage for R2D in D1T1

Agreement in RAN4#110bis: - FFS on whether to prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation. Agreement in RAN4#111: - Use FDD DL as starting point for co-existence evaluation for R2D in D1T1 - FFS on FDD UL spectrum.

Agreement in RAN4#112:

- Use FDD DL as baseline for co-existence evaluation for R2D in D1T1, FDD UL can be considered as optional

Issue 2-2-2: CW spectrum for D2T2

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. · Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum · Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum · Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement in RAN4#111: - Use case 2-2 as starting point for co-existence evaluation for calibration. - FFS on case 2-3 - Further discuss the difference of outside topology (case2-4) from co-existence study perspective.

Agreement in RAN4#112:

- For D2T2, CW transmitted in UL is baseline for co-existence evaluation, CW transmitted in DL (i.e. case 2-3) can be considered once RAN4 agreed on the CW distribution for outside topology.

Evaluation methodology and cases

- Topic 3-1: Evaluation methodology

Issue 3-1-1: LLS to derive guard RBs

Agreement:

- perform the study within the scope of the feasibility study in issue 2-1-1.

Issue 3-1-2: SINR vs BLER

Agreement:

- RAN4 does not perform LLS to derive SINR values for R2D and D2R in this SI.

- RAN4 can take RAN1 LLS as the reference to derive SINR values for R2D and D2R in this SI.

Issue 3-1-3: Assumption of R2D transmission and CW transmission for evaluation

Agreement:

- Assume transmission timeline for R2D and CW signal among different A-IoT BS/CW node transmission are aligned for co-existence evaluation.

Issue 3-1-4: Assumption of R2D and NR UL for D2T2

Agreement:

- Assume intermediate UE in D2T2 transmitted R2D and transmitted NR UL in TDM manner for co-existence evaluation.

Issue 3-1-5: whether to study interference mitigation scheme for scenario option 1-2

Agreement:

- RAN4 focus on the co-existence evaluation in this study item.

- Topic 3-2: Performance metric and SINR definition

Issue 3-2-1: Performance metric for AIOT

Agreement in RAN4#110bis: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded Agreement in RAN4#111: - Use SINR for calibration purpose - FFS on performance metric for co-existence evaluation and requirements definition.

Agreement in RAN4#112:

For inter-system interference (between AIOT and NR):

- If SINR degradation is smaller than or equal to [1]dB, it can be considered that inter-system interference is negligible.

- If SINR degradation is lager than [1]dB, consider following criteria:

  - Outage percentage consider SINR level with [10%] BLER

  - FFS on the outage percentage

- Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

For intra-system interference (between AIOT and AIOT):

  - Outage percentage consider SINR level with [10%] BLER

  - FFS on the outage percentage

Issue 3-2-2: SINR definition for D2R

Agreement in RAN4#111: Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2 FFS on how to consider CW cancellation capability in formal simulation

Agreement in RAN4#112:

- SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

- SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intra_system interference + inter_system interferencewithin total receiver bandwidth+residual CW interference after cancellationin linear scale

Issue 3-2-3: SINR definition for R2D

Agreement in RAN4#111: SINR for R2D for calibration purposes - signal power of device to the noise and interference within 10MHz - Assume interference NR BW is 10MHz - FFS on BB LPF

Agreement in RAN4#112:

- signal power of device to the noise and interference within 10MHz is baseline assumption

- Consider [180KHz] noise and interference bandwidth after BB LPF as optional

- Topic 3-3: CW considerations

Issue 3-3-1: Layout of CW for outside topology

Agreement in RAN4#112:

- The CW outside topology layout is the same as AIoT reader

- For each device, the nearest CW node will be activated during the simulation

  - CW node is not co-located with any activated reader.

Issue 3-3-2: CW cancellation capability

Agreement in RAN4#111: Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2 FFS on how to consider CW cancellation capability in formal simulation

Agreement in RAN4#112:

- Companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different.

  - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation

  - CW transmission impact on the received SINR, e.g. degradation

Issue 3-3-3: CW unwanted emissions

Agreement:

- FFS on how to model CW unwanted emissions.

- Topic 3-4: Evaluation cases

Issue 3-4-1: Evaluation cases for co-existence evaluation

| No. | deployment scenario and topology | spectrum | (Aggressor -> Victim) | note |
| --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 4 (O) | D1T1-B-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-1 (O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-2 (O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |

Issue 3-4-3: Device type

Agreement:

- Prioritize device 1 and 2a without a frequency shifter for coexistence evaluation.

Issue 3-4-4: Multi-operator scenario

Agreement:

- Focus on the co-existence study between A-IoT and NR in this study item.

- The scenario of multiple A-IOT operators in the same band can be considered in the WI phase or in the study of the future release.

Evaluation parameters for formal simulation

- Topic 4-1: Adjacent RB Tx and Rx characteristics

Issue 4-1-1:  A-IOT reader and NR BS

Agreement:

|  | In-band |  |
| --- | --- | --- |
|  | Tx | Rx |
| NR UE/A-IOT Intermediate UE | Legacy UE IBE | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| NR BS | Option 1: 30dBc Option 2: 17dBc Companies to check the leakage of legacy NR BS. | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| A-IOT BS | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dBc，ACLR2:50dBc) | ACS of legacy gNB |

Issue 4-1-2: Tx for device 1 and 2a

Agreement in RAN4#111: For device 1 and 2a, 25dBc is used for calibration purposes

Agreement in RAN4#112:

  - 25dBc with 15KHz bandwidth as baseline

  - 16dBc with 90KHz bandwidth as optional

Issue 4-1-3: Rx for device 1 and 2a

Agreement in RAN4#111: Assume no frequency selectivity for co-existence evaluation for calibration purposes for device 1 and 2a.

Agreement in RAN4#112:

- Alt-1: Assume no frequency selectivity

- Alt-2:

| SINR | R2D with LPF |
| --- | --- |
| Interference from NR | Frequency selectivity: [7.5 dB] |
| Noise bandwidth | [180kHz] |
| SINRR2D_with_LPF=Wanted_SR2D_180kHzNoiseFloor 180kHz+(InterferenceNR10MHz/LPF_Selectivitylinear) |  |

- Alt-3:

| For R2D receiver with LPF |  |
| --- | --- |
| RF BW | 10MHz |
| BB BW | [180kHz] |
| SINRi (linear, not dB) | Input SINR at the antenna connector with noise/interferencde BW=RF BW |
| SINRo (linear, not dB) | Output SINR after BB LPF assuming downconverted noise/interference BW=BB BW |
| SINRo=SINRi28*SINRi+4*BWRFBWBB |  |

- Alt-4: other LLS assumptions are not excluded

Issue 4-1-4: Scaling factor

Agreement:

- when A-IoT reader as victim, the scaling factor is suggested as below to compensate different aggressor and victim bandwidth when calculating inter-system interference.

- Scaling factor = 10*log10BWaggressorBWvictim

- Topic 4-2: General parameters and layout

Issue 4-2-1: Active rate of reader for D1T1 and D2T2

Agreement:

- Minimum distance between active readers: 60m as baseline, other values can be reported by other companies.

-	   2 readers are activated in one drop as baseline

Issue 4-2-2: Indoor UE percentage for scenario option 1-2

Agreements in RAN4#111: For scenario option 1-2, uniformly distributed, 80% indoor, 20% outdoor

Agreements in RAN4#112:

- Option 1: 10%

- Option 2: 100%

Issue 4-2-3: Minimum NR BS-NR UE distance (2D)

Agreements in RAN4#111: Minimum NR BS – NR UE distance (2D): 35 m

Agreements in RAN4#112:

- Use MCL of 70 dB for Minimum NR BS – NR UE distance

Issue 4-2-4: Minimum distance between device and reader for D1T1 and D2T2

Agreements in RAN4#111: For D1T1: MCL between device and reader is 45dB for calibration. For D2T2: Minimum distance between reader and device is 1m

Agreements in RAN4#112:

- Minimum distance between reader and device is 1m.

Issue 4-2-5: NR RB allocation

Agreements in RAN4#111: NR UE number: - DL active UE: 1 UE per cell - UL active UE: 3UE per cell

Agreements in RAN4#112

- For RB allocation, each UE is scheduled with 17PRB and A-IoT using 1PRB is located between the most two UEs. Detailed illustration is listed as below:

Issue 4-2-6: Penetration loss for O2I

Agreement:

- Use the equation of 7.4-2 in 38.901

- PLin = 0.5 * d2D-in where d2D-in is the distance to nearest factory/office boundary on the line between Tx and Rx point.

- Set maximum value of d2D-in as [25m] as optional

Issue 4-2-7: Pathloss

Agreement:

|  | D1T1 | D2T2 |
| --- | --- | --- |
| Indoor legacy UE <-> indoor device | Indoor office |  |
| Indoor legacy UE <-> indoor reader | Indoor factory DH | Indoor office |
| Outdoor macro gNB <-> indoor device/indoor reader | PLb: Uma |  |
| Outdoor UE <-> indoor device/ D2T2 UE, i.e. UE<->UE | PLb: Umi |  |
| Outdoor UE <-> indoor D1T1 reader, i.e. UE<-> reader | PLb: UMi |  |
| Note: For other indoor factory related parameters that are not listed, it’s suggested to refer to 7.8.4 of TR 38.901. |  |  |

Issue 4-2-8: Other general parameters

| General Parameter | D1T1&D2T2 Values for calibration purposes |
| --- | --- |
| Carrier frequency | 900MHz |
| BW for NR | 10MHz with 15KHz SCS |
| BW for AIOT system | 180KHz |
| Waveform (CW) | CW: Unmodulated single tone |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Traffic model | Full buffer |
| Frequency reuse | 1 |

- Topic 4-3: Parameters for AIOT BS/intermediate UE and device

Issue 4-3-1: AIOT micro-BS parameters for D1T1

Agreement:

| A-IoT micro BS parameters | Values for calibration purposes |
| --- | --- |
| A-IoT micro-BS total Tx power | 33dBm |
| A-IoT micro-BS receiver Noise Figure（dB） | 10 |
| A-IoT micro-BS antenna gain (dBi) | 6 dBi |
| Antenna pattern | Antenna Array Geometry： - 1*1*1 antenna element - equals to omni-directional antenna pattern in GCG in horizontal Parameter; Assumption. Antenna pattern (horizontal); AHϕ=-min12ϕϕ3dB2,Am, = 90°, Am = 15 dB. Antenna pattern (vertical); AVθ=-min12θ-90°θ3dB2,SLAv, = 90°, SLAv = 15 dB. Combining method in 3D antenna pattern. BS antenna gain (dBi) (including feeder loss); 6. |

Issue 4-3-2: Intermediate UE parameters for D2T2

Agreement:

| intermediate UE parameters | Values for calibration purposes |
| --- | --- |
| intermediate UE total Tx power（dBm） | 23dBm |
| gain of antenna intermediate UE (dBi) | 0 |
| intermediate UE receiver Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

Issue 4-3-3: AIOT device parameters

Recommended WF:

| A-IoT device parameters | Device 1 | Device 2a |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | [0] |
| A-IoT device reflection （backscatter）loss (dB) Note: due to, e.g., impedance mismatch | OOK: -6 dB | OOK: -6 dB |
| A-IoT device power gain of reflection amplifier (dB) | N/A | 10(M) |
| A-IoT Device receiver sensitivity (dBm) Use this value to determine whether device can camp on the cell. | -36 | [-45] |
| A-IoT device noise figure (dB) | 24 | [20] |
| Guard band | 0PRB | 0PRB |

- Topic 4-4: Paramters for legacy NR

Issue 4-4-1: NR macro BS parameters

Agreement:

| NR macro-BS Parameter | Values for calibration purposes |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry： - 1*1*1 antenna element - BS point at fixed beam direction - vertical: θtilt + 90° - horizontal: 0, 120, 240 ° Parameter; Assumption. Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns); AHϕ=-min12ϕϕ3dB2,Am = 65 degrees, Am = 25 dB. Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns); AVθ=-min12θ-90°-θetiltθ3dB2,SLAv = 10 degrees, SLAv = 25 dB, = 9 degrees. Combining method in 3D antenna pattern. BS antenna gain (dBi) (including feeder loss); 15. |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |

Issue 4-4-2: NR UE parameters

Agreement:

| NR UE Parameter | Values for calibration purposes |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

- Topic 4-5: Paramters for CW

Issue 4-5-1: Other CW parameters

Agreement:

| CW parameters | D1T1 | D2T2 |
| --- | --- | --- |
| Tx power（dBm） | If UL spectrum is used, UE Tx power is assumed, i.e. 23dB If DL spectrum is used, AIOT micro-BS Tx power is assumed. | Inter-mediate UE Tx power is assumed. |
| Antenna gain | Same as AIOT reader | Same as inter-mediate UE |

- Topic 4-6: Parameters for collocated scenario (option 2-1 and 2-2)

Issue 4-6-1: Simulation assumptions for collocated scenario

Agreement:

Reuse the evaluation parameters of scenario option1-1 and 1-2, additionally use following parameters.

1) NR indoor BS deployed co-site with A-IoT indoor reader;

2) NR indoor BS ISD as 20m;

3) NR indoor Min BS-UE distance: 0m;

4) NR indoor UE uniformly distributed.

5) NR indoor BS Tx power assumed [24] dBm

6) Apart from Tx power in 5), other NR indoor BS characteristics, e.g. antenna pattern, use agreed A-IoT BS as starting point

7) The self-interference cancellation of R2D in UL for co-located scenario can be reported by interested company as starting point.