3GPP TSG-RAN WG4 Meeting #112 	R4-2414307

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	T-Mobile USA

Title:	TP for TR38.750 for DL CA_n25-n66 with PC2 2BUL

Agenda item:	7.8.4

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2BDL/2BUL for CA_n66-n71. It is a result of Apple flagging missing MSD for PC2 2BDL/2BUL for CA_n25A-n66A During RAN4#112.

The power class 2 MSD is calculated by calculating the interference power for PC3 IMD MSD, increasing the interference by 3 dB * IMD order, and then calculating the resulting MSD for PC2. The equations are included.

- Reference

[1] RP- 241679, New WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR SUL (supplementary uplink)

- Text Proposal

<Beginning of changes>

## 5.x	CA_n25-n66

### 5.x.1	UE maximum output power

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz)(NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A | n25A8 n66A8 CA_n25A-n66A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |

NOTE 3:   For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs.

NOTE 8:   Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

### 5.x.2	Reference sensitivity requirements

For PC3, DL CA_n25A-n66A with UL CA_n25A-n66A has intermodulation MSD. This section will propose MSD for UL CA_n25A-n66A based on the PC3 MSD. The PC3 MSD for CA_n25-n66 is:

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |

MSD for PC2 UL CA is calculated from IMDy MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMDy increases by 3*y dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 7.3A.5-1a:

Table 7.3A.5-1a: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |  |
|  | n25 | 1855 | 5 | 25 | 1935 | 29 | FDD | IMD3 |  |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 32 | FDD | IMD3 |  |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |  |
|  | n66 | 1750 | 5 | 25 | 2150 | 16.9 | FDD | IMD5 |  |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |  |

### 5.x.3	∆TIB and ∆RIB values (Void)

<End of changes>