3GPP TSG-RAN WG4 Meeting #112										         			R4-2414312

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.3.4

Source:	OPPO

Title:	WF on NR sidelink for intra-band non-contiguous CA

Document for:	Information

Introduction

This WF captures the online agreements of intra-band non-contiguous CA.

Topic #1: System Parameter

Issue 2-1-1: Operating band

Agreement: Merge two tables.

- Table 5.1.2-1 Intra-band non-contiguous SL CA operating bands in FR1

| NR intra-band non-contiguous SL CA operating Band | NR SL Operating Band | Interface |
| --- | --- | --- |
| SL_n47(2A) | n47 | PC5 |
| NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers in n47 band. |  |  |

- Table 5.1.1-1: Intra-band contiguous CA operating bands for SL CA in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SL_n47 | n47 | PC5 |

Issue 2-1-2: Channel bandwidth

Agreement:

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47(2A) | SL_n47(2A) | 10 | 10, 20 |  |  | 30 | 0 |

Topic #2: UE TX RF requirement

Issue 2-1-2: How to define the requirement

Agreement:

- Define MPR to meet -13dBm/MHz and MPR to meet -30dBm/MHz as NR intra-band non-contiguous CA

- Consider B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 for PSSCH as NR intra-band non-contiguous CA

PSSCH/PSCCH with 2x20dBm PA + 1LO or 2x20dBm PA + 2LO

Agreement: use PSSCH/PSCCH with 2x20dBm PA for PC3 sidelink non-contiguous CA evaluation.