3GPP TSG-RAN4 Meeting #112	                                                                 R4-2414322

Maastricht, Netherlands, 19th – 23th Aug 2024

Agenda item:			8.10.5

Source:	CMCC

Title:	WF on Rel-19 ATG UE RF requirements

Document for:	Approval

Introduction

This document captures the agreements on Rel-19 ATG UE RF requirements in RAN4#112.

Topic #1: R18 UE RF requirements maintenance

Tx requirements applicability

Issue 1-1: Tx requirements applicability for ATG Tx requirements for two types of antenna

Agreement:

- There are two types of antenna type for R18 ATG UE. For omni-direction antenna, the requirements should be measured at antenna connectors but for antenna array, the requirements should be measured at TAB connectors.

- Following requirements are defined as the sum of all antenna connectors/TAB connectors

  - 6.3J.1	Minimum output power for ATG

  - 6.3J.4	Power control for ATG

  - 6.5J.2	Out of band emission for ATG

  - 6.5J.3	Spurious emissions for ATG

- Following requirements are defined at each antenna connector/TAB connector

  - 6.3J.2	Transmit OFF power for ATG

  - 6.3J.3	Transmit ON/OFF time mask for ATG

- FFS the following requirements.

  - 6.5J.1	Occupied bandwidth for ATG

  - 6.4J.1	Frequency error for ATG

  - EVM requirements

Topic #2: R19 UE RF for intra-band contiguous CA

Issue 2-1-2: whether/how to reflect operation band information in spec

Agreement:

- BCS 4 and 5 also needs to be specified besides BCS0.

Table 1: BCS for CA_n79C

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 200 | 4 and 5 |

Issue 2-2: ACS testing parameters for case 2

Agreement:

- Further check the following table.

- Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

Topic #3: R19 UE RF for inter-band CA

antenna type assumption for inter-band DL CA_n3-n39

Issue 3-1-1: clarify the antenna type for DL CA_n3-n39

Agreement:

- Omni-antenna type can be assumed for both band n3 and n39.

Issue 3-1-2: whether new capability is needed for ATG CA

Agreement:

- no need for new capability for UE antenna type for ATG CA

- Antenna type per band could be applied to the same band in the band combination.

RF implementation for inter-band CA_n3A-n39A

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input.

  - The filter with [X] MHz frequency gap can be considered.

Issue 3-2-2: BCS for DL CA_n3_n39

Agreement:

- Table 1: BCS for CA_n3A-n39A

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

Rx requirement for inter-band CA_n3A-n39A

Issue 3-4-1: delta RIB for NR CA_n3A-n39A could be used as the baseline

Agreement:

Table 3: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n39 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

Issue 3-4-2: MSD for CA_n3A-n39A

Agreement:

- There is no MSD issue for n3 impact n39.

Issue 3-4-3: OOB exception for CA_n3A-n39A

Agreement:

- No need to specify OOB blocking exception for CA_n3-n39 since the second order intermodulation product of the n3 UL carrier and the CW interfering signal will not overlaps with the n39 DL carrier.

Topic #4: R19 UE RF for UL-MIMO

Issue 4-1-1: ULFPTx mode for UL-MIMO

Agreement:

- FFS on whether to apply ULFPTx mode for ATG UE

Issue 4-1-2: whether preclude the single antenna port related requirement

Agreement:

- FFS on whether to preclude the single antenna port related requirement.

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- If the legacy requirement for UL MIMO is defined at each antenna connector,

  - For ATG UE with antenna array, the requirement should be updated [at each TAB connector].

- If the legacy requirement for UL MIMO is defined per layer, FFS

  - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer.

  - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector.

Issue 4-2-2: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

- FFS on whether to apply coherent UL MIMO requirement for ATG UE

Issue 4-3: receiver requirements

Agreement:

- Reuse ATG single carrier requirements for following Rx requirements (ZTE, Ericsson)

  - REFSENSE

  - Max input level

  - ACS

  - Blocking

  - Spurious emission

  - Intermodulation

- FFS whether to test the Rx requirements under the UL MIMO scenario since Rx requirements is the same for R18 ATG UE and R19 ATG UE supporting UL-MIMO.