3GPP TSG-RAN WG4 Meeting #112	R4-2414330

Maastricht, Netherlands, 19th August – 23th August 2024

Source:	Ericsson, Telstra

Title:	TP for 38.719-02-01 adding UL CA_n78(2A) to DL CA_n26A-n78(2A) and DL CA_n26(2A)-n78(2A)

Agenda item:			7.3.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.719-02-01 adding UL CA_n78(2A) to DL CA_n26A-n78(2A) and DL CA_n26(2A)-n78(2A).

# 2. Text Proposal

# ---Start of changes---

## 5.x		CA_n26-n78

### 5.x.1		Specific for 1 bands UL CA

#### 5.x.1.1 Operating bands for CA

CA_n26-n78 has already been specified in TS 38.101-1 and this section does not need to be revisited.

#### 5.x.1.2	Channel bandwidths per operating band for CA

UL CA_n78(2A) need to be added to DL CA_n26A-n78(2A) and DL CA_n26(2A)-n78(2A) in the existing configuration table of 38.101-1. See Table 5.x.1.2-2 below.

Table 5.x.1.2-2: Supported bandwidths per CA band combination of band n26+n78

| CA_n26A-n78(2A) | CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) | n26 | n26 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n26(2A)-n78(2A) | CA_n26(2A) CA_n78(2A) CA_n26A-n78A | n26 | CA_n26(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.2.2-3 gives a non-contiguous uplink IMD interference analysis

Table 5.x.2.2-3: Non-contiguous uplink IMD analysis

| UE UL carriers | fx_low = 3300 | fx_high = 3320 | fy_low = 3780 | fy_high = 3800 |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 460 – 500 |  | 7080 – 7120 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2800 – 2860 |  | 4240 – 4300 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10380 – 10440 |  | 10860 – 10920 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6100 – 6180 |  | 8020 – 8100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 920 – 1000 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 13680 – 13760 |  | 14640 – 14720 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 14160 – 14240 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11800 – 11900 |  | 9400 – 9500 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4700 – 4800 |  | 2300 – 2400 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18420 – 18520 |  | 16980 – 17080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17940 – 18040 |  | 17460 – 17560 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the table 5.x.2.2-3, there are no IMD products falling inside n26 DL.

#### 5.x.1.4	∆TIB and ∆RIB values

CA_n26-n78 has already been specified and this section does not need to be revisited.

#### 5.x.1.5	REFSENs requirements

Based on the co-existence studies there is no need to define MSD values.

---End of changes---

# Reference

[1] 		RP-241674, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Moderator (RAN4 Chair, Huawei)

[2]		TS 38.101-1 v18.6.0