3GPP TSG-RAN WG4 Meeting # 112					                R4-2414349

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	Skyworks, Murata, China Unicom, CHTTL.

Title:	WF on DC_(n)8AA

Agenda item:	7.2

Document for:	Approval

Introduction

Two companies proposed MSD test points for DC_(n)8AA:

- One company proposed two pairs of test points [1]:

  - A pair for dual-uplink operation,

  - A pair for one uplink operation.

- Another company proposed to restrict this band combination to single switched uplink only operation due to concerns on very high MSD for dual-uplink operation [2]. A single test point was proposed to verify the LTE anchor point MSD for single uplink operation with near identical configuration than [1].

During the meeting, the proponents clarified that only single switched uplink operation is intended, and that two MSD test points are preferred. This WF captures these agreements.

WF

<Way forward on Single-Switched Uplink for DC_(n)8AA>

For DC_(n)8AA, only single switched uplink operation is supported.

<Way forward on MSD test point for DC_(n)8AA>

Interested companies are invited to evaluate the DC_(n)8AA MSD for the following test points.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | FFS | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | FFS |  |
|  | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | FFS |  |
|  | n8 | N/A | 20 | N/A | 940 | FFS |  |
| NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

- References

- R4-2411319, Test points for DC_(n)8AA BCS0, 3GPP TSG-RAN WG4 Meeting # 112, Maastricht, The Netherlands, Murata Manufacturing Co. Ltd.

- R4-2413070, DC_(n)8AA MSD, 3GPP TSG-RAN WG4 Meeting # 112, Maastricht, The Netherlands, Skyworks Solutions, Inc.