3GPP TSG-RAN WG4 Meeting #112	R4-2414351

Maastricht, Netherland, August 19 – 23, 2024

Title: 	Way Forward on intra-band UL CA interference and RF requirements

Agenda Item:	7.1

Source: 	Huawei, HiSilicon, ….

Document for:	Approval

Background

In the RAN4#112 meeting, companies provided some contributions [1][2][3][4][5][6][7] to discuss the intra-band UL CA interference and corresponding RF requirements.

In the contribution [1], companies provided the measurement results for intra-band UL CA and single carrier. The intra-band UL CA interference with some discrete RB allocations can be observed as an example below.

In the contribution [2], operators show the interests and preference and it’s expected that the typical scenario for intra-band UL CA can be considered when RAN4 specify the RF requirements instead of only considering the extreme cases.

In the contribution [3], companies proposed to reconsider the 1RB+1RB RB allocation for intra-band TDD ULCA within an inter-band DL CA and that fully allocated maximum aggregated BW is considered to define the cross-band MSD requirements resulting from intra-band contiguous UL CA.

In the contribution [6], companies observed the following points:

- The extreme discrete RB allocations for intra-band UL CA will lead to the poor RF performance for UE and decrease the output power.

- From network perspective, the extreme discrete RB allocations for intra-band UL CA not only can’t achieve higher UL throughput, but also network have to face the system performance loss due to the reduction of UE output power.

Thus, from RAN4 perspective, some reasonable UL RB allocations for intra-band UL CA can be recommended to benefit both UE and network. Some extreme cases with discrete RB allocations can be discussed from test perspective, but it’s still allowed for proponent to care about the typical scenario.

After ad-hoc discussion, a WF is proposed to capture the background and some consensus in order to address this issue.

Way forward on intra-band ULCA within an inter-band DL CA

Proposal 1: In Rel-19, to keep as-is in spec as most of cases are covered and some history is built since Rel-17.

Proposal 2: For intra-band TDD ULCA within an inter-band DL CA, the proponent/operator can choose either one of the following test configurations for MSD analysis and potential definition of RF requirements.

- Test case 1: Discrete UL RB allocation, e.g. 1RB + 1RB UL RB allocation in UL intra-band CA.

- Test case 2: fully UL RB allocation with maximum aggregated BW.

Proposal 3: For the specific cases DL_n40A-n41C_UL_n41C and DL_n41C-n79A_UL_n41C, fully UL RB allocation with maximum aggregated BW will be used for MSD analysis and potential definition based on the evaluation. It’s encouraged for companies to provide the evaluation in next meeting.

Proposal 4: From RAN4 perspective, some reasonable UL RB allocations for intra-band UL CA can be recommended to benefit both UE and network and the corresponding condition can be reflected in the spec.

For example, it’s proposed to introduce the following condition into the specification.

For intra-band UL CA, the [Tx] RF requirements are specified based on the UL RB allocations with the following assumed conditions. Lcrb1×2μ1+Lcrb2×2μ2≥Z Where, Z=min(NRB1×2μ1, NRB2×2μ2) Lcrb1 is the Transmission bandwidth in CC1, which represents the length of a contiguous resource block allocation expressed in units of resources blocks. Lcrb2 is the Transmission bandwidth in CC2, which represents the length of a contiguous resource block allocation expressed in units of resources blocks. NRB1 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks. NRB2 is the Transmission bandwidth configuration in CC1, expressed in units of resource blocks. μ1/μ2 is same as μ defined in TS 38.211 [6]

Reference

[1] R4-2411551, On intra-band ULCA interference versus single ULCC, Skyworks Solutions Inc.

[2] R4-2411738, Discussion on MSD requirements with intra-band contiguous UL CA, CMCC

[3] R4-2411887, On MSD requirements with intra-band contiguous UL CA, ZTE Corporation, Sanechips

[4] R4-2412934, Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, Huawei, HiSilicon

[5] R4-2412935, Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C, Huawei, HiSilicon

[6] R4-2412940, Discussion on the reasonable UL RB configurations for intra-band UL CA, Huawei, HiSilicon

[7] R4-2412941, Draft CR for TS 38.101-1 to introduce the reasonable UL RB configurations for intra-band UL CA, Huawei, HiSilicon