3GPP TSG-RAN WG4 Meeting #112	R4-2414410

# Maastricht, Netherlands, 19th – 23rd Aug, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1290 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC) Correction on configured output power of 3Tx EN-DC including UL MIMO and Tx diversity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the 3Tx with UL MIMO EN-DC configured output power section, it mentions that the requirements in 6.2B.4.1.3 apply except the MPR and A-MPR for the NR component carrier if with UL MIMO configured. However, the PPowerClass,EN-DC & ∆PPowerClass,EN-DC also need to be referred to the 3Tx related sections instead of refering to section 6.2B. Same issue in the 3Tx with Tx diversity EN-DC configured output power section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding descriptions for PPowerClass,EN-DC & ∆PPowerClass,EN-DC under the sections 6.2H.4.1.3, providing the correct reference for the configured output power of 3Tx EN-DC. Similar changes are applied in section 6.2L.4.1.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification seems not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.4.1.3, 6.2L.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2H.4	Configured output power for DC with UL MIMO

6.2H.4.1	Configured output power level

6.2H.4.1.1	void

6.2H.4.1.2	void

##### 6.2H.4.1.3	Inter-band EN-DC with UL MIMO within FR1

For inter-band EN-DC with UL MIMO in one NR band, the requirements in clause 6.2B.4.1.3 apply except that:

-	PPowerClass,EN-DC is the maximum UE power specified in Table 6.2H.1.3-1 without taking into account the tolerance;

-	If the NR component carrier is configured with UL MIMO, the MPRc and A-MPRc are specified in clause 6.2D.2 and clause 6.2D.3 of [2] respectively.

-	∆PPowerClass,EN-DC:

–	For a power class 2 capable UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2H.1.3, otherwise ΔPPowerClass,EN-DC = 0 dB;

<< Next changes >>

6.2L.4	Configured output power for DC with Tx Diversity

6.2L.4.1	Configured output power level

6.2L.4.1.1	void

6.2L.4.1.2	void

##### 6.2L.4.1.3	Inter-band EN-DC with Tx Diversity within FR1

For inter-band EN-DC with Tx Diversity in one NR band, the requirements in clause 6.2B.4.1.3 apply except that:

-	PPowerClass,EN-DC is the maximum UE power specified in Table 6.2L.1.3-1 without taking into account the tolerance;

-	If the NR component carrier is configured with Tx Diversity, the MPRc and A-MPRc are specified in clause 6.2G.2 and clause 6.2G.3 of [2] respectively.

-	∆PPowerClass,EN-DC:

–	For a power class 2 capable UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2L.1.3, otherwise ΔPPowerClass,EN-DC = 0 dB;

<< End of changes >>