3GPP TSG-RAN4 Meeting #112					R4-2414435

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	China Telecom

Title:	WF on downlink interruption for UL Tx switching

Agenda item:			7.1

Document for:	Approval

Background

This WF captures the agreements for the criteria for analyzing DL interruption applicability for TX switching among 3 or 4 bands, and give the definition format for indicating no DL interruption requirement in the spec.

Wayforward

Issue 2-1-1: Applicability criteria

Agreement: Considering the following criterias when deciding to mandate no DL interruption applicability for TX switching among 3 or 4 bands

- Frequency separation between the DL band and the bands involved in the Tx switching for FDD+TDD band combinations and the corresponding higher order combinations

- Whether the same UL-DL duplex mode is used for TDD+TDD band combinations and the corresponding higher order combinations

  - Whether simultaneous RX/TX is mandated between the DL band(s) and the UL band(s)

- Whether same antenna group is used for band combinations and the corresponding higher order combinations

  - Number of TX chains to be switched

Issue 2-2-1: Definition format for indicating no DL interruption requirement

Wayforward: Considering the following format as starting point for indicating no DL interruption requirement for TX switching among 3 or 4 bands

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n1-n3-n783 | n1, n3, n78 | No for CA_n1-n78, CA_n3-n78 |
| CA_n3-n5-n78 | n3, n5, n78 | No for CA_n3-n78, CA_n5-n78 |
| CA_n1-n3-n5-n78 | n1, n3, n5, n78 | No for CA_n1-n78, CA_n3-n78, CA_n5-n78 |