3GPP TSG-RAN WG4 Meeting #112	R4-2414441

Maastricht, Netherlands, 19th -23th Aug, 2024

Title: 	WF on power domain enhancements

Agenda Item:	8.1.3

Source: 	Huawei, HiSilicon

Document for:	Approval

Topic #1: Power domain enhancements for single carrier

Sub-topic 1-1: Approaches to enable MPR reduction for scenario 1

Issue 1-1-1: Approaches for scenario 1

- WF

  - Further evaluate the reduced MPR values with relaxation of ACLR/SEM to certain levels or waived

  - Other ways to reduce MPR are not precluded

Issue 1-2-1: Clarification of BS CBW

- Proposals

  - Proposal 1: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios. (Huawei)

- WF

  - FFS in next meeting

Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation

- Proposals

  - Approach 1: UE CBW based

    - Option 1: The extended UE CBW based method as captured in the WF in last meeting.

  - Approach 2: BS CBW based

    - Option 2: BS CBW based inner/outer method. (Skyworks)

      - BS NRB instead of UE NRB.

      - Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart.

      - The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR.

      - For margin or stricter emissions, the BS can signal a reduced NRB and RBshift.

    - Option 2a: the feature ‘narrower UE channel BW within wider BS bandwidth’ is specified for a UE configured with a UE-specific CHBW within a wider BS bandwidth, the said UE compliant with selected OOBE requirements applicable for the wider BS bandwidth (the cell bandwidth). (Ericsson, Nokia)

- WF

  - Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach

Issue 1-2-3: whether extended UE CBW edge could exceed the BS CBW edge

- Proposals

  - Proposal 1: The extended UE CBW should not exceed the BS CBW. (Samsung, Sony, vivo, MTK, LGE)

  - Proposal 2: One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge. (ZTE)

- WF

  - proposal 1 is agreeable for singe CC case, FFS multi-CC/BS transmitted BW case

Issue 1-2-4: Whether and where to use IBE in the larger BS CBW

- WF

  - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW

Issue 1-2-5: Boundary to apply ACLR and SEM

- Proposals

  - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. (Samsung, Sony, ZTE, vivo, LGE)

  - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth. (Nokia, Ericsson)

- WF

  - FFS in next meeting

Issue 1-2-6: Boundary to apply SE

- WF

  - [SE applicability depends on the extended CBW]

Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE

- Proposals

  - Option 1: The integral region and the boundary of OOBE should be based on UE CBW. (Samsung, vivo, ZTE, China Telecom)

  - Option 2: The integral region and boundary of OOBE is based on extended UE CBW. (Sony)

- WF

  - FFS in next meeting

Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW

- Proposals

  - Proposal 1: (Apple)

    - If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel.

    - If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third.

  - Proposal 2: (Qualcomm)

    - If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW.

    - If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW.

  - Proposal 3: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point. (vivo)

  - Proposal 4: Consider extended UE CBW approach to improve MPR starting from 1.5*CBW (OPPO)

  - Proposal 5: (Huawei)

    - For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication.

    - Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency.

      - For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK

- WF

  - FFS with consideration of above proposals in next meeting

Issue 1-2-9: Whether and how to consider the Asymmetrical extended CBW approach

- WF

  - Asymmetrical extended CBW approach can be considered, FFS the details

Issue 1-2-11: Signaling aspects

- Proposals

  - Proposal 1: no changes of signalling are specified for the feature ‘narrower UE channel BW within wider BS bandwidth’ except possibly specification of a capability bit to indicate support of the feature. (Ericsson)

  - Proposal 2: Need to consider capability for extended UE CBW procedure. (LGE)

  - Proposal 3: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability. (Sony)

  - Proposal 4: The frequency intervals for the UE CBW and the BS CBW should be judged by the NW and the indication of MPR reduction could be directly sent to the UE when the corresponding conditions are met. (vivo)

  - Proposal 5: Establish a start point and continue to details for BS indication with the deepen in MPR reduction mechanism. (CTC)

- WF

  - To discuss the signalling aspects after sufficient evaluation of power boosting and/or MPR reduction in terms of relaxed requirements.

Topic #2: MPR applicability for FR1 intra-band UL CA

Sub-topic 2-1: Intra-band contiguous UL CA

Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell

- Proposals

  - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung)

  - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE)

    - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

    - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

    - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm).

        - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4

        - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4

        - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4

- WF

  - FFS in next meeting

Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE

- Proposals

  - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung)

  - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei)

- WF

  - FFS in next meeting

Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell

- Proposals

  - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks)

  - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo)

  - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson)

- WF

  - FFS in next meeting

Issue 2-1-4: Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell

- Proposals

  - Proposal 1: Add the following description into clause 6.2A.2.1:

    - “For intra-band contiguous carrier aggregation with single CC with activated cell, MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.”

- WF

  - FFS the details of draft CR

    - Sub-topic 2-2: Intra-band non-contiguous CA

Issue 2-2-1: General considerations

- Agreement

  - The work scope for FR1 NC CA should be limited to only study MPR applicability.

Issue 2-2-2: Applicable MPR for FR1 intra-band non-contiguous UL CA

- Proposals

  - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. (Qualcomm, ZTE, Huawei)

  - Proposal 2: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC. (Skyworks)

  - Proposal 3: for non-contiguous UL CA configurations with sub-blocks consisting of one cell and supported by dual PA architecture, the non-CA (single CC) MPR applies for one cell active among the configured uplink serving cells, the other cell deactivated. (Ericsson)

  - Proposal 4: for non-contiguous UL CA configurations with sub-blocks consisting of one cell not supported by a dual PA architecture, applicability of the non-CA (single CC) MPR for one active cell among the configured uplink serving cells, the other cell deactivated, is subject to UE capability, e.g. indication of [mpr-singleCC-activated-FR1] for the band combination. (Ericsson)

- WF

  - FFS in next meeting

Topic #3: MPR applicability for FR2

Sub-topic 3-1: Scope and cases considered for FR2 MPR enhancements

Issue 3-1-1: Whether new CA MPR for CABW < 400MHz should be considered in the WI

- Agreement in main session:

  - Hold on discussions on whether new MPR requirement is defined for CABW < 400MHz unless the WID can be updated accordingly.

  - Only MPR applicability needs to be discussed in this WID, and defining new MPR requirement is out of scope.

Issue 3-1-2: whether FR2 MPR enhancement for the single UL case is also applicable for intra-band DL non-contiguous CA with single UL

- Agreement in AH:

  - FR2 MPR enhancement for the single UL case is not applicable for intra-band DL non-contiguous CA with single UL unless the WID is revised

Issue 3-1-3: Power classes considered for FR2 MPR enhancement

- Agreement in main session:

  - All FR2 power classes could be considered for the MPR enhancement

Issue 3-1-4: sub-FR2 frequency ranges

- Agreement in main session:

  - MPR reduction applies to both FR2-1 and FR2-2.

    - Sub-topic 3-2: Applicable MPR

Issue 3-2-1:  Applicable MPR for FR2 single carrier UL with DL intra band CA

- Proposals

  - Proposal 1: For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier. (Huawei)

- WF

  - FFS in next meeting.

Issue 3-2-2:  Applicable MPR for FR2 UL CA with DL intra band CA

- WF

  - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW)

    - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused

    - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity

    - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2

    - Sub-topic 3-3: UE capability

Issue 3-3-1:  Whether an optional UE capability is considered

- Proposals

  - Proposal 1: no need to introduce a new UE capability for such MPR improvement as Network does not need to enable the MPR improvement originating from the application of MPR requirements. (CATT)

  - Proposal 2: The new capability shall be defined as optional and per UE, and only applicable to FR2. In addition, it could be release independent. (ZTE)

  - Proposal 3: RAN4 to consider as an optional capability. (Qualcomm)

  - Proposal 4: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA. (Huawei)

- WF

  - FFS in next meeting.

Issue 3-3-2:  Whether UE capability, if needed, is agnostic to specific LO implementations

- Proposals

  - Proposal 1: Consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability. (ZTE)

  - Proposal 2: Capability should not be dependent on a specific implementation. (NTT DOCOMO)

- WF

  - FFS in next meeting

Issue 3-3-3:  Proposal for CC activation-based MPR improvement

- Proposals

  - Proposal 1: FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration that are also enabled for UL, provided the activated CCs with UL comprise a contiguous block. (Qualcomm)

- Recommended WF

  - FFS in next meeting