3GPP RAN WG4 Meeting #112						R4-2414447

Maastricht, NL, August 19 –August 23, 2024

Title: 	Alignment of model training and data set parameters for AI/ML-based CSI compression

Source: 	Nokia

Agenda item:	8.17.4

Document for:	Approval

# Introduction

This document captures the discussion on the AI/M model training and dataset parameters for AI/ML-based CSI compression for RAN4 study.

When there are differences to the baseline/reference assumptions listed below, they are not prohibited but need to be clarified in the contributions.

# Discussion

| Parameter/Configurations | Options/Values | Company comments |
| --- | --- | --- |
| Channel/data-set related |  |  |
| UE antenna element assumptions | - Option 1: Omni-directional antenna elements (Ericsson) - Option 2: Directional antenna elements - Option 3: Other | [Apple] We provided 2 simulation results: Directional (both horizontal/vertical) Omni (both horizontal/vertical) [QC] Option 1 (both horizontal and vertical) [Intel] Option 1 [OPPO]:Option1 [CMCC] option 1 |
|  | Configuration for alignment: - Ideal omni-directional antenna elements following RAN1 assumptions |  |
| Sub-band channel averaging | - [Nokia, QC, Apple, vivo, Ericsson,OPPO] Average of covariance channel matrix per RBs in the sub-band before SVD - [ZTE]: Per sub-band - [CMCC]: Average of raw channel matrix per RBs in the sub-band before SVD | CATT: Prefer not to do the averaging before SVD. |
|  | Configuration for alignment: - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected |  |
| Any other pre-processing | - To be listed if used - [QC, Ericsson] Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - [OPPO] Normalize the phase of all entries of the eigenvector based on the phase of the last entry of the eigenvector - [Apple]: Normalize the phase of all entries of the eigenvector based on the phase of that entry that has the maximum amplitude averaged over the sub-bands | [Intel] prefer no other pre-processing |
|  | Configuration for alignment: - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1. |  |
| Data collection assumptions | - [Nokia, CATT, ZTE, Ericsson, CMCC] Each channel realization is collected from independent UE drop - Multiple channel realization can be collected from the same UE - Other - [QC] Multiple channels realizations are collected from multiple UEs and multiple drops of same UE - [Apple, Intel, OPPO]: Multiple drops. Each drop collect data from multiple UEs. Additionally collect samples from channel realization from each UE spaced at de-correlated interval. |  |
|  | Configuration for alignment: - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs. |  |
| Training dataset size | - [Nokia]: 630K - [QC]: 450K - [Samsung]: 25K - [R&S]: 614,440 - [MTK]: 700K - [Ericsson]: 800K - [Intel]: 200K - [CATT]: 102,600 - [ZTE]: 600k - [Apple]: 600K - [vivo]: 243000 - [OPPO]：600k - [CMCC]: 100K |  |
|  | Configuration for alignment: - Aim to use the dataset around 600K. - At least 100K sample to be used. |  |
| Dataset quantization | - Float32 |  |
| Other data-set characteristics/statistics | - Companies to propose |  |
|  | - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII. |  |
| Model-related |  |  |
| Quantization of encoder output | - [Nokia]: Non-trainable quantization (bypassed during training) - [QC, CMCC] Quantization aware training - [Apple]: Quantization in forward and approximation in backward to enable learning. (avoid zero gradient of round function) - [vivo] VQ with quantization aware training - [Intel] Quantization aware training. Scalar quantization with non-trainable quantization grid. - [Ericsson] 2 bit scalar quantization, quantizer aware training | CATT: Consider to align the activation function and quantization method. |
|  | - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization |  |
| Normalization before quantization in the encoder | - [Nokia, Ericsson]: Normalization with tanh function after FC layer - [CATT, ZTE, vivo, CMCC]: Sigmoid function used after FC layer - [QC] Normalization not done due to the use of quantization aware training - [Apple]: Normalization with tanh function after FC layer. We don’t apply inverse of that in de-quantization - [Intel] Sigmoid after FC layer. Inverse sigmoid after de-quantization stage in decoder. |  |
|  | Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional. |  |
| Use and configuration of drop-out | - [Nokia]: Use of on single drop-out layer after FC layer in the encoder and decoder, Drop-out rate = 0.4. - [CATT, QC, ZTE, Apple, vivo, Intel, Ericsson, CMCC]: No drop-out used. |  |
|  | Configuration for alignment: - Not to use drop-out layers |  |
| Training loss function, use of regularization | - [Nokia]: - SGCS with L2 regularization with weight decay, λ = 10-2 - [QC]: SGCS + additional terms for quantization aware training - [Apple, CATT, CMCC]: 1- SGCS - [R&S, MTK, Intel]: SGCS - [Ericsson]: NMSE - [ZTE]: MSE - [vivo]: 1-SGCS plus VQ loss for encoder output (MSE between original and quantized encoder output) - [Intel]: SGCS, no L1/L2 regularization |  |
|  | Configuration for alignment: - Use (1-SGCS) as loss function as reference |  |
| Optimizer | - [Samsung, QCM, Apple, MTK, Intel, CATT, Ericsson, CMCC]: Adam - [Nokia]: RMSProp (Root Mean Square Propagation) - [ZTE, vivo]: AdamW |  |
|  | Configuration for alignment: - Use Adam |  |
| Learning rate | - [Nokia]: - [QC, R&S, CATT, Ericsson]: 10^-4 - [R&S]: Learning rate adaptation: Reduce on plateau by factor 0.5 with patience of 50 epochs. - [Apple]: 0.0025 - [Samsung, Intel, MTK, CMCC]: 0.001 - [ZTE]:1.5e-3 - [vivo]: Warm-up to initial lr 1e-3 and then linearly decay to final lr 8e-4, update after each batch |  |
|  | Configuration for alignment: - Fixed learning rate of 10^-4 |  |
| Batch size | - [Nokia]:1024 - [QC, MTK, CMCC]: 128 - [Apple]: 256 - [Samsung]: 200 - [R&S, CATT, vivo, Ericsson]: 512 - [Ericsson]: 32 - [Intel]: 10 - [ZTE]:400 |  |
|  | Configuration for alignment: - 256 |  |
| (Max) Number of epochs | - [QC]: 150 - [Apple, vivo, Intel]: 100 - [R&S, Nokia, CATT]: 200 - [MTK, ZTE]: 300 - [CMCC]: 500 |  |
|  | Configuration for alignment: - 150 epochs |  |
| Stopping criteria | - [Nokia, Apple, CATT, QC, ZTE, Intel, CMCC]: No change in metric over a given number of epochs - Validation patience: [Intel, Nokia] 10, [QC] 50 - + Ericsson with constraint on max Epochs - [vivo] Stop when reaching the configured number of epochs |  |
|  | Configuration for alignment: - No improvement over 25 epochs. |  |
| Weight initialization | - [Apple, CATT, QC, vivo, Intel, CMCC]: Default - Ericsson: Variance scaling |  |
|  | Configuration for alignment: - Default |  |
| Kernel Parameters: - Dilation - Padding Type The value of groups (VG) | - [CMCC, Samsung, Vivo, CATT]: Dilation:1, Padding type=0, VG= 1 - [QC, ZTE, Apple, Intel]: Default |  |
|  | Configuration for alignment: - Default |  |