3GPP TSG-RAN WG4 Meeting #112	R4-2414452

Maastricht, Netherlands, August 19 – August 23, 2024

Title: 	WF on UE RF requirements for NTN HPUE

Agenda Item:	8.8.5

Source: 	Samsung

Document for:	Approval

1. NTN HPUE Co-existence study

1.1 General starting point

General starting point for co-ex assumptions and scenarios

Agreement:

- Agree to use TR 38.863 and WF R4-2217473 for NR-NTN and IoT-NTN HPUE coex study assumptions and scenarios as starting point.

  - The detailed modifications to these references will be discussed and agreed in case-by-case manner.

1.2 Scenarios for coexistence study

Issue 2-2-1: NTN scenarios for co-ex study

Agreement:

- For the scenario, consider the scenarios in TR 38.863 as a baseline

  - Prioritize GEO and LEO1200 for co-existence evaluation

    - LEO600 is not precluded for the requirements and the conclusion for LEO1200 can be applied to LEO600

Issue 2-2-2: TN scenarios for co-ex study

- Agreement: Prioritize Urban Macro for Scenario 4 and Rural Macro for Scenario 5

Issue 2-2-3: Co-ex scenario # to be studied

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |

Agreement:

- Consider both Scenario 4 and 5 for co-existence study

  - Prioritize scenario 4 since it is the worst case

- Do not consider Scenario 1, 2, 3 and 6

1.3 Deployment and layout

Issue 2-3-1 NTN and TN network isolation distance

Agreement:

- Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex.

  - Use the isolation distance 1.5km as the starting point

  - Other values for isolation distance are not precluded

Issue 2-3-2 TN network topology

- Agreement: Reuse existing method from TR 38.863 as baseline, and encourage companies to further evaluate any performance impact with and w/o wrap-around functionality.

Issue 2-3-3 NTN UE dropping methods

- Agreement: Random dropping on circle, and to use uniform distribution for random.

Issue 2-3-4 Observed TN cells for Scenario 4 evaluation in Table 6.2.1.1-1 in TR 38.863

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | TN with NTN satellite | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the TN sectors which have NTN UE(s) at their edges. Option 1 is the baseline and it is not precluded companies can follow Option 2 to bring results |
| 5 | TN with NTN satellite | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which has the NTN UE(s) at its edge |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: NTN UEs dropped at the edge of TN clusters | TN clusters randomly placed in this NTN beam | All active TN clusters which has the NTN UE(s) at its edge. |

Moderator note: The table from TR 38.863 v18.2.0 is provided here to help discussion.

- Agreement: Option 1 as baseline assumption, other options not precluded.

Issue 2-3-5 NTN HPUE ratio

- Agreement

  - Assuming all NR-NTN/IoT-NTN UEs are HPUE, i.e. 100% HPUE ratio as baseline.

  - FFS other ratio values

Issue 2-3-6: NR and NR-NTN channel bandwidth

- Agreement

  - Consider 20MHz system bandwidth for NR-NTN and NR in co-existence study.

1.4 Detailed parameters modifications

Issue 2-4-1: NR-NTN SAN parameter set

- Agreement: To use Set-1 as NTN SAN parameters.

Issue 2-4-2: IoT-NTN UL UE number and SCS

- Agreement

  - Consider 9 UEs for 15kHz, 18 and 36UEs for 3.75kHz single tone as starting point

  - FFS whether to down scope.

Issue 2-4-3: NR-NTN UE parameter for PC2

- Agreement

  - Baseline: Use NF as 9dB, and Tx power 26/29/31 dBm for PC2/1.5/1 NTN HPUE

  - FFS option of 7dB NF for PC2 and LEO600.

Issue 2-4-4: NTN HPUE uplink power control

- Agreement

  - To update the existing uplink power control parameters considering different power classes and bandwidth (Samsung, Qualcomm, Huawei)

    - Update Pmax to 26/29/31 for PC2/1.5/1;

    - Update Rmin to -66/-69/-71 assuming Pmin as -40;

    - Update BW to corresponding transmission BW.

  - Agree on the following detailed parameters

where:

-	Pmax = 26dBm for PC2, 29 dBm for PC1.5, 31 dBm for PC1.

-	Rmin = -66 dB for PC2, -69 dB for PC1.5, -71 dB, assuming -40dBm minimum output power,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ),

  - Where:

  - SNRtarget and BW

For NR-NTN HPUE:

    - Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only)

    - Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption

  - NF is the SAN noise figure, i.e. 4.3dB.

  - 10log10(kT) = -174dBm/Hz.

  - γ = 1 For uplink scenario.

1.5 Other considerations

Issue 2-5-1: Power classes for co-ex study

- Agreement

  - All power classes listed in WID will be considered for co-existence study.

Issue 2-5-2: Handheld and non-handheld type

Agreement:

- To use TR 38.863 UE characteristics as starting point for NTN HPUE co-ex studies.

1.6 Others

Issue 2-6-1: Preliminary results and observations

- Agreement

  - Please take submitted results and analysis for information

  - Encourage companies to follow work plan and submit co-ex results starting from next meeting.

Issue 2-6-2: Running document to capture assumptions

- Agreement

  - To revise the R4-2412557 in this meeting to capture agreements for information.

2. NTN HPUE UE RF – Tx requirements

2.1 General considerations

Issue 3-1-1: General considerations

- Agreement

  - The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements.

Issue 3-1-2: Consideration of band(s)

Agreement:

- Use 2GHz for the co-existence study

- WI can be completed when the band specific requirements for one pair of {n256,256}and {n255, 255} are completed in this WI

  - The contributions on the other existing Rel-18 FR1-NTN bands for specifying common requirements are allowed.

- Specify the band specific requirements for other potential NTN bands in the separate spectrum WI

Issue 3-1-3: Consideration of power classes

- Agreement

  - All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed

Issue 3-1-4: Consideration of regulation(s)

- Agreement

  - Applicable regulations should be taken into account for requirements discussions.

  - If any identified corrections to be made for PC3, those should be considered in another work item, it is not in the current scope of this WI.

Issue 3-1-5: Consideration of HPUE Architecture

- Agreement:

  - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements.

  - For PC2 IoT NTN, at least 1Tx shall be considered

  - For other PCs (both IoT NTN and NR NTN), FFS

2.2 Feasibility

Issue 3-2-1: NR-NTN HPUE feasibility for different power classes

Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes

Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes

- Agreement of Issue 3-2-1 to Issue 3-2-3:

  - For PC2, it is feasible to support both handheld and non-handheld devices.

  - FFS other power classes for applicable device type.

2.3 Detailed requirements

Issue 3-3-1: Maximum output power

- Agreement

  - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN.

Issue 3-3-2: MPR

- Agreement:

  - FFS the MPR requirements. Following potential factors to be further discussed:

    - ACLR from co-existence study

    - Current TN MPR requirements

    - MPR simulation assumptions

    - Other factors not precluded if identified.

Issue 3-3-3: A-MPR

- Agreement

  - FFS A-MPR requirements. Following potential factors to be further discussed:

    - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations

    - A-MPR simulation assumption including ACLR from co-existence study

    - Applicable bands

    - Regional requirements for A-MPR

    - Other factors

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

Issue 3-3-5: SEM

- Agreement:

  - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

  - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

Issue 3-3-6: ACLR

- Agreement:

  - The ACLR requirements will depend on co-existence study outcomes.

3. NTN HPUE UE RF – Rx requirements

Issue 4-1: General considerations

- Agreement

  - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

Issue 4-2: RSD for NR-NTN

- Agreement

  - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE.

    - Further discuss the following aspects:

      - Use similar way in TN spec by introducing a new table of RSD compared to the PC3

      - Whether default Tx-Rx frequency separation is used for test point

Issue 4-3: RSD for IoT-NTN

- Agreement:

  - For NB-Iot based Iot-NTN, no RSD needed for HPUE

  - For eMTC based Iot-NTN with half duplex, no RSD needed for HPUE

  - For eMTC based Iot-NTN with full duplex, further discuss RSD impact