3GPP TSG-RAN WG4 Meeting #112	R4-2411006

Maastricht, Netherlands, 19 – 23 August, 2024

3GPP TSG RAN WG1 #117			R1-2405719

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Title:	Reply LS on Reference Point for SSB-TimeOffset

Response to: 	R2-2403771

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact Person:

Name:	   Chunxuan Ye

Email Address:  chunxuan_ye@apple.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# Overall description

RAN1 thanks RAN2 for the LS on reference point for SSB-TimeOffset.

RAN1 would like to provide answer to the following question:

In RAN2#125bis meeting, RAN2 is considering adopting the gNB as the reference point of ssb-TimeOffset. RAN2 would like to check with RAN4 and RAN1 whether this would be acceptable. To RAN4, RAN1: ACTION: RAN2 respectfully asks RAN4 and RAN1 to take the above into consideration and come back if any issues are determined.

Reply to Question:

- If adopting gNB as the reference point of ssb-TimeOffset, then there is no common understanding in RAN1 on whether the signalling of ssb-TimeOffset can help the UE reduce the amount of blind search for the SSB of the target satellite.

  - This implies RAN1 has no common understanding on whether it is feasible to set gNB as the reference point of ssb-TimeOffset.

- If adopting the UL time synchronization reference point as the reference point of ssb-TimeOffset, then the amount of blind search for the SSB of the target satellite depends on the granularity of the ssb-TimeOffset.

  - This implies there is no technical issue from RAN1 perspective by setting UL time synchronization reference point as the reference point of ssb-TimeOffset.

# Actions

To RAN WG2

ACTION:  RAN1 respectfully asks RAN2 to take above information into account for their future works.

# Dates of Next TSG-RAN WG1

TSG RAN WG1 Meeting #118   			   19 – 23 August, 2024  		  Maastricht, Netherland

TSG RAN WG1 Meeting #118bis         14 – 18 October, 2024     	TBD, China