3GPP TSG-RAN WG4 Meeting #112	R4-2411007

Maastricht, Netherlands, 19 – 23 August, 2024

3GPP TSG RAN WG 1 Meeting #117		R1-2405736

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Title:	LS on UE assistance information

Response to:

Release:	Rel-19

Work Item:	NR XR Phase 3 (NR_XR_Ph3-Core)

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact person:	Margarita Gapeyenko

margarita.gapeyenko (at) Nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN1 discussed UE assistance information related to measurements occasions and related to channel conditions, traffic, UE mobility. The following agreement and conclusions were agreed:

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period.

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

# 2	Actions

To RAN4

ACTION:  RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.

# 3	Dates of next TSG RAN WG 1 meetings

TSG-RAN WG1 Meeting #118		19  -  23 August 2024 		   	 	    Maastricht, NL

TSG-RAN WG1 Meeting #118-bis		14  -  18 October 2024         			China (TBC), CN