3GPP TSG-RAN WG4 Meeting # 112bis	                                                R4-2414931

Hefei, China, Oct. 14 – Oct. 18, 2024

Agenda item:	6.18.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements of AI mobility

Document for:	Discussion

# Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

In this contribution, we would like to discuss RRM requirements of AIML mobility.

# Discussion

In the SID [1], TSG RAN identified the following use-cases to be considered.

- Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model)

- HO failure/RLF prediction (UE sided model)

- Measurement events prediction (UE sided model)

RAN2 had discussed AI/ML mobility for three meetings and the focus was mostly on RRM measurement prediction use-case and common evaluation assumptions for it. At the RAN2#127 meeting, the discussion of the RLF use-case has also started. However, no major contribution on RLF is planned at RAN2#127bis as shown in the following RAN2 agenda. On the other hand, at RAN2#127bis, the measurement even prediction use-case will be discussed, as it assumed by the agenda below. We suggest RAN4 to focus on RRM measurement prediction use case this meeting and further discuss other use cases based on RAN2 progress.

8.3.3 Measurement event predictions Contributions should focus on measurement event prediction use cases/scenarios to focus during the study, simulation assumptions and relevant performance metrics/KPIs to evaluate 8.3.4 RLF/HO failure prediction No contributions expected for this meeting Contributions should focus on discussing RLF specific methodology and simulation assumptions (addressing the differences or additional aspects from RRM predicution asssumptions). Relevant metrics and assumptions not covered by email discussion No evaluations/simulation results expected for this meeting

Proposal 1: Focus on RRM measurement prediction use case this meeting and further discuss other use cases based on RAN2 progress.

In the next, we would like to provide our views on RAN2 agreed scenarios for RRM measurement prediction.

Table 5.2.1-1 (in TR 38.744) prioritization of evaluation scenarios

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

The scenarios to define RRM requirements for

As shown above, the scenarios agreed in RAN2 are of different priority. RAN4 can discuss which scenarios to define RRM requirements for. In general, we think RAN4 should follow the priority suggested in RAN2. For scenarios 1 and 5, as they are of low priority and the methodologies have not been concluded in RAN2. RAN4 can put them as low priority too. For scenario 6 which is intra-frequency intra-cell spatial domain prediction and evaluated by measuring a sub-set of configured SSB, the target study goal is to reduce measurement efforts by using predicted measurements (1st goal). But from the point of measurement, in each SSB period, UE can measure all the Tx beams with a fixed Rx beam. The reduced measurement efforts by measuring less Tx beams are quite minor. Although this scenario is of middle priority in RAN2, we suggest not to discuss this scenario in RAN4.

Proposal 2: Focus on scenarios 2,3,4 for RRM measurement prediction use case at first.

Metrics used to define the model performance

For AI/ML BM, there were a lot of discussions on the metrics used to define the model performance. Similar as AI/ML BM, the metrics should be discussed for AI/ML mobility too. RAN2 agreed to use average RSRP difference as prediction accuracy metric for RRM measurement prediction.

RAN2#126 Agreements Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases. average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged.

For scenarios 2 and 4, i.e., intra-frequency intra-cell temporal prediction, the final output will be cell-level measurement result of a single cell. Even for sub-use case 1, the output of the model may be beam-level measurement results for multiple beams, but it is the generated cell-level measurement result that matters. So the only requirement to define is the absolute accuracy of predicted/generated L3 cell level RSRP.

For information Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

Proposal 3: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated L3 cell level RSRP.

For scenario 3, i.e., FR1 to FR1 inter-frequency inter-cell prediction (frequency domain), now RAN2 focuses on the prediction on single cell on a single inter-frequency layer. In our understanding, the benefit of scenario 3 comes from allowing to use a much longer MGRP or skipping some gap occasions for inter-frequency measurements. As well known in RAN4, if UE only predicts the measurement results on a single cell, it is not possible to reduce MGRP. Because one predictable cell does not mean all the other cells in the same frequency are also predictable (or can be skipped). Moreover, MG is configured to monitor not just one inter-frequency layers. Whether the MGRP can be extended, or some gap occasions can be skipped should be a decision with all frequency layers taken into consideration. We believe the scenario is more valuable to predict the measurement results for multiple cells and cells on multiple frequency layers. So the output will be cell-level RSRP of multiple cells. There will be absolute and relative accuracy requirements for discussion. In our understanding, the requirements to define shall meet the needs. If the predict measurement results are used directly for HO decision, both absolute and relative accuracy of the predict results are important.

Proposal 4: For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated L3 cell level RSRP.

Similar as AI/ML BM, when defining the prediction accuracy requirements, the impact of measurement error should be considered. The methodology shall be similar too. As this issue is still in discussion in AI/ML BM, we can wait for more progress to avoid duplicated discussion.

Proposal 5: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility.

Other RRM impacts

UE measurement and/or report behavior will be different from legacy for all the scenarios 2,3,4. RAN4 shall study whether there is any impact on measurement delay and report delay requirements. Regarding the impact on report requirements, there will be similar issue for AI/ML BM. We suggest to wait for more progress in AI/ML BM.

Proposal 6: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Next, we will focus on the impact on measurement delay requirements.

Scenario 2, FR1 to FR1 intra-frequency intra-cell temporal domain case B prediction, is to predict a sub set of measurement instances in temporal domain of the same cell. This scenario is similar as BM case 2. The impact on measurement delay would take BM case 2 as a basis.

Proposal 7: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Scenario 3, FR1 to FR1 inter-frequency inter-cell (frequency domain), in our understanding, this scenario includes “prediction on an inter-frequency layer based on measurement on an intra-frequency layer” and also “prediction on an inter-frequency layer based on measurement on a different inter-frequency layer”. As discussed in previous part, the scenario is more valuable to predict the measurement on multiple frequency layers. In our understanding, MG still needs to be configured at least for model performance monitoring. To maintain only the model performance monitoring activity on one frequency layer, it seems possible to extend the time duration between 2 visits to that frequency layer. However if there are still other inter-frequency layers to be monitored or the MGRP cannot be adjusted in time, RAN4 needs to discuss the gap sharing mechanism among different inter-frequency layers both for measurement and monitoring. There would be some impact on measurement delay requirements.

Proposal 8: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for inter-frequency layers which require normal measurement and inter-frequency layers which require model monitoring.

Scenario 4, FR2 to FR2 intra-frequency intra-cell temporal domain case A, is to predict measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell without reducing any measurement in observation window. There is no impact on UE measurement behavior, so there is no impact on the measurement delay.

Proposal 9: Scenario 4 has no impact on measurement delay requirements.

# Conclusion

In this contribution, we provide our views for AI mobility. Proposals are summarized as follows.

Proposal 1: Focus on RRM measurement prediction use case this meeting and further discuss other use cases based on RAN2 progress.

Proposal 2: Focus on scenarios 2,3,4 for RRM measurement prediction use case at first.

Proposal 3: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated L3 cell level RSRP.

Proposal 4: For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated L3 cell level RSRP.

Proposal 5: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility.

Proposal 6: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Proposal 7: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Proposal 8: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for inter-frequency layers which require normal measurement and inter-frequency layers which require model monitoring.

Proposal 9: Scenario 4 has no impact on measurement delay requirements.

# Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.