3GPP TSG-RAN WG4 Meeting # 112bis	                                                R4-2414932

Hefei, China, Oct. 14 – Oct. 18, 2024

Agenda item:	6.18.3

Source:	MediaTek Inc.

Title:	Discussion on testability and interoperability of AI mobility

Document for:	Discussion

# Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

In this contribution, we would like to discuss the testability and interoperability of AIML mobility.

# Discussion

In the SID [1], TSG RAN identified the following use-cases to be considered and only one-sided model is in the scope.

- Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model)

- HO failure/RLF prediction (UE sided model)

- Measurement events prediction (UE sided model)

Regarding “testability”, the issues faced are similar for all the three use cases, but there are some differences between FR1 and FR2 tests due to OTA test is mandatory in FR2. As far as we know, both FR1 and FR2 are under evaluation in RAN2. As guided in the SID, duplicated study should be avoided from “AI/ML for NR air interface” in which testability issues are thoroughly discussed for FR2. In our understanding, most of the agreements there will be applicable here. In this SI, we can focus on the testability issues in FR1 and check if there is any more testability issue for FR2 after RAN4 concluding the testability issues in “AI/ML for NR air interface”.

Proposal 1: Focus on the testability issues in FR1. Check if there is any more testability issue for FR2 AI/ML mobility after RAN4 concluding the testability issues in “AI/ML for NR air interface”.

In FR1, conductive test method is used in legacy. We don’t see any motivation to use OTA test for AI/ML mobility test in FR1. We propose to use conductive test method for AI/ML mobility test. As conductive test is already widely used in legacy, and TE can adjust the transmit power to simulate the RSRP changes of the measured and predicted cell(s).

In AI/ML BM, there are lots of discussions on how to get the ground truth as beamforming gain is unavailable. In FR1, using conductive test, ground truth (e.g., RSRP of each cell) is available at TE as legacy.

Proposal 2: Use conductive test in FR1 for AI/ML mobility.

Observation 1: Ground truth is available at TE using conductive test in FR1 for AI/ML mobility.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Proposal 1: Focus on the testability issues in FR1. Check if there is any more testability issue for FR2 AI/ML mobility after RAN4 concluding the testability issues in “AI/ML for NR air interface”.

Proposal 2: Use conductive test in FR1 for AI/ML mobility.

Observation 1: Ground truth is available at TE using conductive test in FR1 for AI/ML mobility.

# Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.