3GPP TSG-RAN WG4 Meeting # 112bis	R4-2414952

# Hefei, China, 14th - 18th, Oct 2024

Source: 	Samsung

Title: 	Discussion on UE and NW behavior for non-collocated deployment

Agenda Item:	6.5.2.2

Document for:	Discussion

- Introduction

In RAN#103, a new WID on Non-collocated deployment is approved [1] which focus on specifying UE RF/RRM requirements for Type-4a/b UE capability and clarifying the UE behaviour for Type-4a/b supported UE.

Discussion was organized in RAN4#112 and the agreements were captured in [2] and the Chairman notes.

(Note that Type-1/2/3/4 in Rel-18 discussion are intended for different UE capabilities, rather than UE type.)

- 2. Background

Standardization on non-collocated deployment is motivated by the fact the spectrum on C band in Japan/Korea was allocated at different time, hence the legacy BS usually only supports part of the C-band spectrum. There are two choices when it comes to the new BS, either developed based on the whole C-band spectrum holding of an operator or based on the spectrum allocated later to the operator, the latter makes it non-collocated deployment along with a legacy BS to support non-collocated NR-CA or EN-DC.

Rel-15 Inter-band EN-DC with overlapping DL bands

The discussion on the necessity of non-collocated inter-band EN-DC with overlapping DL bands deployment can date back to Rel-15, during which operators proposed the possible non-collocated scenario, while due to time limitation RAN4 concluded the simple assumption, i.e., “only co-located deployment” is allowed. It should be noted that due to different band numbering system, DC_42_n77 is inter-band EN-DC with overlapping DL bands but essentially equivalent to intra-band EN-DC. Accordingly, the following restriction is provided in Rel-15 RAN4 requirement, which forbid non-collocated deployment for inter-band EN-DC with overlapping DL bands: (1) MRTD ＜3us; (2) Power imbalance between downlink carriers is no larger than 6dB.

Rel-15 Intra-band NR-CA

Similarly for intra-band NR-CA, only co-located deployment is assumed, and the restriction is provided in Rel-15 RAN4 requirement, which forbid non-collocated deployment for intra-band NR-CA, i.e., MRTD＜3us.

Rel-16/17 Inter-band EN-DC with overlapping DL bands for Non-collocated Deployment

In Rel-16/17, the requirement on Type-2 UE (i.e., UE supporting interBandMRDC-WithOverlapDL-Bands-r16, which indicates 2 layer/2 Rx Chain per CC supported for non-collocated EN-DC) has been specified in terms of RF and RRM requirements. Specifically, 25dB power imbalance and 1dB REFSENS relaxation for Type-2 UE in-band blocking requirements of inter-band EN-DC with overlapping DL bands have been specified in [2] in RAN4#102e meeting.

Rel-18 Inter-band EN-DC with overlapping DL bands for Non-collocated Deployment

Though the RF/RRM requirements have already been specified for Type-2 Inter-band EN-DC with overlapping DL bands from Rel-16, one issue was identified during the Rel-18 discussion for Type-2 intra-band NR-CA, which is deemed as a common issue for both Type-2 EN-DC and Type-2 NR-CA, i.e., NW may not be able to control UE to work under Type-1 mode or Type-2 mode, especially when maxMIMO-Layers is less than 2

Consequently, a new NW control signaling (nonCollocatedTypeMRDC) and a new UE indication capability (requirementTypeIndication-r18) are introduced in RAN2 [3][4][5] to allow NW configure UE work under Type-1 mode or Type-2 mode for Rel-18 and onwards(excerpted as below). Note that Type-2 is deemed as the default mode if UE indicate the support of Type-2, which is to align with previous releases.

In addition, the Rel-18 discussion focus on 2CC non-contiguous EN-DC such as DC_42A_77A.

(TS 38.331)

Note that there is an error in TS38.331 (yellow marked), should be “intra-band”.

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

(TS 38.306)

Note that this requirementTypeIndication-r18 is only applicable to EN-DC case but not NR-CA, and to our understanding “requirementTypeIndication-r18+ interBandMRDC-WithOverlapDL-Bands-r16= interBandMRDC-WithOverlapDL-Bands-r18”. Further, This IE is basically to inform NW “I am a Rel-18 UE and I can understand nonCollocatedTypeMRDC” thus NW can configure this UE.

| requirementTypeIndication-r18 Indicates whether the UE supports network control of requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16. The UE supports this feature shall also indicate support of interBandMRDC-WithOverlapDL-Bands-r16. | UE | No | No | FR1 only |
| --- | --- | --- | --- | --- |

Rel-18 Intra-band NR-CA for Non-collocated Deployment

Similarly as the discussion of non-collocated inter-band EN-DC with overlapping DL bands, 25dB power imbalance and 1dB REFSENS relaxation is defined, while a new UE capability(intraBandNR-CA-non-collocated-r18) and a new NW control signalling(nonCollocatedTypeNR-CA) are introduced[3][4][5] for Rel-18 and onwards to enable non-collocated intra-band NR-CA assuming 2 MIMO layer/2 Rx Chain per CC. In addition, the Rel-18 discussion focus on 2CC non-contiguous NR-CA such as n77(2A)/n78(2A).

(TS 38.331)

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15].

(TS 38.306)

| intraBandNR-CA-non-collocated-r18 Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 [2]. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. This capability is only supported for band n77/n78. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

- Discussion

- 3.1 New UE capability

The following is the intermediate agreement made online but not agreed in the end. It was argued that it does not make sense to only have Type-4a capability for EN-DC, but not for NR-CA. We understand that 4Rx is mandate per band rather than per CC according to current spec, however, Type-4a for NR-CA is not attractive in our opinion, as it is not justified for imbalanced maximum MIMO layer capability for different NR CCs.

Moreover, as described in the WID, “UE types are defined in R4-2217734”, note that here our understanding is “UE types” is intended for “UE capability types”. The following table is excerpted from R4-2217734. It is clearly stated that Type-4a is for EN-DC only, hence the extension for Type-4a to NR-CA is WID up-scoping thus belong to RAN-P level discussion.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

Observation 1: Type-4a for NR-CA is not attractive, as imbalanced maximum MIMO layer capability between two different NR CCs is not justified.

Observation 2: The extension for Type-4a to NR-CA is WID up-scoping thus belong to RAN-P level discussion.

Proposal 1：Do not consider Type-4a for NR-CA.

Then come back to UE capability discussion, the following is the final agreement of last meeting. Companies have proposed different feasible UE capability design [3-8] including leveraging legacy IE or fully relying on new IE. In the spirit of moving forward, companies agreed to have one dedicated capability for EN-DC and one for NR-CA which we think could be a more clean solution and easier to get converged.

One open issue is how to differentiate Type-4a and Type-4b, for EN-DC only aligned with Proposal 1. Given we already agreed to have two UE capabilities, we think we do not have to leverage legacy IE anymore, Type-4a and Type-4b can be differentiated by “0-1-absent” RAN2 signaling design, this is simpler in our view.

Proposal 2: For UE capability, we propose,

- Define one UE capability for NR CA and one UE capability for EN-DC

- For EN-DC, differentiate Type-4a and Type-4b, covering/achieving by new UE capability design (which means not leveraging legacy UE capability)

- For NR-CA, only have Type-4b

- Specific UE capabilities’ design up to RAN2

- 3.2 UE behavior

We discuss UE behavior from two aspects:

- UE initial configuration/behavior on collocated or non-collocated deployment, at the initial connection mode

- UE switching behavior between collocated and non-collocated deployment

- 3.2.1 UE switching behavior between Type-4 and Type1/Type-2/single carrier

Our thinking is illustrated in the following figure assuming UE is configured on Type-4 mode at this moment. The figure shows all the possible “switching path” from Type-4 mode to either Type-2 mode or Type-1 mode or single carrier operation. In our understanding, probably there is no need to argue which path is the most desirable one with maximum benefit, but rather focus on making everything possible and under NW full control.

(Please note that in Rel-18 design, it points to Type-2 when “nonCollocatedTypeMRDC” or “nonCollocatedTypeNR-CA” is “absent”, rather than when they are “present”.)

More explanation of above figure,

- From Type-4 to Type-2: As both Type-4 and Type-2 are non-collocated with same RF requirements and most likely same RRM requirements as well, there is no new NW signaling needed as the deployment scenario does not change, UE can just do MIMO layer re-config to become 2 layer per CC(Type-2). →No new signaling needed was agreed/confirmed in RAN4#112.

- From Type-4 to Type-1: This is from non-collocated to collocated scenario. Collocated can be either up to 4-layer/per CC or 8 layer/per CC or 2-layer per CC. We think 4 layer/CC can be the baseline for collocated Type-1 in this path, and if NW want UE to have 8-layer or 2-layer per CC, it can be achieved by further MIMO layer re-config. In our understanding new NW signaling is needed here, owing to the pre-condition/restriction of the usage of the Rel-18 NW signaling which is only present for a UE configured with maxMIMO-layers≤2. Therefore, the exiting NW IE (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) cannot be used to instruct the Type-4 to Type-1 switch because Type-4 can support up to 4 layer per CC. More details of the Rel-19 new signaling would be shared in next subclause. In addition, technically, this path maybe the least likely one, as there is no strong justification for a FWA to do switch between collocated and non-collocated. →To be further discussed in this meeting.

- From Type-4 to single carrier operation: Achieving by Scell release, and Pcell remains, which is a legacy approach. →No new signaling needed was agreed/confirmed in RAN4#112.

Proposal 3: New NW signaling needed between Type-4 and Type-1 switch. (more details of New NW signaling is present in Proposal 4)

- 3.2.2 At initial connection mode

We discuss the “switching behavior” in last clause, and the other case to be discussed is for the initial connection mode. How to configure the UE working under either collocated scenario or non-collocated scenario at the beginning.

In Rel-18, the definition of Rel-18 NW signaling is copied as below for reference. It is worth nothing that the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2 which is a pre-condition/restriction of the usage as mentioned in last subclause. And if maxMIMO-layers ＞2, it is collocated.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15].

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

Observation 3: In Rel-18, the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2. And if maxMIMO-layers＞2, it is collocated.

In Rel-19, we think same methodology can be followed in RAN2 NW signaling design. More specific,

1) New Rel-19 NW signaling is utilized to differentiate Type-4 and Type-1 with the condition/restriction it is only present when 2＜maxMIMO-layers ≤4

2) If maxMIMO-layers ＞4, it is collocated

Proposal 4: New Rel-19 NW signaling is needed to differentiate Type-4 and Type-1 when 2＜maxMIMO-layers ≤4.

Our initial design is something like the following, in a simple “present-absent” way.

For the initial connection,

1）maxMIMO-layers＞4, it is collocated

2）maxMIMO-layers=4, it could be either collocated (Type1) or non-collocated(Type-4), pending on whether new NW signaling is present

3）maxMIMO-layers=2, it could be either collocated (Type 1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

While during the offline discussion among companies before submission deadline, there are two good counter-examples provided by MTK, which demonstrates the above Rel-19 NW signaling design is not good enough. More specific, if Rel-19 NW signaling is designed in a simple “present-absent” way, we would face the following issues, and the Rel-18 argument would repeat.

- Rel-19 type 4 capable UE behavior operate under Rel-18 network

[Samsung] Assuming a Rel-19 UE indicates both Type-4 and Type-2, then Rel-18 NW only understand Type-2 and use maxMIMO-layers=4 to control UE work under collocated if following above design, but Rel-19 UE would see "new Rel-19 NW signaling =absent + maxMIMO-layers=4", then this UE misthink Non-collocated Type-4. Discrepancy between UE and gNB happens.

(Note in our understanding a UE has no idea of the NW is of which release, and also a NW has no idea of the UE is of which release)

- Only type 2 capable Rel-19 UE behavior under Rel-19 network.

[Samsung] Seems we can just follow Rel-18 design in this case? If Type-4 is not reported, it implies this UE does not understand new Rel-19 NW signaling. So for collocated, Rel-19 NW just use " maxMIMO-layers=4". If this is common understanding, seems there would be no issue. But this needs to be clarified.

To well resolve above issues, it is proposed to have the Rel-19 NW signaling design in the “absent, present with value=0, present with value=1” way:

Proposal 5: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling and Rel-18 NW signaling:

1）maxMIMO-layers ＞4, it is collocated

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, it could be either collocated(Type-1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

More specific, “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

- 4. Conclusion

(WI scope)

Observation 1: Type-4a for NR-CA is not attractive, as imbalanced maximum MIMO layer capability between two different NR CCs is not justified.

Observation 2: The extension for Type-4a to NR-CA is WID up-scoping thus belong to RAN-P level discussion.

Proposal 1：Do not consider Type-4a for NR-CA.

(UE capability)

Proposal 2: For UE capability, we propose,

- Define one UE capability for NR CA and one UE capability for EN-DC

- For EN-DC, differentiate Type-4a and Type-4b, covering/achieving by new UE capability design (which means not leveraging legacy UE capability)

- For NR-CA, only have Type-4b

- Specific UE capabilities’ design up to RAN2

(NW signalling)

Proposal 3: New NW signaling needed between Type-4 and Type-1 switch. (more details of New NW signaling is present in Proposal 4)

Observation 3: In Rel-18, the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2. And if maxMIMO-layers＞2, it is collocated.

Proposal 4: New Rel-19 NW signaling is needed to differentiate Type-4 and Type-1 when 2＜maxMIMO-layers ≤4.

Proposal 5: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling and Rel-18 NW signaling:

1）maxMIMO-layers ＞4, it is collocated

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, it could be either collocated(Type-1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

Note: “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

- 5. Reference

[1] RP-240838, R19 WID_Intra-band non-collocated EN-DC and NR-CA, RAN#103

[2] R4-2414453, WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#112

[3] R4-2412013, Discussion intra-band non-collocated deployment Phase2 UE Capability/UE behavior and network signaling, Nokia, NSB, RAN4#112

[4] R4-2412439, Remaining open UE capability discussions for type 4 UEs, Huawei, RAN4#112

[5] R4-2412612, Non-collocated Intra-band NR CA/EN-DC UE requirements, Qualcomm, RAN4#112

[6] R4-2412848, UE behavior and network signaling for type 4a/4b capable FWA UE for EN-DC/NR-CA, Ericsson, RAN4#112

[7] R4-2411415, On type 4 UE capability, Apple, RAN4#112

[8] R4-2411290, Discussion on UE Capability and BS Signaling for non-collocated Type 4a4b, KDDI, RAN4#112