3GPP TSG-RAN WG4 Meeting #112bis                      R4-2414956

Hefei, Anhui, China, 14th – 18th October 2024

Source:	China Telecom

Title:	Discussion on BS demodulation performance requirements for MMSE-IRC

Agenda Item:	6.16.3

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for BS requirements:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions:

  - Receiver type: MMSE-IRC receiver

  - Scenarios:

    - FDD synchronous deployments

    - TDD synchronous deployments with aligned UL:DL configurations among cells

    - Slot-based transmission and aligned SCS among cells

  - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

RAN4#112 is the first meeting for the discussion on the above objectives, as an outcome, the WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

Test scenarios

Status in the WF [1]: - Proposals: - Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104 - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

The LTE BS MMSE-IRC test requirements have covered both HomNet and HetNet scenarios. In our understanding, both scenarios are also typical scenarios for 5G NR deployments and thus we propose to also cover HomNet and HetNet scenarios for 5G BS MMSE-IRC test requirements.

Proposal 1: Cover HomNet and HetNet scenarios for 5G BS MMSE-IRC test requirements.

In addition, in our field test with different TDD UL/DL configurations between neighboring cells (celled dynamic frame structure in our field tests), heavy gNB-to-gNB co-channel CLI is observed, which can also be mitigated by BS MMSE-IRC. Our field test shows that without MMSE-IRC processing, the UL TP of the victim cell with DSUUU pattern has decreased by 37% when the aggressor gNB with 7D3U is on.

We understand this proposal is out of the agreed objectives in the revised WID in [2], however, considering this is a possible scenario in our practical deployment, we would still like to propose this possible scenario in our deployment and discuss how to model the corresponding interference profile.

Proposal 2: Consider the following scenario for BS MMSE-IRC requirement definition and discuss how to model the corresponding interference profile:

- Serving cell: DSUUU

- Neighbour cell: 7D3U

Interference profile

Status in the WF [1]: - Candidate options on interference modelling criteria: - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value - Option 2: DIP-based modelling - Candidate options on interference modelling: - Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - Other options not precluded - On interference power level: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - FFS how to account for HPUE in existing LTE profiles.

As agreed in the WID, we are generally fine to reuse the same interference modeling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbor cell PUSCH. And to avoid duplicate burden for new system level simulation, we propose to also reuse the same interference power level as in LTE, i.e., DIP -1.11dB and -10.91dB for HomNet scenario and DIP -0.43dB and -13.78dB for HetNet scenario.

Same time, as Rel-17 UE MMSE-IRC requirement definition has agreed to use INR-based modeling + SNR-based requirement definition criteria. It is also proposed to use the same approach for the BS side and transfer the DIP value into INR value.

As for coverage of HPUE, on the one hand, we agree with the motivation of considering HPUE in the test scope since it is widely used in 5G NR system. On the other hand, similar as the un-aligned TDD pattern scenario we have proposed, such approach requires system level simulation from companies to derive the interference profile.

We do not expect different implementations of BS MMSE-IRC receiver under HPUE scenario or unaligned TDD pattern scenarios either. Thus, to avoid system level simulation work, if the performance gain of MMSE-IRC over baseline MMSE can be confirmed under legacy LTE settings, we are fine not to cover HPUE or un-aligned TDD pattern scenarios. At the same time, RAN4 should agree that with the test under legacy LTE settings, the BS MMSE-IRC performance can also be confirmed under HPUE or un-aligned TDD pattern scenarios.

Proposal 3: Reuse the same interference modeling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbor cell PUSCH.

Proposal 4: Reuse the same interference power level as in LTE, i.e., DIP -1.11dB and -10.91dB for HomNet scenario and DIP -0.43dB and -13.78dB for HetNet scenario.

Proposal 5: Use INR-based modeling + SNR-based requirement definition criteria and transfer the DIP value into INR value.

Proposal 6: If the performance gain of MMSE-IRC over baseline MMSE can be confirmed under legacy LTE settings, not to cover HPUE or un-aligned TDD pattern scenarios. At the same time, RAN4 should agree that with the test under legacy LTE settings, the BS MMSE-IRC performance can also be confirmed under HPUE or un-aligned TDD pattern scenarios.

Test scope

The BS MMSE-IRC feature is a baseband feature which should be applicable for all SCS, TDD patterns, CHBW, PUSCH mapping type if supported.

Therefore, as for the test scope, it is proposed to align with the same test scope for Rel-15 PUSCH requirements with transform precoding disabled:

- Cover the following SCS and TDD patterns:

- 15kHz SCS with TDD pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS with TDD pattern 7D1S2U, S=6D:4G:4U

- Cover both PUSCH mapping Type A and Type B.

- Channel bandwidth:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

Proposal 7: Align with the same test scope for Rel-15 PUSCH requirements with transform precoding disabled:

- Cover the following SCS and TDD patterns:

- 15kHz SCS with TDD pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS with TDD pattern 7D1S2U, S=6D:4G:4U

- Cover both PUSCH mapping Type A and Type B.

- Channel bandwidth:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

MCS

Status in the WF [1]: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20

We support to cover QPSK, 16QAM and 64QAM in the test. It is proposed to decide the final MCS based on the simulation results considering the performance gain over baseline receiver.

Proposal 8: Cover QPSK, 16QAM and 64QAM in the test and decide the final MCS based on the simulation results considering the performance gain over baseline receiver.

Waveform

Status in the WF [1]: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

Considering the BS MMSE-IRC performance verification should be applicable for almost all BS products, it is proposed to also cover DFT-s-OFDM waveform for the test coverage.

Proposal 9: Also cover DFT-s-OFDM waveform.

Antenna configuration

The LTE BS MMSE-IRC test requirements have covered 2/4/8 Rx with 1Tx. Based on the test purpose to verify the BS MMSE-IRC processing itself, we propose to reuse the same antenna configuration.

Proposal 10: Cover 2/4/8 Rx with 1Tx.

Channel condition

The LTE BS MMSE-IRC test requirements use EPA 5 and EVA 70 channel model for the serving cell for HetNet and HomNet respectively. And ETU channel model is used for the interference cell(s).

Number of TX antennas (Note 1); Number of RX antennas (Note 1); Propagation conditions and correlation matrix (Annex B) (Note 2); DIP set. Tested signal; Interferer 1; Interferer 2. 1; 2; EPA 5 Low; ETU 5 Low; N/A; Set 2. EVA 70 Low; ETU 70 Low; N/A; Set 1*. 4; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*. 8; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*.

Since we have proposed to reuse the same interference power level (i.e., DIPs) for the interference cell, we should also use channel models with similar delay spread and Doppler frequency. Therefore, we propose to use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

As for the nonboring cell, currently TS38.104 has not yet modeled a propagation channel with delay spread as large as ETU model (i.e., 991ns) and with maximum excess tap delay as 5000ns. It is proposed to introduce new propagation channel for testing with desired delay spread 1000ns, i.e., TDLC1000.

Proposal 11: Use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

Proposal 12: Introduce new propagation channel for testing with desired delay spread 1000ns for the nonboring cell, i.e., TDLC1000.

Test metric

Status in the WF [1]: - Option 1: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation - Other options not precluded

We support to reuse the existing test metric for NR MMSE-IRC performance verification, i.e., SNR @ 70% of maximum throughput.

Proposal 13: Reuse the existing test metric for NR MMSE-IRC performance verification, i.e., SNR @ 70% of maximum throughput.

Conclusion

Proposal 1: Cover HomNet and HetNet scenarios for 5G BS MMSE-IRC test requirements.

Proposal 2: Consider the following scenario for BS MMSE-IRC requirement definition and discuss how to model the corresponding interference profile:

- Serving cell: DSUUU

- Neighbour cell: 7D3U

Proposal 3: Reuse the same interference modeling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbor cell PUSCH.

Proposal 4: Reuse the same interference power level as in LTE, i.e., DIP -1.11dB and -10.91dB for HomNet scenario and DIP -0.43dB and -13.78dB for HetNet scenario.

Proposal 5: Use INR-based modeling + SNR-based requirement definition criteria and transfer the DIP value into INR value.

Proposal 6: If the performance gain of MMSE-IRC over baseline MMSE can be confirmed under legacy LTE settings, not to cover HPUE or un-aligned TDD pattern scenarios. At the same time, RAN4 should agree that with the test under legacy LTE settings, the BS MMSE-IRC performance can also be confirmed under HPUE or un-aligned TDD pattern scenarios.

Proposal 7: Align with the same test scope for Rel-15 PUSCH requirements with transform precoding disabled:

- Cover the following SCS and TDD patterns:

- 15kHz SCS with TDD pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS with TDD pattern 7D1S2U, S=6D:4G:4U

- Cover both PUSCH mapping Type A and Type B.

- Channel bandwidth:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

Proposal 8: Cover QPSK, 16QAM and 64QAM in the test and decide the final MCS based on the simulation results considering the performance gain over baseline receiver.

Proposal 9: Also cover DFT-s-OFDM waveform.

Proposal 10: Cover 2/4/8 Rx with 1Tx.

Proposal 11: Use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

Proposal 12: Introduce new propagation channel for testing with desired delay spread 1000ns for the nonboring cell, i.e., TDLC1000.

Proposal 13: Reuse the existing test metric for NR MMSE-IRC performance verification, i.e., SNR @ 70% of maximum throughput.

Reference

R4-2413570, WF on NR demodulation performance: Phase 5, China Telecom, RAN4#112, Aug 2024.

RP-241297, Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN #104, June 2024.