3GPP TSG-RAN WG4 Meeting #112bis	R4-2414958

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.1

Source:	CAICT

Title:	Discussion on general aspects of AI/ML for NR air interface

Document for:	Discussion

1	Introduction

In this contribution, we are going to discuss following two issues from last meeting.

- Issue 1-1: Post deployment testing

- Issue 1-5: Generalization

2	Discussion

2.1 Issue 1-1: Post deployment testing

We provide some thoughts on this issue:

- Doing conformance test for all potential models/updates before deployment would induce frequent operations which require significant efforts. Both the feasibility and necessity are questionable. Major update or new version release need to pass conformance test before loading to devices to ensure the performance meeting requirements. But for minor update or fine-tuning, the performance could be guaranteed by device vendors via sufficient offline tests before loading to devices.

Further discussion is needed on how to define whether an update is considered as a major or minor update. Criteria for this distinction might include the scope of changes introduced, the impact on hardware capability, the extent of engineering required, and the potential risks involved. Establishing clear guidelines will help in planning and managing updates more effectively.

- To ensure the reliability of offline tests conducted by device vendors, it is critical to build a training/testing dataset that can cover as many scenarios/conditions as possible that are likely to be encountered in real deployment. The source and the composition of the database could be further discussed, for example, the data could be collected from previous conformance tests, or through public datasets in the industry, or collected internally by device vendors.

- Our understanding of post-deployment handing is to devise a mechanism to check how the functionality/model actually operates in the field after loading to devices. From this perspective, both conformance test and offline test still belong to the scope of pre-deployment.

Performance monitoring is an indispensable feature of AI/ML functionality. Effective monitoring can timely react to varied conditions in the field to avoid severe performance degradation. The effectiveness of performance monitoring can be ensured by specifying requirements and tests for performance monitoring and LCM procedure. In RAN4 #111, it was agreed that UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode. So, fallback to non-AI mode can be taken as baseline mechanism for post-deployment handling.

Proposal 1: Consider effective performance monitoring for post-deployment handling. The effectiveness of performance monitoring is guaranteed by specifying requirements and tests for monitoring and LCM procedure.

Proposal 2: Suggest to discuss potential solutions to build dataset for offline performance tests conducted inside vendor.

Proposal 3: Fallback to non-AI mode can be taken as baseline mechanism for post-deployment handling.

2.2 Issue 1-5: Generalization

<TR 38.843> The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition.

According to description in TR 38.843, the key point of generalization study is to identify what is the significant degradation and what is “other scenarios and/or configurations”. In RAN2, following agreement was achieved:

RAN2 agreed the following understandings on terminologies: - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signaling) - Applicable functionalities refer to functionalities that the UE is ready to apply for inference - Activated functionalities refer to functionalities already enabled for performing inference

In our understanding, “applicable”means UE has at least an available model for the targeting scenarios and/or configurations, thus “the other scenarios and/or configurations” shall be the scenarios and/or configurations that are not reported as “applicable” on UE side. Whether to include scenarios and/or configurations that are not reported as “supported” on UE side needs further discussion. From this perspective, in “other scenarios and/or configurations”, the AI/ML functionality is non-applicable on UE side, and then UE would fallback to non-AI mode and legacy non-AI requirement shall be met in this status.

Proposal 4: For generalization tests, “other scenarios and/or configurations” are the scenarios and/or configurations that are not reported as “applicable”. Further discuss whether to include scenarios and/or configurations that are not reported as “supported” on UE side.

Proposal 5: For generalization tests, legacy non-AI performance requirement could be considered for “other scenarios and/or configurations”.

Conclusion

In this contribution, the issues of post deployment testing and generalization are discussed with following proposals:

Proposal 1: Consider effective performance monitoring for post-deployment handling. The effectiveness of performance monitoring is guaranteed by specifying requirements and tests for monitoring and LCM procedure.

Proposal 2: Suggest to discuss potential solutions to build dataset for offline performance tests conducted inside vendor.

Proposal 3: Fallback to non-AI mode can be taken as baseline mechanism for post-deployment handling.

Proposal 4: For generalization tests, “other scenarios and/or configurations” are the scenarios and/or configurations that are not reported as “applicable”. Further discuss whether to include scenarios and/or configurations that are not reported as “supported” on UE side.

Proposal 5: For generalization tests, legacy non-AI performance requirement could be considered for “other scenarios and/or configurations”.