3GPP TSG-RAN WG4 Meeting #112bis	R4-2414959

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.3.1

Source:	CAICT

Title:	Discussion on testability and interoperability issues for CSI prediction

Document for:	Discussion

1	Introduction

In this contribution, we will discuss testability and interoperability issues for CSI prediction in two parts:

- Part 1: General aspects

- Part 2: Testing approaches

2	Discussion

2.1 Part 1: General aspects

This part mainly discusses whether the previous agreements on “General aspects” are applicable to CSI prediction. In general, we see no special impact introduced by CSI prediction on previous agreements, while some issues need to be discussed per use case.

- Channel model

Agreement (RAN4 #112): - Use synthetic channels as the default assumption for conformance requirements/test. - RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data. - The field data can be considered for the analysis.

For channel model of CSI prediction test, we suggest to follow the settings in current test cases of PMI reporting, i.e., use TDL channel as starting point.

Proposal 1: Suggest to consider TDL channel as starting point for CSI prediction tests. FFS whether to introduce CDL channel.

- Static and non-static conditions

Agreement (RAN4 #110bis): - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

In our understanding, performance test is analogous to legacy PMI reporting test, thus static condition is suggested to be taken as default assumption. A larger maximum doppler frequency and/or LOS path could be considered in channel realization under specific test cases of CSI prediction.

LCM test, if any, seems to be irrelevant to static/non-static conditions. But non-static conditions that would induce significant performance fluctuation could be introduced to trigger certain LCM procedure.

Non-static conditions are primarily considered in generalization test. Which conditions shall be dynamic in the test could refer to generalization evaluation in TR 38.843. For CSI prediction, the generalization is evaluated mainly over various UE speed, deployment scenario, carrier frequency and UE distribution. How to introduce non-static conditions is another critical point. We see two possible alternatives:

- Alt 1: the conditions are randomly changed

Firstly, the feasibility of such implementation requires clarification from TE vendors. Secondly, the necessity of this method is questionable in certain aspects, such as carrier frequency and deployment scenario, where conditions may not vary too dynamically in practice. Lastly, this method is more suitable for the generalization case of training in certain scenario/condition and testing in mixed scenarios/conditions.

- Alt 2: the conditions are sequentially changed in predefined order

In our understanding, this method is similar to splitting into individual tests with static conditions. From an implementation perspective, DUT can train a model based on a mixed dataset and then test under sequentially changing conditions. We consider this to be generalization case 3 (defined in TR 38.843). Alternatively, DUT can train multiples models with each model corresponding to specific condition(s), and internally select different models based on changes in the testing conditions.

Proposal 2: For performance test of CSI prediction, suggest to take static condition as default assumption.

Observation 1: Two possible alternations for introducing non-static conditions in generalization test:

- Alt 1: the conditions are randomly changed

- Alt 2: the conditions are sequentially changed in predefined order.

Proposal 3: Further discuss whether and how to introduce non-static conditions in generalization test.

- Ground truth handling

Agreement (RAN4 #109) - Explicit definition of ground truth could be discussed further in WI use case by use case when a necessity is identified

Agreement (RAN4 #111): - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

Given the agreements on ground truth for beam management, we think CSI prediction could follow the same logic to define ground truth, i.e., the ground truth for the predicted CSI is the ideal measurement of CSI on the predicted time instance, which can be approximated as the reported CSI measurement under the certain SNR (SNR level is FFS). But the solid definition needs to wait RAN1’s progress on input and output of CSI prediction.

Observation 2: The definition of ground truth for CSI prediction is analogous to beam management.

Proposal 4: The discussion on ground truth shall wait for RAN1’ s progress on input and output of CSI prediction.

2.2 Part 2: Testing approaches

In last meeting, we have following agreements:

Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

In legacy performance requirement of PMI reporting, the baseline (denominator) for relative throughput is established using random precoding. The similar approach could be applied to CSI prediction testing. The specific codebook employed for random precoding needs further discussion. TypeI single panel codebook used in current test cases offers ease of implementation, yet the throughput gains and performance requirements would be influenced by measurements, prediction and fine-grained codebook used in measurements/prediction. An alternative way is to utilize the same codebook for both random precoding and prediction, for example, eTypeII codebook, which could mitigate the performance discrepancy arising from variations in codebook granularity. Additionally, the method for randomly selecting matrices from eTypeII codebook needs further exploration.

Proposal 5: Suggest to follow legacy testing as starting point, i.e., random precoding use TypeI single panel codebook as baseline for relative throughput.

Proposal 6: Further discuss other alternatives, for example, random precoding using the same codebook as prediction as the baseline for relative throughput.

On the other hand, following UE reported precoder could also be considered as baseline for relative throughput to make the test more concentrated on prediction accuracy. A basic test procedure is illustrated as figure 1. For CSI prediction (numerator), the reported precoder is the output of AI model generated based on past measurements in observation window. For baseline, the reported precoder is generated from nearest measurement using specific codebook.

Figure 1 illustration of basic test procedure for CSI prediction

Furthermore, the benefit of CSI prediction may come from not only more precise CSI feedback, but also the potential reduction of measurement resource. Then the testing procedure would be varied as figure 2. The upper part is for CSI prediction, the output of AI model covers multiple future reporting instances in the prediction window. This enables NW to configure a larger interval for CSI-RS resource. The grey dash line indicates the reduced CSI-RS resource. Meanwhile, the period of PMI reporting in baseline (lower part in figure 2) should be set to the same length as prediction window for fair comparison.

Figure 2 illustration of testing procedure for CSI prediction with multiple instances

Observation 3: For AI-based CSI prediction, the CSI information predicted based on measurements within one measurement period or one measurement window is able to cover multiple reporting instances.

Proposal 7: Suggest to discuss taking UE reported precoder generated from measurement as baseline for relative throughput. FFS the specific test procedure (e.g., as illustrated in figure 1 and 2).

Conclusion

In this contribution, testability and interoperability issues for CSI prediction are discussed with following proposals:

Proposal 1: Suggest to consider TDL channel as starting point for CSI prediction tests. FFS whether to introduce CDL channel.

Proposal 2: For performance test of CSI prediction, suggest to take static condition as default assumption.

Observation 1: Two possible alternations for introducing non-static conditions in generalization test:

- Alt 1: the conditions are randomly changed

- Alt 2: the conditions are sequentially changed in predefined order.

Proposal 3: Further discuss whether and how to introduce non-static conditions in generalization test.

Observation 2: The definition of ground truth for CSI prediction is analogous to beam management.

Proposal 4: The discussion on ground truth shall wait for RAN1’ s progress on input and output of CSI prediction.

Proposal 5: Suggest to follow legacy testing as starting point, i.e., random precoding use TypeI single panel codebook as baseline for relative throughput.

Proposal 6: Further discuss other alternatives, for example, random precoding using the same codebook as prediction as the baseline for relative throughput.

Observation 3: For AI-based CSI prediction, the CSI information predicted based on measurements within one measurement period or one measurement window is able to cover multiple reporting instances.

Proposal 7: Suggest to discuss taking UE reported precoder generated from measurement as baseline for relative throughput. FFS the specific test procedure (e.g., as illustrated in figure 1 and 2).