3GPP TSG-RAN WG4 Meeting #112bis	R4-2414965

# Hefei, China,14 – 18 Oct., 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38101-3 to add x LTE and x NR inter-band EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Rogers |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R19_xBLTE_yBNR |  |  |  |  |  | Date: |  |  | 2024-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | The current specification TS 38.101-3 has not included the following DC_R19_xBLTE_yBNR_zDLqUL (x>0, y>0, x+y=z, 3<z≤6, 2≤q≤3) combinations in the WID RP-241786 based on operator request. DC_2A-5A_n41A-n77A DC_2A-5A_n41A-n78A DC_2A-66A_n41A-n66A DC_2A-66A_n41A-n77A DC_2A-66A_n41A-n78A DC_5A-66A_n41A-n66A DC_5A-66A_n41A-n77A DC_5A-66A_n41A-n78A DC_2A-5A-66A_n41A-n66A DC_2A-5A-66A_n41A-n77A DC_2A-5A-66A_n41A-n78A The correspongding fallbacks are all completed in TS 38.101-3 or submitted this meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the above mentioned DC_R19_xBLTE_yBNR_zDLqUL combinations in the spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above mentioned combinations are not supported in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.3, 5.5B.4.4,6.2B.4.2.3.3, 6.2B.4.2.3.4, 7.3B.3.3.3 and 7.3B.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A |

…

| DC_2A-5A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_5A_n41A DC_5A_n66A |
| --- | --- |
| DC_2A-12A-(n)5AA | DC_2A_n5A DC_12A_n5A DC_(n)5AA4 |
| DC_2A-5A-30A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A |
| DC_2A-5A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-5A-30A_n77A9 DC_2A-2A-5A-30A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-30A_n77(2A) 9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A_n41A-n77A | DC_2A_n41A DC_2A_n77A DC_5A_n41A DC_5A_n77A |
| DC_2A-5A_n41A-n78A | DC_2A_n41A DC_2A_n78A DC_5A_n41A DC_5A_n78A |
| DC_2A-5A-48A_n12A | DC_2A_n12A DC_5A_n12A DC_48A_n12A |

…

| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A |
| --- | --- |
| DC_2A-66A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_66A_n41A |
| DC_2A-66A_n41A-n77A | DC_2A_n41A DC_2A_n77A DC_66A_n41A DC_66A_n77A |
| DC_2A-66A_n41A-n78A | DC_2A_n41A DC_2A_n78A DC_66A_n41A DC_66A_n78A |
| DC_2A-66A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_66A_n66A4 DC_66A_n71A |

…

| DC_5A-66A_n2A-n78A | DC_5A_n2A DC_66A_n2A DC_5A_n78A DC_66A_n78A |
| --- | --- |
| DC_5A-66A-(n)12AA | DC_5A_n12A DC_66A_n12A DC_(n)12AA4 |
| DC_5A-66A_n41A-n66A | DC_5A_n41A DC_5A_n66A DC_66A_n41A |
| DC_5A-66A_n41A-n77A | DC_5A_n41A DC_5A_n77A DC_66A_n41A DC_66A_n77A |
| DC_5A-66A_n41A-n78A | DC_5A_n41A DC_5A_n78A DC_66A_n41A DC_66A_n78A |
| DC_5A-66A_n66A-n77A9 DC_5A-66A_n66A-n77C9 | DC_5A_n66A DC_5A_n77A9 DC_66A_n77A9 |

……

< Non-changed part is omitted >

<<Next Change>>

5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_5A_n28A DC_7A_n28A |

…

| DC_2A-5A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_5A_n2A DC_5A_n78A DC_66A_n2A DC_66A_n78A |
| --- | --- |
| DC_2A-5A-66A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_5A_n41A DC_5A_n66A DC_66A_n41A DC_66A_n66A |
| DC_2A-5A-66A_n41A-n77A | DC_2A_n41A DC_2A_n77A DC_5A_n41A DC_5A_n77A DC_66A_n41A DC_66A_n77A |
| DC_2A-5A-66A_n41A-n78A | DC_2A_n41A DC_2A_n78A DC_5A_n41A DC_5A_n78A DC_66A_n41A DC_66A_n78A |
| DC_2A-5A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_5A_n66A DC_5A_n77A8 DC_66A_n77A8 |

…

< Non-changed part is omitted >

<<Next Change>>

6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-(n)3-n8 | 0.3 | 0.3 | 0.3 | 0.3 |
| DC_1-3_n3-n41 | 0.5 | 0.5 | 0.5 | 0.34/0.85 |

…

| DC_2-5_n41-n66 | 0.5 | 0.6 | 0.51 / 12 | 0.5 |
| --- | --- | --- | --- | --- |
| DC_2-5_n41-n77 | 0.6 | 0.6 | 0.41 / 0.92 | 0.8 |
| DC_2-5_n41-n78 | 0.6 | 0.6 | 0.41 / 0.92 | 0.8 |
| DC_2-5-48_n12 | 0.6 | 0.8 | 0.8 | 0.4 |

…

| DC_2-66_n38-n78 | 0.6 | 0.6 | 0.9 | 0.8 |
| --- | --- | --- | --- | --- |
| DC_2-66_n41-n66 | 0.5 | 0.5 | 0.81 / 1.32 | 0.5 |
| DC_2-66_n41-n71 | 0.5 | 0.5 | 0.81 / 1.32 | 0.8 |
| DC_2-66_n41-n77 | 0.6 | 0.6 | 0.81 / 1.32 | 0.8 |
| DC_2-66_n41-n78 | 0.5 | 0.5 | 0.81 / 1.32 | 0.8 |
| DC_2-66_n66-n71 | 0.5 | 0.5 | 0.5 | 0.3 |

…

| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| --- | --- | --- | --- | --- |
| DC_5-66_(n)12 | 0.3 | 0.8 | 0.8 | 0.8 |
| DC_5-66_n41-n66 | 0.6 | 0.5 | 0.81 / 1.32 | 0.5 |
| DC_5-66_n41-n77 | 0.6 | 0.6 | 0.81 / 1.32 | 0.8 |
| DC_5-66_n41-n78 | 0.6 | 0.6 | 0.81 / 1.32 | 0.8 |
| DC_5-66_n66-n77 | 0.6 | 0.6 | 0.6 | 0.8 |

…

< Non-changed part is omitted >

<<Next Change>>

6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-5-7_n28 | 0.5 | 0.5 | 0.7 | 0.6 | 0.7 |
| DC_1-3-5-7_n40 DC_1-3-5-7-7_n40 | 0.6 | 0.6 | 0.6 | 0.8 | 0.9 |

…

| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| --- | --- | --- | --- | --- | --- |
| DC_2-5-66_n41-n66 | 0.6 | 0.6 | 0.81 / 1.32 | 0.41 / 0.92 | 0.81 / 1.32 |
| DC_2-5-66_n41-n77 | 0.6 | 0.6 | 0.81 / 1.32 | 0.41 / 0.92 | 0.8 |
| DC_2-5-66_n41-n78 | 0.6 | 0.5 | 0.81 / 1.32 | 0.41 / 0.92 | 0.8 |
| DC_2-5-30-66_n2 | 0.5 | 0.3 | 0.3 | 0.5 | 0.5 |

…

< Non-changed part is omitted >

<<Next Change>>

7.3B.3.3.3	ΔRIB,c for EN-DC four bands

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-(n)3-n8 | - | - | - | - |
| DC_1-3_n3-n41 | - | - | - | 03 / 0.54 |

…

| DC_2-5-30_n77 DC_2-2-5-30_n77 | 0.2 | 0.2 | - | 0.5 |
| --- | --- | --- | --- | --- |
| DC_2-5_n41-n66 | 0.3 | 0.2 | 0.51 / 12 | 0.5 |
| DC_2-5_n41-n77 | 0.2 | 0.2 | - | 0.5 |
| DC_2-5_n41-n78 | 0.2 | 0.2 | - | 0.5 |
| DC_2-5-48_n12 | 0.2 | 0.5 | 0.5 | 0.3 |

…

| DC_2-66_n38-n78 | 0.5 | 0.5 | 0.5 | 0.5 |
| --- | --- | --- | --- | --- |
| DC_2-66_n41-n66 | 0.3 | 0.5 | 0.51 / 12 | 0.5 |
| DC_2-66_n41-n71 | 0.3 | 0.3 | 0.51 / 12 | 0.5 |
| DC_2-66_n41-n77 | 0.3 0.6 | 0.5 0.6 | 0.51 / 12 | 0.5 0.8 |
| DC_2-66_n41-n78 | 0.3 0.6 | 0.5 0.6 | 0.51 / 12 | 0.5 0.8 |
| DC_2-66_n66-n71 | 0.3 | 0.3 | 0.3 | - |

…

| DC_5-66_(n)12 | - | 0.5 | 0.5 | 0.5 |
| --- | --- | --- | --- | --- |
| DC_5-66_n41-n66 | 0.6 | 0.5 | 0.81 / 1.32 | 0.5 |
| DC_5-66_n41-n77 | 0.6 | 0.5 | 0.81 / 1.32 | 0.8 |
| DC_5-66_n41-n78 | 0.6 | 0.5 | 0.81 / 1.32 | 0.8 |
| DC_5-66_n66-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| DC_7_n1-n75-n78 | 0.2 | 0.2 | - | 0.5 |

…

< Non-changed part is omitted >

<<Next Change>>

7.3B.3.3.4	ΔRIB,c for EN-DC five bands

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-5-7_n28 | - | - | 0.2 | - | 0.2 |
| DC_1-3-5-7_n40 DC_1-3-5-7-7_n40 | - | - | 0.2 | 0.3 | 0.8 |

…

| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 0.3 | 0.2 | 0.2 | 0.2 | 0.5 |
| --- | --- | --- | --- | --- | --- |
| DC_2-5-66_n41-n66 | 0.3 | - | 0.6 | 0.5 | 0.6 |
| DC_2-5-66_n41-n77 | 0.3 | 0.2 | 0.6 | 0.5 | 0.6 |
| DC_2-5-66_n41-n78 | 0.3 | 0.2 | 0.6 | 0.5 | 0.6 |
| DC_2-5-66_n66-n77 | 0.3 | - | 0.3 | 0.3 | 0.5 |
| DC_2-7-12_n2-n66 | 0.3 | 0.3 | 0.5 | 0.3 | 0.5 |

…

< Non-changed part is omitted >

<<End of Change>>