3GPP TSG-RAN WG4 Meeting #112bis	R4-2414981
Hefei, Anhui, China, 14th – 18th October, 2024

Title:	R19 6Rx SRS IL for n104

Source:	OPPO

Agenda item:	6.1.1.3.3

Document for:	Approval

- Introduction

This paper shares our evaluation of the SRS ILs for n104 (6GHz).

- Discussion

2.1 SRS IL parameters

The SRS IL is the difference of ILs between diversity antenna and main antenna. It may include the ILs caused by switches, filters, and the PCB trace/RF cable/routing losses, and so on. And the IL mainly comes from the switches and PCB trace loss. To better understand how much difference between the SRS ILs in n104 and n79, the component ILs have been checked. Below table 1 is the summary of component IL differences between n79 and n104. The exact ILs can be found in figure 1~4.

Table 1 Component IL difference between n79 and n104

|  | Delta IL (dB) = n104 – n79 |
| --- | --- |
| DP4T IL | 0.3 |
| DP3T IL | 0.3 |
| DPDT IL | 0.4 |
| SP4T IL | 0.5 |
| SPDT IL | 0.2 |
| 3P3T IL | 0.5 |
| PCB trace IL (SRS path) | 0.6 |

Figure 1 SPDT IL difference                                  Figure 2 SP4T IL difference

Figure 3 DPDT IL difference                                    Figure 4 DP4T IL difference

Based on above IL differences. The SRS IL for n104 are proposed as below.

2.2 t1r6

Figure 5 Reference UE architecture of t1r6 SRS antenna switching

Table 2 SRS IL difference between n79 and n104 for t1r6

| Ant 5 – Ant 1 | 3P3T + SPDT*2 +PCB IL = 1.5 |
| --- | --- |

Observation 1:         The SRS IL difference between n79 and n104 for t1r6 is 1.5dB.

2.2 t2r6

Figure 6 Reference UE architecture of t2r6 SRS antenna switching

Table 3 SRS IL difference between n79 and n104 for t2r6

| Ant 5 – Ant 1 | SPDT +PCB IL = 0.8 |
| --- | --- |

Observation 2:         The SRS IL difference between n79 and n104 for t2r6 is 0.8dB.

2.2 t2r6+t1r6

Figure 7 Reference UE architecture of t2r6+t1r6 SRS antenna switching

Table 4 SRS IL difference between n79 and n104 for t2r6+t1r6

| Ant 4 – Ant 1 | DP3T + SPDT*2 +PCB IL = 1.3 |
| --- | --- |

Observation 3:         The SRS IL difference between n79 and n104 for t2r6+t1r6 is 1.3dB.

Proposal 1:            The SRS IL for n104 is as below.

|  | n104 SRS IL |
| --- | --- |
| t1r6 | n79 IL + 1.5 dB |
| t2r6 | n79 IL + 0.8 dB |
| t1r6 + t2r6 | n79 IL + 1.3 dB |

For n79 IL, the averaged value was used in WF [2], it can be used as baseline to derive the value for n104.

For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers. - Option 10: Adopt the average values from companies Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79, n104; 5.4; 4.7; 5.9. - Option 11: Adopt the average values from companies considering an additional breakpoint Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79; 5.4; 4.7; 5.9. Band n104; 5.7; 5.0; 6.3.

Proposal 2:            The SRS IL for n79 is as below (the averaged value in last meeting WF).

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

- Conclusion

This paper analyzed the SRS IL differences between n79 and n104, and got below proposals.

Observation 1:         The SRS IL difference between n79 and n104 for t1r6 is 1.5dB.

Observation 2:         The SRS IL difference between n79 and n104 for t2r6 is 0.8dB.

Observation 3:         The SRS IL difference between n79 and n104 for t2r6+t1r6 is 1.3dB.

Proposal 1:            The SRS IL for n104 is as below.

|  | n104 SRS IL |
| --- | --- |
| t1r6 | n79 IL + 1.5 dB |
| t2r6 | n79 IL + 0.8 dB |
| t1r6 + t2r6 | n79 IL + 1.3 dB |

Proposal 2:            The SRS IL for n79 is as below (the averaged value in last meeting WF).

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

- Reference

[1] R4-2408761 R19 6Rx SRS IL, OPPO, RAN4#111 2024-05