3GPP TSG-RAN WG4 Meeting #112bis												R4-2414986
Hefei, Anhui, China, 14th – 18th October, 2024

Title:	RAN task R17 power class of 1UL CC with DL CA

Source:	OPPO

Agenda item:	8.2

Document for:	Approval

- Introduction

The power class ambiguity issue has been discussed for a long time and in RAN4#111 meeting the group decided to stop the discussion unless there is clear offline consensus. And then in RAN#104 meeting, a company paper was submitted to further pursue this topic, especially for the DL CA only case and would like to enable HPUE in DL CA with single UL CC case. And RAN task is given to RAN4 as below [1].

Way Forward - RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL. - RAN task shall conclude by RAN#106 - RAN4 online meeting time should be limited to <0.1 TUs

This paper further discusses this issue.

- Clarification of the RAN task

From the RAN task WF, it is clear that this task is to investigate “how to enable HPUE in DL CA with 1UL CC”. However, in last meeting, there is view that the task is for “the band combinations with no HPUE MSD for 1UL CC / DL CA”. This kind of view has misunderstood the RAN task.

Observation 1:  The RAN task is for all HPUE band combinations with 1UL CC and DL CA, instead of only the band combinations without MSD.

- How to enable HPUE for DL CA with UL single CC

In last meeting, there are some offline discussions on how to enable UE to transmit higher power than default power class in 1ULCC / DLCA band combination. There are three candidate approaches as in table 1.

Table 1 Candidate approaches to enable HPUE for 1UL CC / DL CA

|  | Pros | Cons | OPPO View |
| --- | --- | --- | --- |
| Approach 1: Follow normal basket approach, introduce the missing MSD | - Normal procedure followed, no ambiguity in the spec - Band combination HPUE is clear for implementation | - The MSD need to be introduced before HPUE can be enabled. - Need wait for 3 months before enable HPUE (i.e., add the band combination in this meeting, and requirement can be specified in next meeting) | We support this normal basket approach: - This is clean solution which has been applied for many years in RAN4. It needs to be followed unless critical issues. - With this normal approach, it only takes 3 months to specify the MSD for a HPUE which seems not a strong reason to break this procedure. |
| Approach 2: Test PC3 first, then test PC2 after MSD is introduced | - HPUE can be enabled before spec is ready (3 months earlier) | - Normal basket procedure is broken, it introduces a precedent to bypass the basket approach - The requirements for a band combination will be changed from meeting to meeting, which cause chaos for implementation - UE implemented by following PC3 requirements may fail the PC2 requirements defined later which will cause certification problems | We have concerns on this approach: - This breaks the normal basket approach with no strong reason. - It is unclear what prevents companies to specify the MSD into spec which actually only takes one meeting. - The changing requirements will make UE implementation be problematic, and have risk to fail certifications. |
| Approach 3: Only test PC3 | - HPUE can be enabled before spec is ready (3 months earlier) - The HPUE MSD definition can be simplified once for all - It is clear for UE implementation and no risk of violating certification requirements | - MSD procedure need to be changed for HPUE, and no HPUE MSD is needed anymore | We may be fine with this approach: - It can save 3 months to implement a HPUE if this is a real concern. - It is clear for UE implementation, i.e. only PC3 is certified. - It can save efforts to specify the MSD for 1ULCC/DL CA. |

Proposal:   	To enable HPUE for 1UL CC / DL CA, it is proposed to either of following approaches:

        - Option 1: Follow normal basket approach and specify the MSD (it only takes 1 meeting cycle to specify the MSD)

        - Option 2: Only test PC3 for 1UL CC / DL CA band combinations.

- Conclusions

In this paper, it clarifies the RAN task first due to some different views on the scope of this RAN task in last meeting. Then this paper has analyzed three candidate approaches with pros and cons as in table 1, also our view on each approach. Finally, we got below observations and proposals.

Observation 1:  The RAN task is for all HPUE band combinations with 1UL CC and DL CA, instead of only the band combinations without MSD.

Proposal:   	To enable HPUE for 1UL CC / DL CA, it is proposed to adopt either of following approaches:

        - Option 1: Follow normal basket approach and specify the MSD (it only takes 1 meeting cycle to specify the MSD)

        - Option 2: Only test PC3 for 1UL CC / DL CA band combinations.

- References

- RP-241687, Moderator's summary for offline discussion on CA Power Class Support, RAN vice-chair, RAN#104 2024-06