3GPP TSG-RAN WG4 Meeting #112bis				                                                                                    R4-2414994

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	5.20.2

Source: 	CMCC

Title: 			  UE RF requirements for n28

Document for:	  Discussion

- Introduction

In last RAN4 meeting, the UE requirements for n28 PC2 and n40 had been discussed and way forward had been agreed in [1].

In this contribution, we discuss the left open issues for n28 requirements.

- Discussion

Issue 2-1-1: UE architecture assumption Agreement: - No need to study the feasibility of UE architecture, companies are encouraged to provide analysis on RF requirements based on their own implementation. - RAN4 RF requirements should accommodate different UE architecture assumption. - FFS on whether single set of requirements for different UE architectures should be defined.

PC2 requirements for n28 were not specified in R18 due to lack of consensus on the UE architecture and A-MPR requirements. In Rel-19 WI, last meeting, RAN4 agreed to not specify any A-MPR requirements for PC3 regardless of UE architecture assumption. For PC2, it is also more straightforward to only specify single set of requirements.

Proposal 1: It is proposed to specify single set of requirements for PC2 n28.

Issue 2-3-8 channel location

Agreement:

- FFS on how to capture the UL channel location for UE supporting maximum bandwidth of 40MHz in n28.

Firstly, consider 40MHz as an optional feature, hence the legacy note should be kept for UE supporting maximum 30MHz, a new note should be specified to apply for UE supporting 40MHz.

Secondly, according to the WID, the channel location is allowed as follows. Since UE is not required to support the whole band for 40MHz. Hence, the new note should reflect the channel location within 703-743MHz.

- Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2.

  - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz.

Thirdly, when RAN4 discussed the guard band misaligned issue between 30MHz and 40MHz for n28, it was observed that the UE channel bandwidth cannot extends beyond band edge. So one note is added to the channel raster table to clarify, and also one additional channel raster is added to 40MHz for n28.

“NOTE 1:	The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used.”

According to the above agreement, the channel bandwidth for 40MHz with additional channel raster will exceed 743MHz, i.e. 743.04MHz. In last meeting, there was discussion on the term use for this note, i.e. whether “UL transmission bandwidth” or “UE channel bandwidths” should be used. No matter which term is used, it should allow the 40MHz channel bandwidth exceed 743MHz.

To summarize, we propose add a new note of UL channel location for UE supporting maximum 40MHz channel bandwidth for n28 as follows:

- For UEs supporting 40MHz, for the 20MHz, 25MHz and 30MHz, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-743 MHz or 718-748 MHz. For the 40MHz, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to 703-743.04MHz.

Proposal 2: It is proposed to add a new note of UL channel location for UE supporting maximum 40MHz channel bandwidth for n28 as follows:

- For UEs supporting 40MHz, for the 20MHz, 25MHz and 30MHz, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-743 MHz or 718-748 MHz. For the 40MHz, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to 703-743.04MHz.

Issue 2-3-9 channel raster

Agreement:

- The following options can be considered for n28 40MHz

  - Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

  - UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

RAN4 had been agreed to introduce exceptional channel raster of n28 for n40MHz in BS specification to resolve the guard band misaligned issue between 30MHz and 40MHz. And it had also been agreed to add the exceptional channel raster to UE when 40MHz is introduced for UE in the future. Since the UE channel bandwidth cannot extend beyond band, in order to accommodate both legacy 30MHz and new 40MHz UE, the only way is to place 40MHz channel raster at the exceptional raster point. Hence, from our perspective, it is essential to mandate support of exceptional channel raster of n28 for UE CBW 40MHz.

Proposal 3: it is proposed to add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz, and UE supports n28 40MHz should mandatory support the exceptional channel raster.

Regarding the feature of enhanced channel raster, this was discussed in Rel-18 and agreement was to mandatory support enhanced channel raster for n28. And enhanced channel raster feature has no relation to the supported maximum channel bandwidth. Hence, we think the agreement for enhanced channel raster can be kept, there is no need to discuss whether UE supporting 40MHz CBW should support enhanced channel raster or not.

Proposal 4: For enhanced channel raster feature, it is proposed to follow the agreement of Rel-18 enhanced channel raster, i.e. n28 mandatory support enhanced channel raster from Rel-18.

Issue 2-3-10 Release independence

Agreement:

- FFS on the release independence of 40MHz for n28

  - Option 1: Rel-15

  - Option 2: Rel-16

In last meeting contributions, some companies proposed that 40MHz should be release independence from Rel-16 since the additional channel raster point for n28 was introduced in Rel-16 BS specification. However, in our view, BS does not really have the release concept, operator can choose to update BS by feature.

According to TS38.331, the Rel-15 supportedBandwidth IE already supports indicating 40MHz, hence from specification perspective, 40MHz  can be release independence from Rel-15.

Proposal 5: it is proposed that UE 40MHz channel bandwidth is release independent from Rel-15.

- Conclusion

In this contribution, we discussed the open issues for n28 UE requirements. The proposals are provided as follows:

Proposal 1: It is proposed to specify single set of requirements for PC2 n28.

Proposal 2: It is proposed to add a new note of UL channel location for UE supporting maximum 40MHz channel bandwidth for n28 as follows:

- For UEs supporting 40MHz, for the 20MHz, 25MHz and 30MHz, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-743 MHz or 718-748 MHz. For the 40MHz, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to 703-743.04MHz.

Proposal 3: it is proposed to add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz, and UE supports n28 40MHz should mandatory support the exceptional channel raster.

Proposal 4: For enhanced channel raster feature, it is proposed to follow the agreement of Rel-18 enhanced channel raster, i.e. n28 mandatory support enhanced channel raster from Rel-18.

Proposal 5: it is proposed that UE 40MHz channel bandwidth is release independent from Rel-15.

- Reference

- R4-2414274, WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, CBN, China Broadnet

- RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC CBN.

[2] R4-2410558, WF on A-MPR for n28, Apple (approved).

[3] R4-2406708, WF on band n28 full band duplexer, Apple.

[4] R4-2403629, WF on HPUE for FDD bands, China Unicom, Skyworks, RAN4#110 (approved).